3GPP TSG-RAN WG4 #89 Meeting                                        	R4-1814793

Spokane, USA, 12 – 16 November, 2018

Source: 	LG Electronics, Verizon

Title: 	MSD test results for new x bands DL (x=3, 4, 5) with 2 bands UL CA in Rel-16

Agenda item:	8.6.2

Document for:	Approval

1	Introduction

From self-desense analysis [1], MSD results are provided for new x bands DL (x=3, 4, 5) and 2 bands UL CA band combinations with self-interference problems in rel-16.

For 3 bands DL and 2 bands UL CA band combinations, MSD results are provided for one of three downlink bands which is impacted by harmonics and IMD products during dual uplink transmission.

For 4 bands DL with 2 bands UL and 5 bands DL with 2 bands UL CA band combinations, there is no need to study more for the case when harmonics and IMDs impact to their own receiving bands (4th and 5th) since all self-desense problems will be covered in 2 bands DL/2 bands UL and 3 bands DL/2 bands UL CA band combinations.

### 2.	Summary of interference studies

Table 1 shows 3 bands DL/ 2 bands UL CA band combinations with self-interference problems in Rel-16 from agreed TP [1] in RAN4#88BIS meeting.

Table 1: Summary of Self-interference analysis for 3 bands DL/ 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | FFS |
|  | CA_4A-13A | - | 4th IMD | - | FFS |
| CA_4A-5A-13A | CA_4A-13A | - | - | - | N/A |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | FFS |
| CA_2A-2A-4A-5A | CA_4A-5A | - | - | - | N/A |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | - | 4th IMD | - | FFS |
| CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | FFS |
| CA_2A-5B-66A-66A | CA_5A-66A | - | - | - | N/A |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - FFS for IMDs |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | FFS |
|  | CA_5A_66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - FFS for IMDs |
| CA_2A-13A-66A-66B | CA_2A-13A | - | 4th IMD | - | FFS |
| CA_2A-13A-66A-66B | CA_13A-66A | - | - | - | N/A |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - FFS for IMDs |
| CA_2A-13A-46D | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - FFS for IMD |

For MSD analysis, UE RF component parameters in Table 2 and Table 3 are considered.

Table 2: UE RF Front-end component parameters

| UE ref. architecture | Cas-caded Diplexer Architecture |  |  |  |
| --- | --- | --- | --- | --- |
| Component | All CA band combos |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| Quadplexer | 110 | 72 | 55 | 52 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Each isolation level of RF componenets are shown in Table 3 below.

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Diplexer | 25 | High/low band isolation |
| Quadplexer | 15 | Adjacent Tx-Rx atten level |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

From our analysis, new MSD levels are proposed in Table 4 for new 3 bands DL with 2 bands UL inter-band CA combinations with self-interference problems in the TR 36.715-00-02.

Table 4: Expected MSD levels for 3 bands DL/2 bands UL CA band combination w/ potential IMD problems

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B4+B13 | B2 | IMD4 | ¦2*fB2-2*fB13¦ | 1855 | 5 | 25 | 1935 | 5 | 1.5 | N/A |
|  | B13 |  |  | 782 | 5 | 25 | 751 | 5 |  |  |
|  | B4 |  |  | 1746 | 5 | 25 | 2146 | 5 |  |  |
| B2+B4+B13 | B4 | IMD4 | ¦2*fB4 -2*fB13¦ | 1750 | 5 | 25 | 2150 | 5 | 1.5 | N/A |
|  | B13 |  |  | 780 | 5 | 25 | 749 | 5 |  |  |
|  | B2 |  |  | 1860 | 5 | 25 | 1940 | 5 |  |  |
| B2+B4+B5 | B2 | IMD4 | ¦2*fB2 -2*fB5¦ | 1900 | 5 | 25 | 1980 | 5 | 1.5 | N/A |
|  | B5 |  |  | 834 | 5 | 25 | 879 | 5 |  |  |
|  | B4 |  |  | 1732 | 5 | 25 | 2132 | 5 |  |  |
| B2+B5+B66 | B2 | IMD4 | ¦2*fB2 -2*fB5¦ | 1900 | 5 | 25 | 1980 | 5 | 1.5 | N/A |
|  | B5 |  |  | 834 | 5 | 25 | 879 | 5 |  |  |
|  | B66 |  |  | 1712 | 5 | 25 | 2132 | 5 |  |  |
| B2+B5+B66 | B2 | IMD4 | ¦2*fB2 -2*fB5¦ | 1900 | 5 | 25 | 1980 | 5 | 1.5 | N/A |
|  | B5 |  |  | 834 | 5 | 25 | 879 | 5 |  |  |
|  | B66 |  |  | 1712 | 5 | 25 | 2132 | 5 |  |  |
| B2+B5+B46 | B2 | IMD4 | ¦2*fB2 +2*fB5¦ | 1900 | 5 | 25 | 1980 | 5 | 1.5 | N/A |
|  | B5 |  |  | 844 | 5 | 25 | 889 | 5 |  |  |
|  | B46 |  |  | 5488 | 20 | 100 | 5488 | 20 |  |  |
| B2+B5+B46 | B2 | IMD5 | ¦1*fB2 +4*fB5¦ | 1900 | 5 | 25 | 1980 | 5 | 1.0 | N/A |
|  | B5 |  |  | 844 | 5 | 25 | 889 | 5 |  |  |
|  | B46 |  |  | 5276 | 20 | 100 | 5276 | 20 |  |  |
| B5+B46+B66 | B5 | IMD5 | ¦1*fB46 -4*fB5¦ | 834 | 5 | 25 | 879 | 5 | 1.0 | N/A |
|  | B46 |  |  | 5491 | 20 | 100 | 5491 | 20 |  |  |
|  | B66 |  |  | 1755 | 5 | 25 | 2155 | 5 |  |  |
| B5+B46+B66 | B5 | IMD4 | ¦2*fB46 +2*fB5¦ | 834 | 5 | 25 | 879 | 5 | 1.5 | N/A |
|  | B66 |  |  | 1770 | 5 | 25 | 2190 | 5 |  |  |
|  | B46 |  |  | 5208 | 20 | 100 | 5208 | 20 |  |  |
| B5+B46+B66 | B5 | IMD5 | ¦1*fB66 +4*fB5¦ | 846.5 | 5 | 25 | 891.5 | 5 | 1.0 | N/A |
|  | B66 |  |  | 1774 | 5 | 25 | 2194 | 5 |  |  |
|  | B46 |  |  | 5160 | 20 | 100 | 5160 | 20 |  |  |
| B2+B13+B66 | B2 | IMD4 | ¦2*fB2 -2*fB13¦ | 1860 | 5 | 25 | 1940 | 5 | 1.5 | N/A |
|  | B13 |  |  | 782 | 5 | 25 | 751 | 5 |  |  |
|  | B66 |  |  | 1736 | 5 | 25 | 2156 | 5 |  |  |
| B13+B46+B66 | B13 | IMD4 | ¦3*fB2 -1*fB46¦ | 782 | 5 | 25 | 751 | 5 | 1.5 | N/A |
|  | B46 |  |  | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |
|  | B66 |  |  | 5922.5 | 20 | 100 | 5922.5 | 20 |  |  |
| B13+B46+B66 | B13 | IMD5 | ¦2*fB2 -3*fB66¦ | 782 | 5 | 25 | 751 | 5 | 1.0 | N/A |
|  | B66 |  |  | 1727 | 5 | 25 | 2127 | 5 |  |  |
|  | B46 |  |  | 5800 | 20 | 100 | 5800 | 20 |  |  |
| B2+B13+B46 | B2 | IMD4 | ¦2*fB2 +2*fB13¦ | 1905 | 5 | 25 | 1985 | 5 | 1.5 | N/A |
|  | B13 |  |  | 782 | 5 | 25 | 751 | 5 |  |  |
|  | B46 |  |  | 5374 | 20 | 100 | 5374 | 20 |  |  |

In the Table 5, the required MSD levels are proposed into TS36.101 for these inter-band CA band combinations with self-interference problems. The final MSD levels will be averaged with the proposed level for each CA band combination from interested companies.

Table 5: Proposed MSD levels for 3DL/2ULs inter-band CA band combinations in rel-16

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  |  |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |
| CA_2A-5A-46D | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5488 | 20 | 100 | 5488 | 20 | 2.5 | TDD |  |
|  | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD5 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5276 | 20 | 100 | 5276 | 20 | 2.7 | TDD |  |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  | TDD |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 7.5 | FDD |  |
|  | CA_5A_66A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 |  |  |  |
|  |  | 46 | 5208 | 20 | 100 | 5208 | 20 | 2.5 | TDD |  |
|  | CA_5A_66A | 5 | 846.5 | 5 | 25 | 891.5 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1774 | 5 | 25 | 2194 | 5 |  |  |  |
|  |  | 46 | 5160 | 20 | 100 | 5160 | 20 | 0.0 | TDD |  |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  |  |
| CA_13A-46D-66A | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |  |
|  |  | 46 | 5922.5 | 20 | 100 | 5922.5 | 20 | 7.2 | TDD |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1727 | 5 | 25 | 2127 | 5 |  |  |  |
|  |  | 46 | 5800 | 20 | 100 | 5800 | 20 | 0.0 | TDD |  |
| CA_2A-13A-46D | CA_2A-13A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 46 | 5374 | 20 | 100 | 5374 | 20 | 2.5 | TDD |  |

3		Conclusion

In this contribution, we propose the MSD test results for each CA combination with potential IMD problems. The MSD levels can be merged with other MSD test results from interested companies to decide MSD level.

=== Text Proposal for TR36.716-03-02 section 6.x ===

6.x LTE-A inter-band CA: Band 2 and Band 4 and Band 13 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-4A-13A, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 4.

When uplink CA_4A_13A is paired with downlink CA_2A-4A-13A, the 4th order IMD product by Band 4 and band 13 falls into its own Rx frequency band 2.

In Table 6.X.1.3-1, evaluated MSD values of 3 bands DL_CA_2A-4A-13A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-4A-13A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  |  |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  |  |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 4 and Band 5 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-4A-5A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 4.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-4A-5A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-4A-5A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-4A-66A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5A-66A with 2 bands UL is shown.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-5B-66A, the 4th order IMD product by band 2 and band 5 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5B-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-5B-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  |  |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 46 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_5A is paired with downlink CA_2A-5A-46D, the 4th and 5th  order IMD products by band 2 and band 5 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-5A-46D with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-5A-46D

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-5A-46D | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD4 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5488 | 20 | 100 | 5488 | 20 | 2.5 | TDD |  |
|  | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | IMD5 |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 |  |  |  |
|  |  | 46 | 5276 | 20 | 100 | 5276 | 20 | 2.7 | TDD |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 5 and Band 46 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_5A_46A is paired with downlink CA_5A-46D-66A, the 5th order IMD product by band 5 and band 46 falls into the own Rx frequency band 66.

When uplink CA_5A_66A is paired with downlink CA_5A-46D-66A, the 4th and 5th order IMD products by band 5 and band 66 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD values of 3 bands DL_CA_5A-46D-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_5A-46D-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD5 |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  | TDD |  |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 7.5 | FDD |  |
|  | CA_5A_66A | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 |  |  |  |
|  |  | 46 | 5208 | 20 | 100 | 5208 | 20 | 2.5 | TDD |  |
|  | CA_5A_66A | 5 | 846.5 | 5 | 25 | 891.5 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1774 | 5 | 25 | 2194 | 5 |  |  |  |
|  |  | 46 | 5160 | 20 | 100 | 5160 | 20 | 0.0 | TDD |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-66A-66B, the 4th order IMD product by band 2 and band  13 falls into the own Rx frequency band 66.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-13A-66A-66B with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-66A-66B

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 13 and Band 46 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_13A_66A is paired with downlink CA_13A-46D-66A, the 4th and 5th order IMD products by band 13 and band 66 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_13A-46D-66A with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_13A-46D-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_13A-46D-66A | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD4 |
|  |  | 66 | 1713.5 | 5 | 25 | 2113.5 | 5 |  |  |  |
|  |  | 46 | 5922.5 | 20 | 100 | 5922.5 | 20 | 7.2 | TDD |  |
|  | CA_13A-66A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD | IMD5 |
|  |  | 66 | 1727 | 5 | 25 | 2127 | 5 |  |  |  |
|  |  | 46 | 5800 | 20 | 100 | 5800 | 20 | 0.0 | TDD |  |

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 46 DL with 2 bands UL

----- Unchanged sections omitted -----

6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-46D, the 4th order IMD product by band 2 and band 13 falls into the own Rx frequency band 46.

In Table 6.X.1.3-1, evaluated MSD value of 3 bands DL_CA_2A-13A-46D with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-46D

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-46D | CA_2A-13A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  |  |
|  |  | 46 | 5374 | 20 | 100 | 5374 | 20 | 2.5 | TDD |  |

----- End of TP -----

References

- R4-1812214, “TP on summary of interference studies for new x bands DL (x=3,4,5) with 2 bands UL inter-band CA combinations in rel-16” LG Electronics

- TR 36.716-03-02 v0.1.0

- TR 36.715-00-02 v1.0.0

- TS 36.101 v15.4.0