3GPP TSG-RAN WG4 meeting #89	R4-1814820
Spokane, USA, 12 Nov – 16 Nov, 2018

Source:	ZTE Corporation

Title:	Simulation results on NR PUCCH

Agenda item:	7.13.2.3.1

Document for:	Approval

## 1. Background

In RAN4#88bis, the simulation setup was agreed for NR PUCCH demodulation [1] [2] for both FR1 and FR2. In this contribution we provide our corresponding simulation results.

## 2. Discussion

Table – 1 PUCCH simulation results for FR1

| PUCCH FR1 1T2R, TDLC300-100 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | ZTE |
| --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 15kHz, 10MHz | 1 | 1 | 1 |  | 6.5 |
|  |  | 1 | 2 | 1 |  | 4.2 |
|  | 30kHz, 40MHz | 1 | 1 | 1 |  | 6.8 |
|  |  | 1 | 2 | 1 |  | 4.4 |
| Format 1 | 15kHz, 10MHz | 2 | 14 | 1 | ACK miss | -4.4 |
|  |  |  |  |  | NACK to ACK | -5.3 |
|  | 30kHz, 40MHz |  |  |  | ACK miss | -4.5 |
|  |  |  |  |  | NACK to ACK | -5.2 |
| Format 2 | 15kHz, 10MHz | 4 | 1 | 4 | ACK miss | 3.1 |
|  |  |  |  |  | NACK to ACK | 2.5 |
|  |  | 22 | 2 | 9 | BLER@1% | 0.95 |
|  | 30kHz, 40MHz | 4 | 1 | 4 | ACK miss | 3.3 |
|  |  |  |  |  | NACK to ACK | 2.3 |
|  |  | 22 | 2 | 9 | BLER@1% | 1 |
| Format 3 | 15kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS | -0.29 |
|  |  |  |  |  | With additional DMRS | -1.2 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.4 |
|  | 30kHz, 40MHz | 16 | 14 | 1 | Without additional DMRS | -0.1 |
|  |  |  |  |  | With additional DMRS | -1.3 |
|  |  | 16 | 4 | 3 | Without additional DMRS | 0.4 |
| Format 4 | 15kHz, 10MHz | 22 | 14 | 1 | Without additional DMRS | 1.42 |
|  |  |  |  |  | With additional DMRS | -0.6 |
|  | 30kHz, 40MHz |  |  |  | Without additional DMRS | 1.5 |
|  |  |  |  |  | With additional DMRS | -0.8 |
| Format 0 | 15kHz, 5MHz | 1 | 1 | 1 |  | 6.6 |
|  |  | 1 | 2 | 1 |  | 5.05 |
|  | 15kHz, 20MHz | 1 | 1 | 1 |  | 6 |
|  |  | 1 | 2 | 1 |  | 4 |
|  | 30kHz, 10MHz | 1 | 1 | 1 |  |  |
|  |  | 1 | 2 | 1 |  |  |
|  | 30kHz, 20MHz | 1 | 1 | 1 |  |  |
|  |  | 1 | 2 | 1 |  |  |
|  | 30kHz, 100MHz | 1 | 1 | 1 |  |  |
|  |  | 1 | 2 | 1 |  |  |
| Format 1 | 15kHz, 5MHz | 2 | 14 | 1 | ACK miss | -4.2 |
|  |  |  |  |  | NACK to ACK | -4.8 |
|  | 15kHz, 20MHz |  |  |  | ACK miss | -4.3 |
|  |  |  |  |  | NACK to ACK | -5 |
|  | 30kHz, 10MHz |  |  |  | ACK miss | . |
|  |  |  |  |  | NACK to ACK |  |
|  | 30kHz, 20MHz |  |  |  | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  | 30kHz, 100MHz |  |  |  | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
| Format 2 | 15kHz, 5MHz | 4 | 1 | 4 | ACK miss | 3.8 |
|  |  |  |  |  | NACK to ACK | 2.6 |
|  |  | 22 | 2 | 9 | BLER@1% | 1.25 |
|  | 15kHz, 20MHz | 4 | 1 | 4 | ACK miss | 3.6 |
|  |  |  |  |  | NACK to ACK | 2.7 |
|  |  | 22 | 2 | 9 | BLER@1% | 1.2 |
|  | 30kHz, 10MHz | 4 | 1 | 4 | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |
|  | 30kHz, 20MHz | 4 | 1 | 4 | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |
|  | 30kHz, 100MHz | 4 | 1 | 4 | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |
| Format 3 | 15kHz, 5MHz | 16 | 14 | 1 | Without additional DMRS | 0 |
|  |  |  |  |  | With additional DMRS | -0.9 |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |
|  | 15kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS | -0.3 |
|  |  |  |  |  | With additional DMRS | -1.45 |
|  |  | 16 | 4 | 3 | Without additional DMRS | -0.2 |
|  | 30kHz, 10MHz | 16 | 14 | 1 | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |
|  | 30kHz, 20MHz | 16 | 14 | 1 | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |
|  | 30kHz, 100MHz | 16 | 14 | 1 | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |
|  |  | 16 | 4 | 3 | Without additional DMRS |  |
| Format 4 | 15kHz, 5MHz | 22 | 14 | 1 | Without additional DMRS | 1.36 |
|  |  |  |  |  | With additional DMRS | -0.5 |
|  | 15kHz, 20MHz |  |  |  | Without additional DMRS | 1.9 |
|  |  |  |  |  | With additional DMRS | -0.2 |
|  | 30kHz, 10MHz |  |  |  | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |
|  | 30kHz, 20MHz |  |  |  | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |
|  | 30kHz, 100MHz |  |  |  | Without additional DMRS |  |
|  |  |  |  |  | With additional DMRS |  |

Table – 2 PUCCH simulation results for FR2

| PUCCH FR2 1T2R, TDLA30-300 |  | Number of bits | Number of OFDM symbols | Number of PRBs | Note | ZTE |
| --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 60KHz, 100MHz | 1 | 1 | 1 |  |  |
|  |  | 1 | 2 | 1 |  |  |
|  | 120KHz, 100MHz | 1 | 1 | 1 |  | 7.6 |
|  |  | 1 | 2 | 1 |  | 5.3 |
| Format 1 | 60KHz, 100MHz | 2 | 14 | 1 | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  | 120KHz, 100MHz |  |  |  | ACK miss | 0.4 |
|  |  |  |  |  | NACK to ACK |  |
| Format 2 | 60KHz, 100MHz | 4 | 1 | 4 | ACK miss |  |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | BLER@1% |  |
|  | 120KHz, 100MHz | 4 | 1 | 4 | ACK miss | 2.2 |
|  |  |  |  |  | NACK to ACK |  |
|  |  | 22 | 2 | 9 | BLER@1% | 1.1 |
| Format 3 | 60KHz, 100MHz | 16 | 14 | 1 | Without additional DMRS |  |
|  |  | 16 | 4 | 3 |  |  |
|  | 120KHz, 100MHz | 16 | 14 | 1 |  | 1.1 |
|  |  | 16 | 4 | 3 |  | 0.2 |
| Format 4 | 60KHz, 100MHz | 22 | 14 | 1 |  |  |
|  | 120KHz, 100MHz |  |  |  |  | 3.0 |

## 3. Conclusions

In this contribution, simulation results under the agreed setup are provided.

# References

[1] R4-1813755	Way forward on NR BS demodulation general part, Huawei

[2] R4-1813943	Way forward on NR PUCCH demodulation requirements, Ericsson