3GPP TSG-RAN WG4 Meeting #89	R4- 1814923

Spokane, US, 12th – 16th Nov, 2018

Agenda item:	7.13.1.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for PDSCH demodulation tests with follow PMI

Document for:	Discussion

1. Introduction

In this contribution, we present our views on PDSCH demodulation tests with follow PMI.

2. Discussion

Following is an agreement in RAN4 #88 meeting [1] regarding follow PMI test for DDDSU. This was proposed to test quality of CSI for DDDSU that is not supported in CSI test.

Agreement: introduce the PDSCH demodulation test cases with following PMI for TDD configuration of DDDSU for FR2.

In addition, following is the agreed simulation assumptions for PDSCH test for FR2 [2]. In order to guarantee performance gain of PMI selection, it is suitable to design follow PMI test for low rank. One idea is to replace one test case with TDD Config. 1 (DDDSU) to follow PMI. Considering above discussion, we prefer that follow PMI test is designed analogous to either of case 1, 2 or 7.

Table I: Agreed list of simulation cases for FR2 TDD with 100 MHz + 120 kHz

| Case Number | CHBW/ SCS | MIMO | PDSCH type and MCS | Number of layer | Number of PDSCH PRBs | Channel Model | Receiver | TDD Configuration | Max # of HARQ process |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 2 | 100MHz/120kHz | 2Tx 2Rx XPL Med | Type A, 64QAM MCS 18 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 3 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, QPSK MCS 4 | 2 Layers | 6 (in the middle of CBW) | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| 4 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | [8 or 16] |
| 5 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 64QAM MCS 17 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 8 |
| 6 | 100MHz/120kHz | 2Tx 2Rx ULA Med | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | R-ML | 2 | 10 |
| 7 | 100MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 1 Layer | 66 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 16 |
| 8 | 50MHz/120kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 32 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |
| 9 | 200MHz/20kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 132 | TDL-A 30ns, 300Hz | MMSE-IRC | 1 | 8 |
| 10 | 50MHz/60kHz | 2Tx 2Rx ULA Low | Type A, 16QAM MCS 13 | 2 Layers | 66 | TDL-A 30ns, 75Hz | MMSE-IRC | 2 | 10 |

However, these tests are designed for normal demodulation test using random precoding and not optimal for testing performance gain from precoding. Following table lists major parameters that impacts precoding gain. It is preferred that we use test parameters that achieves higher precoding gain.

Table II. Major CSI related parameters

|  | Doppler frequency | CSI-RS periodicity | CSI feedback delay |
| --- | --- | --- | --- |
| Current values in cases 1, 2 and 7 | 300 Hz | 20 ms | [27] slots * |
| Example parameters to achieve higher gain | 100 Hz | 5 slots | [18] slots |

* Assumption from CSI tests

Proposal 1: One additional test is introduced with follow PMI based on PDSCH FR2 test case number 1.

Proposal 2: Replace CSI related parameters as follows.

- Doppler frequency: 100 Hz

- CSI-RS periodicity: 5 slots

- CSI feedback delay: [18] slots

3. Simulation results

Figure 1 shows simulation results for follow PMI tests. We basically follow simulation assumptions for PDSCH simulation for FR2 [2] with additional assumptions in Table AI.

Figure 1: Simulation results for follow PMI

From the results, the required SNR for achieving 70% throughput is -3.2 dB for both of the cases 1 and 2.

Observation: Required SNR for achieving 70% throughput is -3.2 dB for PDSCH FR2 test case number 1 with follow PMI with additional parameters in Table AI.

4. Conclusion

This contribution presents our initial simulation results for follow PMI test. An observation and proposals are derived as follows.

Proposal 1: One additional test is introduced with follow PMI based on PDSCH FR2 test case number 1.

Proposal 2: Replace CSI related parameters as follows.

- Doppler frequency: 100 Hz

- CSI-RS periodicity: 5 slots

- CSI feedback delay: [18] slots

Observation: Required SNR for achieving 70% throughput is -3.2 dB for PDSCH FR2 test case number 1 with follow PMI with additional parameters in Table AI.

References

[1] 3GPP, “Draft meeting report for RAN4 #88,” Oct. 2018.

[2] 3GPP, R4-1814239, “Simulation assumptions for NR PDSCH demodulation performance requirements,” Aug. 2018.

Appendix

Table AI: Simulation parameters (additional parameters based on [2])

| Parameters | Case 1 | Case 2 |
| --- | --- | --- |
| Doppler frequency | 300 Hz | 100 Hz |
| CSI-RS periodicity | 20 ms | 10 slots |
| PRB bundling | WB |  |
| Precoding method | Closed Loop (SP Type I) |  |
| CSI feedback | Wideband |  |
| CSI feedback delay | 27 slots |  |