3GPP TSG RAN WG4 Meeting #89	R4-1815014

Spokane, USA, November 12th – 16th, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability rule on measurement requirements in DRX mode for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The measurement requirements for intra-frequency and inter-frequency in DRX mode are defined based on DRX cycle. However, in case of EN-DC UE may be configured with separate DRX configuration in MN and SN. It is not clear which DRX configuration will apply in the requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the appplicability of DRX cycle in EN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DRX requirements are ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new section 9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

9.6	NSA: Applicability rule on measurement requirements in DRX mode

### 9.6.1	EN-DC

#### 9.6.1.1	Intra-frequency measurement requirements on PCell/PSCell and SCells

If MCG DRX is in use, then the LTE intra-frequency measurement requirements when DRX is in use shall apply and depend on the MCG DRX cycle. Otherwise, the measurement requirements for when DRX is not in use shall apply.

If SCG DRX is in use, then the NR intra-frequency measurement requirements when DRX is in use shall apply and depend on the SCG DRX cycle. Otherwise, the measurement requirements for when DRX is not in use shall apply.

If both MCG and SCG DRX are in use with different value of DRX cycle which is longer than 40ms, intra-frequency measurement requirements when DRX is in use shall apply and depend on the shorter value of DRX cycle.

If both MCG and SCG DRX are in use with different value of DRX cycle and either MCG DRX cycle or SCG DRX cycle is shorter than 40ms, LTE intra-frequency measurement requirements shall apply and depend on MGC DRX cycle and NR intra-frequency measurement requirements shall apply and depend on SCG DRX cycle and NR inter-frequency measurement requirements shall apply and depend on SGC DRX cycle.

#### 9.6.1.2 	Inter-frequency and inter-RAT measurement requirements

If both MCG and SCG DRX are in use with different value of DRX cycle which is longer than 40ms, inter-frequency and inter-RAT measurement requirements when DRX is in use shall apply and depend on the shorter value of DRX cycle.

If both MCG and SCG DRX are in use with different value of DRX cycle and either MCG DRX cycle or SCG DRX cycle is shorter than 40ms, LTE inter-frequency measurement requirements shall apply and depend on MGC DRX cycle and NR inter-frewuency and inter-RAT measurement requirements shall apply and depend on SCG DRX cycle.

If either MCG or SCG is in non-DRX, then LTE inter-frequency measurement requirements shall apply and depend on MGC DRX cycle and state, and NR inter-RAT and NR inter-frequency measurement requirements shall apply and depend on SCG DRX cycle and state.

---END OF CHANGE 1-