3GPP TSG-RAN WG4 Meeting #89	R4-1815201

Spokane, Washington, USA 12 – 16 November 2018

Source:	Ericsson

Title:	TP to TS 38.141-1: Section 4.9.2 2 NR FR1 Test Models

Agenda item:	7.9.3.2

Document for:	Approval

# 1	Introduction

In this contribution some additional updates were made for clarification and missed implementation based upon previous agreements.

# 2	References

- TS 38.141-1 v1.1.0

[Start of Text Proposal]

Table 4.9.2.2-2: Common physical channel parameters for PDCCH

| Parameter |  |
| --- | --- |
| # of symbols used for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| Note: Remaining empty REs in 1st symbol not containing PDCCH will be filled with PDSCH |  |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in each symbol | NRB – 6 |
| DM-RS configuration and density |  | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |  |
| ratio of PDCCH EPRE to DM-RS EPRS | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH

| Parameter |  |
| --- | --- |
| DM-RS configuration and density | Type 1, Comb2 I0 = 2, additional 1 DMRS |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

[Unchanged Sections]

Table 4.9.2.2.2-1: Physical channel parameters of NR- FR1-TM-1.2

| Parameter |  |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | 3 |
| Location of PDSCH RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-PxNRBP |
| Level of deboosting (dB) | 10log10NRB-PxNRBPNRB-103/10PxNRBP |

[Unchanged Sections]

#### 4.9.2.2.7	NR FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter |  |
| --- | --- |
| Percent of 16QAM PDSCH PRBs boosted | x=60% |
| # of 16QAM PDSCH RBGs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -3 |
| Location of 16QAM RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-3/10PxNRBP |

[End of Text Proposal]