3GPP TSG-RAN WG4 Meeting #89	R4-1815230

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	6.5.6.3

Source:	Ericsson

Title:	CQI requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the way forward on eFeMTC UE demodulation and CSI reporting requirements [1].

- Way forward for CQI definition test for CQI table 5 for BL/CE UEs - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 64QAM - Way forward for CQI definition test for CQI table 6 for BL/CE UEs - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 16QAM

This contribution shows our simulation results for new CQI table and discusses the test point.

# 2	Discussion

Figure 1 shows the median of reported CQI for 4-bit CQI table 5 (CQI table for BL/CE UE supporting 64QAM) and CQI table 6 (CQI table for BL/CE UE supporting wide SNR range).

Figure 1	Median CQI for 4-bit CQI table 5 and 6.

## 2.1	Simulation results for CQI table 5 (64QAM)

Table 1 shows the CQI to MCS table used for CQI test for CQI table 5. As we agreed in [1], we allocate 1PRB for PDSCH and available resource elements are 126. Also we agreed to set SNR test points corresponding to 64QAM, the candidate CQI indexes are 11 to 15. We think it is important to set test points where the target coding rate and the actual coding rate is close for CQI definition test. According to Table 1, CQI indexes 11 and 12 are good candidates.

Table 1	CQI to MCS table and effective coding rate for CQI table 5.

|  |  | Allocated PRBs | 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Number of resource elements | 126 |  |  |
| CQI index | Modulation | Target coding rate | MCS | Effective code rate | Abs diff from target CR |
| 0 |  | N/A | DTX |  |  |
| 1 | QPSK | 0.04 | 0 | 0.16 | 0.12 |
| 2 | QPSK | 0.08 | 0 | 0.16 | 0.08 |
| 3 | QPSK | 0.12 | 0 | 0.16 | 0.04 |
| 4 | QPSK | 0.19 | 1 | 0.19 | 0 |
| 5 | QPSK | 0.3 | 4 | 0.32 | 0.02 |
| 6 | QPSK | 0.44 | 5 | 0.38 | 0.06 |
| 7 | QPSK | 0.59 | 8 | 0.57 | 0.02 |
| 8 | 16QAM | 0.37 | 12 | 0.40 | 0.03 |
| 9 | 16QAM | 0.48 | 14 | 0.49 | 0.01 |
| 10 | 16QAM | 0.6 | 16 | 0.60 | 0 |
| 11 | 64QAM | 0.46 | 18 | 0.47 | 0.01 |
| 12 | 64QAM | 0.55 | 21 | 0.57 | 0.02 |
| 13 | 64QAM | 0.65 | 23 | 0.68 | 0.03 |
| 14 | 64QAM | 0.75 | 25 | 0.76 | 0.01 |
| 15 | 64QAM | 0.85 | 27 | 0.85 | 0 |

Table 2 shows our simulation results. It is observed that the median CQI becomes 11 with SNR=11/12dB and 12 with SNR=13/14dB. Considering the CQI index achieving the closest target coding rate, we propose to set SNR=11/12dB.

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=11/12dB. Reuse the same requirements used for the existing CQI definition test.

Table 2	Simulation results of CQI definition test with table 5.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- |
| 11 | 11 | 100% | 0.00 | 1.00 |
| 12 | 11 | 100% | 0.00 | 0.59 |
| 13 | 12 | 100% | 0.00 | 1.00 |
| 14 | 12 | 100% | 0.00 | 0.79 |
| 15 | 13 | 100% | 0.00 | 1.00 |
| 16 | 13 | 100% | 0.00 | 0.80 |
| 17 | 14 | 100% | 0.00 | 1.00 |
| 18 | 14 | 100% | 0.00 | 0.97 |

## 2.2	Simulation results for CQI table 6

For CQI table 6, on the other hand, the agreement is to set SNR test points corresponding to 16QAM (CQI indexes 9 to 12). Table 3 shows the CQI to MCS table used for CQI test for CQI table 5. As we agreed in [1], we allocate 3PRB for PDSCH and available resource elements are 378. As same as CQI table 5, if we consider the minimum difference between target coding rate and effective coding rate, CQI index 9 is the candidates.

Table 3	CQI to MCS table and effective coding rate for CQI table 6.

|  |  | Allocated PRBs | 3 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Number of resource elements | 378 |  |  |
| CQI index | Modulation | Target coding rate | MCS | Effective code rate | Abs diff from target CR |
| 0 |  | N/A | DTX |  |  |
| 1 | QPSK | 0.05 | 0 | 0.10 | 0.05 |
| 2 | QPSK | 0.2 | 2 | 0.22 | 0.02 |
| 3 | QPSK | 0.26 | 3 | 0.26 | 0 |
| 4 | QPSK | 0.19 | 2 | 0.22 | 0.03 |
| 5 | QPSK | 0.14 | 1 | 0.15 | 0.01 |
| 6 | QPSK | 0.26 | 3 | 0.26 | 0 |
| 7 | QPSK | 0.44 | 7 | 0.47 | 0.03 |
| 8 | QPSK | 0.62 | 9 | 0.63 | 0.01 |
| 9 | 16QAM | 0.41 | 12 | 0.40 | 0.01 |
| 10 | 16QAM | 0.544 | 15 | 0.571 | 0.027 |
| 11 | 16QAM | 0.68 | 16 | 0.61 | 0.07 |
| 12 | 16QAM | 0.83 | 16 | 0.61 | 0.22 |
| 13 | 64QAM | 0.64 | 22 | 0.62 | 0.02 |
| 14 | 64QAM | 0.76 | 25 | 0.78 | 0.02 |
| 15 | 64QAM | 0.87 | 27 | 0.83 | 0.04 |

Table 4 shows our simulation results, and it is observed that median CQI becomes 9 if we set SNR=5/6/7dB. Considering the margin, we propose to set SNR=6/7dB.

Proposal 2: For the CQI definition test with CQI table 6, set test points SNR=6/7dB. Reuse the same requirements used for the existing CQI definition test.

Table 4	Simulation results of CQI definition test with table 6.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI -1 | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 4 | 8 | 100 | 0.00 | 0.00 | 0.99 |
| 5 | 9 | 100 | 0.53 | 0.00 | 1.00 |
| 6 | 9 | 100 | 0.00 | 0.00 | 1.00 |
| 7 | 9 | 100 | 0.00 | 0.00 | 0.65 |
| 8 | 10 | 100 | 0.00 | 0.00 | 1.00 |
| 9 | 10 | 100 | 0.00 | 0.00 | 0.99 |
| 10 | 11 | 100 | 0.07 | 0.00 | 1.00 |
| 11 | 11 | 100 | 0.01 | 0.00 | 0.99 |
| 12 | 12 | 100 | 0.04 | 0.00 | 1.00 |

# 3	Summary

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=11/12dB. Reuse the same requirements used for the existing CQI definition test.

Proposal 2: For the CQI definition test with CQI table 6, set test points SNR=6/7dB. Reuse the same requirements used for the existing CQI definition test.

# 4	References

R4-1811372, “Way forward on eFeMTC UE demodulation and CSI”, Ericsson.

# Appendix

Table 5	4-bit CQI Table 3. (CQI table for BL/CE UE introduced in Rel-13).

| CQI index | modulation | code rate x 1024 x | efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | Reserved | Reserved | Reserved |
| 12 | Reserved | Reserved | Reserved |
| 13 | Reserved | Reserved | Reserved |
| 14 | Reserved | Reserved | Reserved |
| 15 | Reserved | Reserved | Reserved |

Table 6	4-bit CQI Table 5. (CQI table for BL/CE UE supporting 64QAM).

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

Table 7	4-bit CQI Table 6. (CQI table for BL/CE UE supporting wide SNR range).

| CQI index | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |