3GPP TSG-RAN WG4 Meeting #89	R4-1815233

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	6.5.7.1

Source:	Ericsson, Sierra Wireless

Title:	Simulation result of BS demodulation requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed with the CR of BS demodulation requirements for eFeMTC [1].

Table 8.2.10-1 Test Parameters for PUSCH

| Parameter | unit | Mode A | Mode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences (Note 2) |  | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions |  | 1 | 16 |
| Frequency hopping |  | ON | ON |
| Frequency hopping interval | subframes | 4: FDD 5: TDD | 4: FDD 5: TDD |
| Note 1: Guard period shall be created according to TS36.211, 5.2.5 [12] Note 2: RV changes per repetition every (RU size x number of RUs) transmission subframes. |  |  |  |

Table A.25-1 FRC parameters for performance requirements (subPRB transmission)

| Reference channel | A25-1 | A25-2 |
| --- | --- | --- |
| Diversity | No | No |
| Allocated resource blocks | 1 | 1 |
| Number of subcarriers | 2 out of 3 | 2 out of 3 |
| DFT-OFDM Symbols per subframe | 12 | 12 |
| Modulation | π/2 BPSK | π/2 BPSK |
| Code rate | 1/3 | 1/3 |
| MCS index | 0 | 0 |
| Payload size (bits) | 32 | 88 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96*2 | 96*2 |
| Total symbols per resource unit | 96*2 | 96*2 |
| Frequency offset | 0 | 0 |
| Number of RUs | 1 | 2 |
| Tx time (ms) | 8 | 16 |

In this contribution, we show our simulation results of PUSCH with 2 of 3 sub carrier Pi/2 BPSK.

# 2	Simulation results

Table 1 is the ideal simulation result of PUSCH with 2 of 3 sub-carrier Pi/2 BPSK.

Table 1	Simulation results of PUSCH with 2 of 3 sub carrier Pi/2 BPSK.

| Channel Bandwidth | Antenna configuration | Propagation condition | 3MHz | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE Mode A | 1x2 | EPA 5Hz Low | -1.9 | -2.3 | -2.4 | -2.7 | -2.6 |
| CE Mode B | 1x2 | EPA 5Hz Low | -11.9 | -12.2 | -12.3 | -12.2 | -12.3 |

# 3	References

R4-1813703, “Introduction of BS demodulation requirements for eFeMTC (TS36.104)”, Ericsson.