3GPP TSG-RAN WG4 Meeting #89	R4-1815240

Spokane, WA, USA, 12 – 16 November 2018

Agenda Item:	7.13.1.3.2

Source:	Ericsson

Title:	Simulation results of NR PBCH demodulation

Document for:	Discussion

# 1	Introduction

RAN4#88bis agreed with the way forward on PBCH demodulation requirements in Rel-15 [1]. We provide the simulation results based on the way forward.

# 2	Discussion

## 2.1	Simulation parameters

Table 1 and Table 2 show the common setup and simulation assumption of PBCH demodulation requirements respectively based on the WF.

Table 1	Common setup for PBCH demodulation requirements.

| Parameters | Unit | Values |
| --- | --- | --- |
| Cell ID |  | 0 |
| CP length |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/BPCH block index | Ms | 0 |
| SS burst periodicity | bits | 20 |
| MIB size (without CRC) | bits | 24 |
| PBCH payload size (including CRC=24bits and 8bits of SS/PBCH block location information) |  | 56 |

Table 2	Simulation assumption for PBCH demodulation requirements.

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency range | FR1 | FR1 | FR2 | FR2 |
| Channel BW | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern (Note 1) | Case A | Case B Case C | Case D | Case E |
| Antenna configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDLC300-100 | TDLA30-10 | TDLA30-300 | TDLA30-75 |
| Note 1: Depending on the operating band. Refers to TS38.101-1 Table 5.4.3.3-1 for which band uses which pattern |  |  |  |  |

## 2.2	Simulation results for 2Rx

Figure 1	Simulation results of PBCH for FR1 (test 1 and test 2).

[Test 3 with TDLA30-300 to be added]

Figure 2	Simulation results of PBCH for FR2 (test 3 and test 4).

Figure 1 shows the simulation results of PBCH for FR1 and Figure 2 shows the simulation results of PBCH for FR2. For all the results, we also show SSB index detection performance, PBCH single-shot with known DMRS sequence, and PBCH multi-shot (4 attempts) with known DMRS sequence. Considering the initial cell search assumption, UE usually does not know the SSB index. In this case PBCH demodulation performance is given by the combined PBCH demodulation given by (1-Pssb) + Pssb*PBCH_BLER, where Pssb is the SSB index acquisition success rate and PBCH_BLER is the PBCH BLER with the assumption UE knows the DMRS sequence (= SSB index). On the other hand, if UE knows the position of SSB index with some reason, the PBCH demodulation performance is same as PBCH BLER.

From the simulation results, it is observed that PBCH multi-shot performance (without SSB index acquisition) is better than SSB index acquisition performance. This means the required SNR for 1% is different depending on the assumption RAN4 assumes UE knows SSB index or not. Note when RAN4 discussed MIB acquisition time in RRM, we assumed single-shot performance only [2].

Observation: With and without the SSB index knowledge, the performance difference of PBCH demodulation with regard to Pm-bch of 1% is X dB with soft-combining within MIB TTI of 80ms.

Since PBCH performance is directly linked to RRM performance, we propose to specify two sets of PBCH demodulation requirements with/without the SSB index knowledge.

## 2.3	Simulation results for 4Rx

[Simulation results to be added]

## 2.4	Summary of simulation results

Table 3 summarize the simulation results to achieve 1% Pm-bch with/without SSB block index knowledge.

Table 3	Summary of simulation results

|  | Test 1 (FR1, SSB SCS=15kHz) | Test 2 (FR1, SSB SCS=30kHz) | Test 3 (FR2, SSB SCS=120kHz) | Test 4 (FR2, SSB SCS=240kHz) |
| --- | --- | --- | --- | --- |
| 2Rx with SSB index knowledge |  |  |  |  |
| 2Rx without SSB index knowledge |  |  |  |  |
| 4Rx with SSB index knowledge |  |  | N/A | N/A |
| 4Rx without SSB index knowledge |  |  | N/A | N/A |

# 3	Conclusion

Observation: With and without the SSB index knowledge, the performance difference of PBCH demodulation with regard to Pm-bch of 1% is X dB with soft-combining within MIB TTI of 80ms.

Proposal 1: RAN4 specify BPCH demodulation requirement with/without the SSB index knowledge.

# References

- R4-1814240, “Way forward on NR PBCH demodulation requirements”, CMCC, Huawei, HiSilicon, China Telecom, ZTE, CATT, NTT DOCOMO, Vodafone, Orange, Sprint, KDDI, SoftBank.

- R4-1800342, “SSB index acquisition time requirements”, Ericsson.