3GPP TSG-RAN WG4 WG4 #89 	R4-1815242

# Spokane, WA, USA, 12 – 16 November 2018

Source:	Ericsson

Title:	TP to TS 38.101-4: 7.4 FR2 PBCH demodulation requirements

Agenda item:	7.13.1.3.3

Document for:	Approval

## Background

This TP provides the PBCH demodulation requirements FR2.

## Summary

This TP specifies the PBCH demodulation requirements for FR2. This TP also specifies the RMC used for PBCH demodulation requirements.

## Proposal

It is proposed that the attached text proposal is included in TS 38.101-4

## References

- R4-1814240, “Way forward on NR PBCH demodulation requirements”, CMCC, Huawei, HiSilicon, China Telecom, ZTE, CATT, NTT DOCOMO, Vodafone, Orange, Sprint, KDDI, SoftBank.

TEXT PROPOSAL:

<GUIDANCE: 
Add the actual Text Proposal here by pasting in report/specification text and then edit it.
Use “End of Change” / ”Next changed section” if there are updates to separated sections.

For Text Proposals for the TR:

- Describe what the requirements should be and how it is to be expressed.

- Give the background for the choice of requirement, if applicable. This could be references to regulation, LTE requirements other TRs etc. If there are existing WF agreements, copy them into the text instead of referencing.

- Do not use words such as “proposed”, “for agreement”, etc. The text should be descriptive and be proposed for agreement and then go into the TR as is.

- Do not write the final specification text here, but example tables for requirements can be OK..

For Text Proposals for the TS:

- Write text for the requirement as complete as possible. If it cannot be fully complete at this time, it can be submitted as a draft for discussion.>

*********************End of change*****************

## 7.4 	PBCH demodulation requirements

The receiver characteristics of PBCH are determined by the probability of miss-detection of the PBCH (Pm-bch), which is defined as

Pm-bch=1-AB

Where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs. The Pm-bch is derived with the assumption UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms).

### 7.4.1	1RX requirements (Void)

### 7.4.2	2RX requirements

#### 7.4.2.1	FDD (Void)

#### 7.4.2.2	TDD

Table 7.4.2.2-1 Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | FR2.120-1 |
| Note 1 as specified in 4.1 in TS 38.213 [x] Note 2 as specified in 11.1 in TS 38.213 [x] |  |  |

For the parameters specified in Table 7.4.2.2-1 the average probability of a miss-detected PBCH (Pm-bch) shall be below the specified values in Table 7.4.2.2-2 in case SS/PBCH block index is known and in Table.7.4.2.2-3 in case SS/PBCH block index is not known. The downlink physical setup is in accordance with Annex C.3.1.

Table 7.4.2.2-2	Minimum performance PBCH in case SS/PBCH block index is known

| Test number | Bandwidth | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | PBCH SNR (dB) |
| 1 | 100 MHz | R.PBCH.5 | [TDLA30-300] | 1 x 2 Low | 1 | TBD |
| 2 | 100 MHz | R.PBCH.6 | [TDLA30-75] | 1 x 2 Low | 1 | TBD |

Table 7.4.2.2-3	Minimum performance PBCH in case SS/PBCH block index is not known

| Test number | Bandwidth | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | PBCH SNR (dB) |
| 1 | 100 MHz | R.PBCH.5 | [TDLA30-300] | 1 x 2 Low | 1 | TBD |
| 2 | 100 MHz | R.PBCH.6 | [TDLA30-75] | 1 x 2 Low | 1 | TBD |

*********************Next changed section*****************

### A.x.x	Reference measurement channels for PBCH demodulation requirements

#### A.x.x.2	Reference measurement channels for FR2

Table A.x.x-1: PBCH Reference Channel

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | [R.PBCH.5] | [R.PBCH.6] |
| SS/PBCH block subcarrier spacing | kHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

*********************Next changed section*****************