3GPP TSG-RAN WG4 Meeting #89                     	R4-1815272

Spokane, US, Nov 12th –16th, 2018

Agenda item:            7.13.1.3

Source:	MediaTek Inc.

Title:	NR PDCCH simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation result of PDCCH.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of PDCCH performance. Table A.1 (in Appendix) is the test case list. Table A.2 (in Appendix) contains other detailed parameter setting in the simulation.

Table 2.1 is the 1% BLER SNR values for all tests and Figure A.2.1~Figure A.2.16 (in Appendix) are the corresponding performance curves.

Table 2.1 1% BLER SNR of all test cases

|  | 2RX | 4RX |
| --- | --- | --- |
| Case Number | 1% BLER SNR | 1% BLER SNR |
| 1 | 6.67 | 0.66 |
| 2 | 0.86 | -3.67 |
| 3 | 6.62 | 1.2 |
| 4 | 4.76 | -0.92 |
| 5 | -2.72 | -6.36 |
| 6 (Option 1) | 5.46 | 1.12 |
| 6 (Option 2) | 5.63 | 0.99 |
| 7 (Option 1) | 1.83 | -1.74 |
| 7 (Option 2) | 1.40 | -2.14 |
| 8 | -2.13 | -5.35 |
| 9 | 4.74 | N/A |
| 10 | 2.03 | N/A |
| 11 | -1.30 | N/A |
| 12 | 4.05 | -0.79 |
| 13 | -0.31 | -1.64 |
| 14 | -2.09 | -6.63 |
| 15 | -5.44 | -7.85 |
| 16 | -4.77 | N/A |

# 3	Conclusions

In the contribution, we provide our simulation result of PDCCH.

# 4	Reference

[1] R4-1814213, “Way forward on NR PDCCH demodulation requirements”, Ericsson.

Appendix

Table A.1 PDCCH test cases

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 6 | 30 | 1_0 | Opt1: 102 Opt2: 90 | 41 | 1 | 2 | Interleaved | Option 1: 2 Option 2: 6 | 3 | TDL-A, 30ns, 10Hz | 1x2 Low | 1x4 Low |
| 7 | 30 | 1_1 | Opt1: 102 Opt2: 90 | 52 | 1 | 4 | Interleaved | Option 1: 2 Option 2: 6 | 3 | TDL-C, 300ns, 100Hz | 1x2 Low | 1x4 Low |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved | 6 | 3 | TDL-C, 300ns, 100Hz | 2x2 Low | 2x4 Low |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | 3 | [TDL-A, 30ns, 75Hz] | 1x2 Low | NA |
| 10 | 120 | 1_1 | 60 | 51 | 1 | 4 | Interleaved | 6 | 2 | [TDL-A, 30ns, 300Hz] | 1x2 Low | NA |
| 11 | 120 | 1_1 | 60 | 51 | 1 | 8 | Interleaved | 2 | 3 | [TDL-A, 30ns, 75Hz] | 2x2 Low | NA |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | [TDL-A, 30ns, 10Hz] | 1x2 Low | 1x4 Low |
| 13 | 15 | 1_1 | 48 | 50 | 1 | 8 for 2Rx and 4 for 4Rx | Non-interleaved | 6 | NA | [TDL-A, 30ns, 10Hz] | 2x2 Low | 2x4 Low |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | [TDL-A, 30ns, 10Hz] | 1x2 Low | FFS |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | [TDL-C, 300ns, 100Hz] | 1x2 Low | FFS |
| 16 | 120 | 1_0 | 60 | 40 | 2 | 16 | Interleaved | 2 | 3 | [TDL-A, 30ns, 75Hz] | 2x2 Low | NA |

Table A.2 PDCCH simulation detailed parameter setting

| Items | Setting | Notes |
| --- | --- | --- |
| BW and SCS combination | - FR1 FDD: 15kHz + 10MHz - FR1 TDD: 30kHz + 40MHz - FR2 TDD: 120hHz + 100MHz |  |
| TDD UL/DL configurations | - FR1 TDD 30kHz SCS - Slot format = {DDDDDDDSUU} - S = {D6, G4, U4} - FR2 TDD 120kHz SCS - Slot pattern = {DDDSU} - S ={D10, G2, U2} |  |

Figure A.2.1 PDCCH BLER - Test case 1

Figure A.2.2 PDCCH BLER - Test case 2

Figure A.2.3 PDCCH BLER - Test case 3

Figure A.2.4 PDCCH BLER - Test case 4

Figure A.2.5 PDCCH BLER - Test case 5

Figure A.2.6 PDCCH BLER - Test case 6-1

Figure A.2.6 PDCCH BLER - Test case 6-2

Figure A.2.7 PDCCH BLER - Test case 7-1

Figure A.2.9 PDCCH BLER - Test case 7-2

Figure A.2.10 PDCCH BLER - Test case 8

Figure A.2.11 PDCCH BLER - Test case 9

Figure A.2.12 PDCCH BLER - Test case 10

Figure A.2.13 PDCCH BLER - Test case 11

Figure A.2.14 PDCCH BLER - Test case 12

Figure A.2.15 PDCCH BLER - Test case 13

Figure A.2.16 PDCCH BLER - Test case 14

Figure A.2.16 PDCCH BLER - Test case 14

Figure A.2.16 PDCCH BLER - Test case 16