3GPP TSG-RAN WG4 Meeting #89                 	R4-1815349

Spokane, US, 12 - 16 November 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUSCH simulation results

Agenda item:	7.13.2.2.1

Document for:	Discussion

Introduction

In the RAN4#88-bis meeting, a way forward was agreed for NR PUSCH demodulation in [1]. In this contribution we provide simulation results with the simulation assumptions agreed in the WF.

Simulation Results (ideal)

Transform precoding disabled

For transform precoding disabled case, the following simulation assumptions were agreed in [1]:

Table 1: Simulation assumptions for simulation alignment – Transform precoding disabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1, 2 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1, 2 | 1, 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz 30kHz: 10MHz, 20MHz, 40MHz, 100MHz 60kHz(FR2): 50MHz, 100MHz 120kHz: 50MHz, 100MHz, 200MHz |  |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results with these parameters are shown in Table 2 to Table 12.

Table 2: Simulation results for Transform precoding disabled case 1T2R, high priority

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,16 |
|  |  |  |  |  | DMRS 1+1 | -3,42 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,24 |
|  |  |  |  |  | DMRS 1+1 | 9,51 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,49 |
|  |  |  |  |  | DMRS 1+1 | 12,41 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,92 |
|  |  |  |  |  | DMRS 1+1 | -3,57 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,49 |
|  |  |  |  |  | DMRS 1+1 | 9,23 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,65 |
|  |  |  |  |  | DMRS 1+1 | 12,39 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -2,78 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 10,81 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 13,09 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -2,81 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 9,86 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,81 |

Table 3: Simulation results for Transform precoding disabled case 1T2R, low priority

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,58 |
|  |  |  |  |  | DMRS 1+1 | -3,05 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,76 |
|  |  |  |  |  | DMRS 1+1 | 9,55 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,38 |
|  |  |  |  |  | DMRS 1+1 | 12,33 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 3,57 |
|  |  |  |  |  | DMRS 1+1 | -2,62 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 11,4 |
|  |  |  |  |  | DMRS 1+1 | 9,41 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,59 |
|  |  |  |  |  | DMRS 1+1 | 12,36 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,17 |
|  |  |  |  |  | DMRS 1+1 | -3,48 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,42 |
|  |  |  |  |  | DMRS 1+1 | 9,45 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,21 |
|  |  |  |  |  | DMRS 1+1 | 12,34 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,29 |
|  |  |  |  |  | DMRS 1+1 | -3,69 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,6 |
|  |  |  |  |  | DMRS 1+1 | 9,4 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 12,48 |
|  |  |  |  |  | DMRS 1+1 | 12,4 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,33 |
|  |  |  |  |  | DMRS 1+1 | -3,78 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,49 |
|  |  |  |  |  | DMRS 1+1 | 9,1 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 13,2 |
|  |  |  |  |  | DMRS 1+1 | 12,95 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -2,17 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 10,51 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,94 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -2,32 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 9,83 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 12,83 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 |  |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 |  |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 |  |

Table 4: Simulation results for Transform precoding disabled case 1T4R, high priority

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,99 |
|  |  |  |  |  | DMRS 1+1 | -6,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,81 |
|  |  |  |  |  | DMRS 1+1 | 5,11 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,63 |
|  |  |  |  |  | DMRS 1+1 | 8,61 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,32 |
|  |  |  |  |  | DMRS 1+1 | -6,94 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5 |
|  |  |  |  |  | DMRS 1+1 | 4,93 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,48 |
|  |  |  |  |  | DMRS 1+1 | 8,3 |

Table 5: Simulation results for Transform precoding disabled case 1T4R, low priority

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,63 |
|  |  |  |  |  | DMRS 1+1 | -6,49 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,68 |
|  |  |  |  |  | DMRS 1+1 | 5,11 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,46 |
|  |  |  |  |  | DMRS 1+1 | 8,42 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,06 |
|  |  |  |  |  | DMRS 1+1 | -6,21 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,85 |
|  |  |  |  |  | DMRS 1+1 | 5,01 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,43 |
|  |  |  |  |  | DMRS 1+1 | 8,26 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,49 |
|  |  |  |  |  | DMRS 1+1 | -6,79 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,94 |
|  |  |  |  |  | DMRS 1+1 | 5,07 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,37 |
|  |  |  |  |  | DMRS 1+1 | 8,55 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,62 |
|  |  |  |  |  | DMRS 1+1 | -7,04 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,13 |
|  |  |  |  |  | DMRS 1+1 | 5,05 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,56 |
|  |  |  |  |  | DMRS 1+1 | 8,5 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,8 |
|  |  |  |  |  | DMRS 1+1 | -7,04 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,03 |
|  |  |  |  |  | DMRS 1+1 | 4,92 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 8,94 |
|  |  |  |  |  | DMRS 1+1 | 8,79 |

Table 6: Simulation results for Transform precoding disabled case 1T8R, high priority

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,18 |
|  |  |  |  |  | DMRS 1+1 | -9,67 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,94 |
|  |  |  |  |  | DMRS 1+1 | 1,61 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,84 |
|  |  |  |  |  | DMRS 1+1 | 4,81 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,25 |
|  |  |  |  |  | DMRS 1+1 | -9,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,53 |
|  |  |  |  |  | DMRS 1+1 | 1,51 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,77 |
|  |  |  |  |  | DMRS 1+1 | 4,57 |

Table 7: Simulation results for Transform precoding disabled case 1T8R, low priority

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,99 |
|  |  |  |  |  | DMRS 1+1 | -9,52 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,84 |
|  |  |  |  |  | DMRS 1+1 | 1,55 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,72 |
|  |  |  |  |  | DMRS 1+1 | 4,7 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,92 |
|  |  |  |  |  | DMRS 1+1 | -9,26 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,89 |
|  |  |  |  |  | DMRS 1+1 | 1,46 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,71 |
|  |  |  |  |  | DMRS 1+1 | 4,55 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,47 |
|  |  |  |  |  | DMRS 1+1 | -9,74 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,5 |
|  |  |  |  |  | DMRS 1+1 | 1,65 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,57 |
|  |  |  |  |  | DMRS 1+1 | 4,74 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,57 |
|  |  |  |  |  | DMRS 1+1 | -9,94 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,63 |
|  |  |  |  |  | DMRS 1+1 | 1,6 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 4,75 |
|  |  |  |  |  | DMRS 1+1 | 4,68 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,88 |
|  |  |  |  |  | DMRS 1+1 | -9,88 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,58 |
|  |  |  |  |  | DMRS 1+1 | 1,49 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 5,11 |
|  |  |  |  |  | DMRS 1+1 | 4,96 |

Table 8: Simulation results for Transform precoding disabled case 2T2R, high priority

| PUSCH 2T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | Na |
|  |  |  |  |  | DMRS 1+1 | 1,29 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | Na |
|  |  |  |  |  | DMRS 1+1 | 18,53 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | 2,63 |
|  |  |  |  |  | DMRS 1+1 | 0,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | Na |
|  |  |  |  |  | DMRS 1+1 | 17,69 |
|  | FR2 | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | 1,34 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | Na |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | 0,97 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 21,04 |

Table 9: Simulation results for Transform precoding disabled case 2T2R, low priority

| PUSCH 2T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 5,8 |
|  |  |  |  |  | DMRS 1+1 | 0,81 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 18,17 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 1,61 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 18,45 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | 0,94 |
|  |  |  |  |  | DMRS 1+1 | 0,2 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 18,05 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 1,78 |
|  |  |  |  |  | DMRS 1+1 | 0,44 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 17,83 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | 3,42 |
|  |  |  |  |  | DMRS 1+1 | 0,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | 18,1 |
|  | FR2 | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 1,49 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NA |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 1,08 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 20,63 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 |  |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 |  |

Table 10: Simulation results for Transform precoding disabled case 2T4R, high priority

| PUSCH 2T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | 5,14 |
|  |  |  |  |  | DMRS 1+1 | -2,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 17,09 |
|  |  |  |  |  | DMRS 1+1 | 9,93 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,12 |
|  |  |  |  |  | DMRS 1+1 | -3,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,16 |
|  |  |  |  |  | DMRS 1+1 | 9,71 |

Table 11: Simulation results for Transform precoding disabled case 2T4R, low priority

| PUSCH 2T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,75 |
|  |  |  |  |  | DMRS 1+1 | -3,15 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,95 |
|  |  |  |  |  | DMRS 1+1 | 9,84 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NA |
|  |  |  |  |  | DMRS 1+1 | -2,63 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 16,11 |
|  |  |  |  |  | DMRS 1+1 | 9,88 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,09 |
|  |  |  |  |  | DMRS 1+1 | -3,55 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,31 |
|  |  |  |  |  | DMRS 1+1 | 9,82 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,51 |
|  |  |  |  |  | DMRS 1+1 | -3,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,28 |
|  |  |  |  |  | DMRS 1+1 | 9,81 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,66 |
|  |  |  |  |  | DMRS 1+1 | -3,43 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,67 |
|  |  |  |  |  | DMRS 1+1 | 9,94 |

Table 12: Simulation results for Transform precoding disabled case 2T8R

| PUSCH 2T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | FR1 | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,04 |
|  |  |  |  |  | DMRS 1+1 | -6,28 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,59 |
|  |  |  |  |  | DMRS 1+1 | 5,41 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,49 |
|  |  |  |  |  | DMRS 1+1 | -6,5 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,47 |
|  |  |  |  |  | DMRS 1+1 | 5,3 |

Transform precoding enabled

For transform precoding enabled case, the following simulation assumptions were agreed in [1]:

Table 13: Simulation assumptions for simulation alignment – Transform precoding enabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | Type 1 |
| Number of DMRS symbols | 1+1, 1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz | 60kHz: 50 120kHz: 100 |
| PTRS | Not configured | NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results with these parameters are shown in Table 14 to Table 16.

Table 14: Simulation results for Transform precoding enabled case 1T2R

| PUSCH 1T2R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -0,43 |
|  |  |  |  |  | DMRS 1+1 | -2,99 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -2,93 |
|  |  |  |  |  | DMRS 1+1 | -3,28 |
|  | FR2 | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -1,4 |
|  |  | 120kHz, 100MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -1,57 |

Table 15: Simulation results for Transform precoding enabled case 1T4R

| PUSCH 1T4R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -4,58 |
|  |  |  |  |  | DMRS 1+1 | -6,49 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -6,45 |
|  |  |  |  |  | DMRS 1+1 | -6,75 |

Table 16: Simulation results for Transform precoding enabled case 1T8R

| PUSCH 1T8R |  |  |  |  |  | SINR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | FR1 | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -8,02 |
|  |  |  |  |  | DMRS 1+1 | -9,53 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -9,48 |
|  |  |  |  |  | DMRS 1+1 | -9,74 |

Conclusion

In this contribution we have provided simulation results for NR PUSCH.

- References

- R4-1813942, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018