3GPP TSG-RAN WG4 Meeting #89 	R4-1815459

# Spokane, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6127 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced PHR for category NB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Perf |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for enhanced PHR reporting is introduced in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR was endorsed at last meeting (R4-1813988) and resubmitted for approval. Enhanced PHR reporting is introduced by using the new bits to improve the granularities and ranges of current PHR tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced PHR won’t be supported, i.e. reporting will be done using 4 values only. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR was endorsed at last meeting (R4-1813988) and resubmitted for approval. |  |  |  |  |  |  |  |  |

--- Start of Change 1 ---

#### 9.1.23.3	Report Mapping for UE Category NB1

The power headroom reporting range is from [-54] ...+11 dB for UE category NB1 when the enhanced coverage level 0 is selected during the random access procedure [17]. The report mapping is defined in Table 9.1.23.3-1, and in Table 9.1.23.3-1A for UEs supporting enhanced PHR [31].

Table 9.1.23.3-1: Power headroom report mapping for UE category NB1 when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

Table 9.1.23.3-1A: Power headroom report mapping for UE category NB1 when the enhanced coverage level 0 is selected during random access procedure [17] for UEs supporting enhanced PHR [31]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH [-37] |
| POWER_HEADROOM_1 | [-37] PH [-33] |
| POWER_HEADROOM_2 | [-33] PH [-29] |
| POWER_HEADROOM_3 | [-29] PH [-25] |
| POWER_HEADROOM_4 | [-25] PH [-21] |
| POWER_HEADROOM_5 | [-21] PH [-17] |
| POWER_HEADROOM_6 | [-17] PH [-13] |
| POWER_HEADROOM_7 | [-13] PH [-9] |
| POWER_HEADROOM_8 | [-9] PH [-5] |
| POWER_HEADROOM_9 | [-5] PH [-1] |
| POWER_HEADROOM_10 | [-1] PH [3] |
| POWER_HEADROOM_11 | [3] PH [7] |
| POWER_HEADROOM_12 | [7] PH [11] |
| POWER_HEADROOM_13 | [11] PH [15] |
| POWER_HEADROOM_14 | [15] PH [19] |
| POWER_HEADROOM_15 | PH ≥ [19] |

The power headroom reporting range is from [-54] ...+6 dB for UE category NB1 when enhanced coverage level other than 0 is selected during the random access procedure [17]. The report mapping is defined in Table 9.1.23.3-2, and in Table 9.1.23.3-2A for UEs supporting enhanced PHR [31].

Table 9.1.23.3-2: Power headroom report mapping for UE category NB1 when the enhanced coverage level other than 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

Table 9.1.23.3-2A: Power headroom report mapping for UE category NB1 when the enhanced coverage level other than 0 is selected during random access procedure [17] for UEs supporting enhanced PHR [31]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH [-45] |
| POWER_HEADROOM_1 | [-45] PH [-41] |
| POWER_HEADROOM_2 | [-41] PH [-37] |
| POWER_HEADROOM_3 | [-37] PH [-33] |
| POWER_HEADROOM_4 | [-33] PH [-29] |
| POWER_HEADROOM_5 | [-29] PH [-25] |
| POWER_HEADROOM_6 | [-25] PH [-21] |
| POWER_HEADROOM_7 | [-21] PH [-17] |
| POWER_HEADROOM_8 | [-17] PH [-13] |
| POWER_HEADROOM_9 | [-13] PH [-9] |
| POWER_HEADROOM_10 | [-9] PH [-5] |
| POWER_HEADROOM_11 | [-5] PH [-1] |
| POWER_HEADROOM_12 | [-1] PH [3] |
| POWER_HEADROOM_13 | [3] PH [7] |
| POWER_HEADROOM_14 | [7] PH [11] |
| POWER_HEADROOM_15 | PH ≥ [11] |

##### 9.1.23.3.1	Void

##### 9.1.23.3.2	Void

#### 9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6

The power headroom reporting range is -54 … +11 dB for UE category NB1 when the enhanced coverage level 0 is selected during the random access procedure [17] for UE power class 6 [5]. The report mapping is defined in Table 9.1.23.4-1, and in Table 9.1.23.4-1A for UEs supporting enhanced PHR [31].

Table 9.1.23.4-1: Power headroom report mapping for UE category NB1 when the enhanced coverage level 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH [5] |
| POWER_HEADROOM_1 | [5] PH [8] |
| POWER_HEADROOM_2 | [8] PH [11] |
| POWER_HEADROOM_3 | PH ≥ [11] |

Table 9.1.23.4-1A: Power headroom report mapping for UE category NB1 for UE PC6 and supporting enhanced PHR [31]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH [-45] |
| POWER_HEADROOM_1 | [-45] PH [-41] |
| POWER_HEADROOM_2 | [-41] PH [-37] |
| POWER_HEADROOM_3 | [-37] PH [-33] |
| POWER_HEADROOM_4 | [-33] PH [-29] |
| POWER_HEADROOM_5 | [-29] PH [-25] |
| POWER_HEADROOM_6 | [-25] PH [-21] |
| POWER_HEADROOM_7 | [-21] PH [-17] |
| POWER_HEADROOM_8 | [-17] PH [-13] |
| POWER_HEADROOM_9 | [-13] PH [-9] |
| POWER_HEADROOM_10 | [-9] PH [-5] |
| POWER_HEADROOM_11 | [-5] PH [-1] |
| POWER_HEADROOM_12 | [-1] PH [3] |
| POWER_HEADROOM_13 | [3] PH [7] |
| POWER_HEADROOM_14 | [7] PH [11] |
| POWER_HEADROOM_15 | PH ≥ [11] |

The power headroom reporting range is from [-54] ...0 dB for UE category NB1 when the enhanced coverage level other than 0 is selected during the random access procedure [17] for UE power class of 6 [5]. The report mapping is defined in Table 9.1.23.4-2, and in Table 9.1.23.4-2A for UEs supporting enhanced PHR [31].

Table 9.1.23.4-2: Power headroom report mapping for UE category NB1 when the enhanced coverage level other than 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | [-54] PH [-20] |
| POWER_HEADROOM_1 | [-20] PH [-10] |
| POWER_HEADROOM_2 | [-10] PH [0] |
| POWER_HEADROOM_3 | PH ≥ [0] |

--- End of Change 1 ---