3GPP TSG-RAN4 Meeting #89		R4-1815736

# Spokane, WA, USA, November 12-16, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability rules for RRM test cases with Different Duplex Modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To define applicability rules for RRM test cases defined with different duplex modes or their combinations to verify the same requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability rules are defined for: - Single carrier RRM test cases which are defined with different duplex modes or their combinations to verify the same requirements. - EN-DC RRM test cases which are defined with different duplex modes or their combinations to verify the same requirements. SA RRM test cases which are defined with different duplex modes or their combinations to verify the same requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will have to pass several test cases to verify the same requirements. This will increase UE complexity and testing effort. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.4, A.3.4.1, A.3.4.2, A3.4.3, A3.4.1.1, A3.4.1.2, A.3.4.2.1, A.3.4.2.2, A3.4.3.1, A3.4.3.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

## A.3.4	Test Cases with Different Duplex Modes

## A.3.4.1	Single Carrier Test Cases with Different Duplex Modes

### A.3.4.1.1	 Introduction

In Annex A single carrier test cases for inter-frequency or inter-RAT measurements may be defined for different duplex modes or combination of duplex modes (FDD, TDD and TDD-FDD) used for serving and non-serving carriers to verify the same RRM requirement.

### A.3.4.1.2  Principle of testing

If multiple single carrier test cases for inter-frequency or inter-RAT measurements are defined for different duplex modes (FDD or TDD) or for combination of duplex modes (TDD-FDD) to verify the same RRM requirement which is independent of the duplex mode and is identical for different duplex modes or combination of duplex modes, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the corresponding test case(s) with one of the duplex modes supported by the UE.

## A.3.4.2	EN-DC Test Cases with Different Duplex Modes

### A.3.4.2.1	 Introduction

In Annex A EN-DC test cases may be defined for different duplex modes or combination of duplex modes (FDD, TDD and TDD-FDD) to verify the same RRM requirement.

### A.3.4.2.2  Principle of testing

If multiple EN-DC test cases are defined for different duplex modes (FDD or TDD) or for combination of duplex modes (TDD-FDD) to verify the same RRM requirement which is independent of the duplex mode and is identical for different duplex modes or combination of duplex modes, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the corresponding test case(s) with one of the duplex modes supported by the UE.

## A.3.4.3	Carrier Aggregation Test Cases with Different Duplex Modes

### A.3.4.3.1	Introduction

In Annex A carrier aggregation test cases may be defined for different duplex modes or combination of duplex modes (NR FDD, NR TDD and NR TDD-FDD) to verify the same RRM requirement.

### A.3.4.3.2	Principle of testing

If multiple carrier aggregation test cases are defined for different duplex modes (NR FDD or NR TDD) or for combination of duplex modes (NR TDD-FDD) to verify the same RRM requirement which is independent of the duplex mode and is identical for different duplex modes or combination of duplex modes, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the corresponding test case(s) with one of the duplex modes supported by the UE.

---------------------------END OF CHANGES----------------------------