3GPP TSG-RAN4 Meeting #89		R4-1815774

Spokane, USA, 12 – 16 November 2017

Title:	Reply LS on IBE model for NR V2X

Response to:	R1-1810006

Release:	Rel-16

Work Item:	FS_NR_V2X

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Chunhui ZHANG

E-mail Address:	Chunhui.zhang@ericsson.com

1. Overall Description:

RAN4would like to thank RAN1 for the LS on IBE model (R1-1810006) and provide following answer in RAN4 LS:

Answer for [Question 1]

AGC settling time:

The AGC settling time is bounded with # of symbol with relation to different SCS, for example:

SCS=15kHz; AGC settling time= 1 OFDM symbol

TX/RX switching time:

For FR1 will be 13 us and for FR2 will be 7 us.

Timing error between a UE and its synchronization reference:

Depending the reference type of the V2X UE, the time error relative to its reference will be:

- GNSS: up to ± 390ns

- Serving Cell/PCell (LTE): up to ± 390ns, exact number refer to ±Te in the Table 7.1.2-1 in 36.133

- Serving Cell/Pcell (NR): up to ± 390ns, exact number refer to ±Te  in Table 7.1.2-1 in 38.133

Frequency error between a UE and its synchronization reference:

The UE modulated carrier frequency shall be accurate to within ±0.1 ppm observed over a period of 1 msec

compared to the carrier frequency received from synchronization reference.

Answer for [Question 2]

It is recommended to adapt the IBE model defined in Section A.2.1.5 in TR 36.843 to the IBE requirement in 6.4.2.3 in 38.101-1 for FR1.

Answer for [Question 3]

It is recommended to adapt the IBE model defined in Section A.2.1.5 in TR 36.843 to the IBE requirement in 6.4.2.3 in 38.101-2 for FR2.

2. Actions:

To RAN1 WG:

RAN4 kindly asks RAN1 to take the above information into consideration.

3. Date of Next RAN4 Meetings:

TSG-RAN4 Meeting #90 	Feb 25   – Mar 1, 2019		Athens, Greece

TSG-RAN4 Meeting #90b 	Apri  8   – Apri 12, 2019		 China