3GPP TSG-RAN WG4 WG4 #89	R4-1815934

# Spokane, USA, 12-16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-3: MPR for intraband EN-DC in B41/n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sprint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | REl-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | MPR definition for inter band EN-DC with DC_(n)41AA and DC_41A_n41A |  |  |  |  |  |  |  |  |
| Summary of change: |  | MPR definition for inter band EN-DC with DC_(n)41AA and DC_41A_n41A added to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR for inter band EN-DC with DC_(n)41AA and DC_41A_n41A missing from the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.1.1 (new), 6.2B.2.1.2 (new), 6.2B.2.1.2.1 (new),6.2B.2.2.1(new), 6.2B.2.2.2 (new), 6.2B.2.2.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

### 6.2B.2	UE maximum output power reduction and EN-DC

#### 6.2B.2.1	Intra-band contiguous EN-DC

##### 6.2B.2.1.1	General

For EN-DC band combinations, the MPR allowed is specified in Table 6.2B.2.1-1.

Table 6.2B.2.1-1: Allowed power reduction for EN-DC

| DC configuration | MPR (subclause) |
| --- | --- |
| DC_(n)41AA1 | 6.2B.2.1.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. |  |

##### 6.2B.2.1.2	MPR for DC_(n)41AA

When the UE is configured for B41/n41 intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this subclause.

MPRIM3 is defined in Subclause 6.2B.2.1.2.1.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

MPRE-UTRA = MAX( MPRsingle,E-UTRA, MPRIM3 )

MPRNR = MAX( MPRsingle,NR, MPRIM3 )

For UEs supporting dynamic power sharing, with backoff applied equally to E-UTRA and NR

MPRE-UTRA = MPRsingle,E-UTRA

ΔA-MPRE-UTRA = (PPowrClass – PE-UTRA_act) - A-MPRIM3

If ΔA-MPRE-UTRA > 0

A-MPRIM3_NR = A-MPRIM3 - ΔA-MPRE-UTRA/2

Else

A-MPRIM3_NR = A-MPRIM3 – 2*ΔA-MPRE-UTRA

MPR = MAX(MPRsingle,E-UTRA, MPRsingle,NR, MPRIM3_NR )

where

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in [2]

- 	PE-UTRA_req is the required E-UTRA power as calculated by the UE

-	ΔA-MPRE-UTRA is the difference between the IM3 equal A-MPR, and the actual E-UTRA A-MPR

6.2B.2.1.2.1	MPRIM3 for DC_(n)41AA to meet -13 dBm / 1MHz for 26 dBm UE power

MPR in this section is relative to 26 dBm. The same A-MPR is used relative to 23 dBm for power class 3. The allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ B < 0.5

9	;	 0.5 ≤ B < 1.0

7	; 	1.0 ≤ B < 2.0

5	; 	2.0 < B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

##### 6.2B.2.2.1	General

For intra-band EN-DC band combinations the MPR allowed is specified in Table 6.2B.2.2-1.

Table 6.2B.2.2-1: Allowed power reduction for EN-DC

| DC configuration | MPR (subclause) |
| --- | --- |
| DC_41A_n411 | 6.2B.2.2.2 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. |  |

##### 6.2B.2.2.2	MPR for DC_41A_n41 for 26 dBm UE power

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( MIN( SEM-13,low, FIM3,low_block,high ) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

MPRIM3 is defined in Subclause 6.2B.2.1.2.1, with same values as for contiguous DC(n)41AA.

Else

MPRIM3 defined in Subclause 6.2B.2.2.2.1.

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.1.2.2.

-	SEM-13,low = Threshold frequency where lower spectral emission mask for lower channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.1.2.2.

The UE determines the value of MPRACLRoverlap as specified in Table 6.2B.2.2-2

Table 6.2B.2.2-2: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

MPRE-UTRA = MAX( MPRsingle,E-UTRA, MPRIM3, MPRACLRoverlap )

MPRNR = MAX( MPRsingle,NR, MPRIM3, MPRACLRoverlap )

For UEs supporting dynamic power sharing , with backoff applied equally to E-UTRA and NR

MPRE-UTRA = MPRsingle,E-UTRA

ΔA-MPRE-UTRA = (PPowrClass – PE-UTRA_act) - A-MPRIM3

If ΔA-MPRE-UTRA > 0

A-MPRIM3_NR = A-MPRIM3 - ΔA-MPRE-UTRA/2

Else

A-MPRIM3_NR = A-MPRIM3 – 2*ΔA-MPRE-UTRA

MPR = MAX( MPRsingle,E-UTRA, MPRsingle,NR, MPRIM3_NR, MPRACLRoverlap )

where

-	MPRsingle,E-UTRA is the A-MPR defined for the E-UTRA transmission in [4].

-	MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

- 	PE-UTRA_req is the required E-UTRA power as calculated by the UE

-	ΔA-MPRE-UTRA is the difference between the IM3 equal A-MPR, and the actual E-UTRA A-MPR

6.2B.2.2.2.1	MPRIM3 for DC_41A_n41 to meet -30 dBm / 1MHz

MPR in this section is relative to 26 dBm. The allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

MPRIM3 = MA

Where MA is defined as follows

MA = 	17	; 	0 ≤ B < 1.0

16	; 	1.0 ≤ B < 2.0

15	; 	2.0 ≤ A < 5.0

14	; 	5.0 < A

Where:

For UEs supporting dynamic power sharing,

B = (NRB_alloc,E-UTRA * 12* SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (NRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

For NR

B = (12 * SCSE-UTRA + NRB_alloc,NR * 12 * SCSNR)/1,000.000

#### 6.2B.2.3	Inter-band EN-DC within FR1

< conducted requirements >

<Unchanged sections omitted>