3GPP TSG-RAN WG4 Meeting #89	R4-1815973

# Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-3 rel. 15 to fix MSD issues for higher order EN-DC combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel. 15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Refsens exceptions are only listed when the IMD or (sub)harmonic of the UL carriers is falling on the E-UTRA PCC or NR carrier or for combinations with a single SCC the IMD or (sub)harmonic falls onto the SCC. It needs to be clarified that higher order combinations with multiple SCCs including more than three carriers can also use the Refsens exceptions for their IMD or (sub)harmonic falling on one of the RX bands. The same applies for E-UTRA only combinations, for example when a harmonic of the E-UTRA carrier falls into the SCC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifying the sentence in 7.3B.2.3.5 to include higher order combinations, E-UTRA and NR CA combinations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs will fail Refsens test for higher order combinations if exceptions are not allowed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.3B	Reference sensitivity level for DC

### 7.3B.1	General

<Editor’s note: Table number to be updated>

For EN-DC, E-UTRA and NR single carrier REFSENS requirements defined in [2], [3] and [4] apply to all downlink bands of EN-DC configurations listed in the Tables in chapter 5.5 unless sensitivity degradation is allowed as defined in clauses 7.3.2.1 and 7.3.2.2 of this specification, chapter 7.3 in TS 38.101-1 [2] or chapter 7.3 in TS 36.101 [4]. These exceptions also apply to any higher order combination containing one of the exception combinations listed in the chapters above as subset. EN-DC REFSENS requirements shall be met for NR uplink transmissions using QPSK DFT-s-OFDM waveforms as defined in clause 7.3.2 [2].