3GPP TSG-RAN WG4 Meeting #89	R4-1816084

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6094 | rev | 1 | Current version: | 14.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of MSG3-based channel quality reporting test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Perf |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test case of the MSG3-based channel quality report is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of MSG3-based channel quality reporting test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requiremensts for MSG3-based channel quality report are not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.14 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## A.9.14	Channel quality reporting accuracy

### A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage

#### A.9.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.1.2-1 and A.9.14.1.2-2.

Table A.9.14.1.2-1: General Test Parameters for Downlink channel quality reorting accuracy test for for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter; Unit; Value. NB-IoT Operation mode; In-band. DRX; OFF. NPRACH configuration; As specified in A.3.18. NPDCCH repetition level; 1. npdcch-StartSF-USS Note 1; v8. npdcch-NumRepetitions-r13 Note 1; r1. NPUSCH repetition level; 1. Note 1: For further information see clause 6.7.3.2 in TS 36.331 [2].. |  |  |  |
| --- | --- | --- | --- |
| Parameter |  | Unit | Value |  |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| NPRACH configuration |  |  | As specified in A.3.18 |  |
| npdcch-StartSF-USS Note 1 |  |  | v8 |  |
| NPUSCH repetition level |  |  | 1 |  |

Table A.9.14.1.2-2: nCell specific Test Parameters for Downlink channel quality reorting accuracy test for for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under normal coverage

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR |  | [4] |
| NPBCH_RB | dB | 0 |  |  |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |  |
| NRS | dB | [-6] |  |
| Propagation condition |  | AWGN |  |
| Antenna Configuration |  | 2x1 |  |
| Scheduling delay in RAR (IDelay) Note3 |  | [0] |  |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |  |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS36.213. Note 4: See TS36.331. |  |  |  |  |

#### A.9.14.1.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

### A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage

#### A.9.14.2.1	Test Purpose and Environment

The purpose of this test is to verify that the downlink channel quality reporting accuracy is within the specified limits. This test will verify the requirements in Section 9.1.22.16.

#### A.9.14.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. The MSG3-based downlink channel quality reporting accuracy is tested by using the parameters in Tables A.9.14.2.2-1 and A.9.14.2.2-2.

Table A.9.14.2.2-1: General Test Parameters for Downlink channel quality reorting accuracy test for for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| NPRACH configuration |  |  | As specified in A.3.18 |  |
| npdcch-StartSF-USS Note 1 |  |  | v2 |  |
| NPUSCH repetition level |  |  | [1] |  |

Table A.9.14.2.2-2: nCell specific Test Parameters for Downlink channel quality reorting accuracy test for for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| BWchannel | kHz | 200 |
| NPDCCH parameter |  | R.30 HD-FDD |
| NPDCCH repetition level for RAR |  | [32] |
| NPBCH_RB | dB | 0 |
| NPSS_RA | dB |  |
| NSSS_RA | dB |  |
| NPDCCH_RA | dB |  |
| NPDCCH_RB | dB |  |
| NPDSCH_RA | dB |  |
| NPDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
| NRS | dB | [-12] |
| Propagation condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Scheduling delay in RAR (IDelay) Note3 |  | [0] |
| Channel quality IE Note4 |  | CQI-NPDCCH-NB |
| Note 1: OCNG shall be used such that active cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: See section 16.3.3 in TS36.213. Note 4: See TS36.331. |  |  |

#### A.9.14.2.3	Test Requirements

The downlink channel quality reporting accuracy shall fulfil the requirements in section 9.1.22.16.

------------------------------------------------------------- End of change ------------------------------------------------------------