3GPP TSG-RAN WG4 Meeting # 89	R4-1816109

# Spokane, USA, 12 November, 2018 - 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to DRX cycle for EN-DC operation in sections 9.3.x |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  |  |  | Date: |  | 2018/02/11 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Unclear whether DRX referred to in requirements for EN-DC is the MCG or the SCG |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add an additional note in requirements in section 9.2.x and 9.3.x: NOTE x: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 - Add an additional note in inter requirements in section 9.3.x NOTE y: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous which DRX cycle to consider in requirements for EN-DC cases |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Changes proposed in this CR are shown with cyan highlight |  |  |  |  |  |  |  |  |  |

## 9.3	NR inter-frequency measurements

Editor’s note: DRX and non DRX requirement might be separately defined in this section. The numerology and BW combinations might be reflected in the requirement table.

### 9.3.1	Introduction

A measurement is defined as a SSB based inter-frequency measurement provided it is not defined as in intra-frequency measurement according to section 9.2.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by PCell or the PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC and SA, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA, the switching time is 0,25ms Otherwise the switching time is 0.5ms.

### 9.3.2	Requirements applicability

The requirements in Section 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Sections 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in Sections 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in Sections 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

#### 9.3.2.1	Void

#### 9.3.2.2	Void

### 9.3.3	Number of cells and number of SSB

#### 9.3.3.1	Requirements for FR1

For each inter-frequency layer, the UE shall be capable of monitoring at least 4 cells.

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the inter-frequency layer.

#### 9.3.3.2	Requirements for FR2

For each inter-frequency layer, the UE shall be capable of monitoring at least 4 cells.

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the inter-frequency layer. The UE shall be capable of monitoring at least one SSB per cell.

### 9.3.4	Inter frequency cell identification

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within TSSB_time_index_inter.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1 and table 9.3.4-2.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3 and table 9.3.4-4.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1 and table 9.3.5-2.

Mpss/sss_sync_inter : For a UE supporting power class 1, Mpss/sss_sync_inter=[64] samples. For a UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 3 (handheld), Mpss/sss_sync_inter=[40] samples. For a UE supporting power class 4, Mpss/sss_sync=[TBD] samples.

MSSB_index_inter: For a UE supporting power class 1, MSSB_index_inter =[TBD] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[TBD] samples.For a UE supporting power class 3 (handheld), MSSB_index_inter =[TBD] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

Mmeas_period_inter: For a UE supporting power class 1, Mmeas_period_inter =[64] samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_inter=[40] samples.For a UE supporting power class 3 (handheld), Mmeas_period_inter =[40] samples. For a UE supporting power class 4, Mmeas_period_inter =[TBD] samples.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_i in section 9.1.5.2 for measurement conducted within measurement gaps.

# Table 9.3.4-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition NOTE2,3 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, [8] x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ [320]ms | max[ 600ms, ceil(8x1.5) x max(MGRP, SMTC period, DRX cycle)x CSSFinter] Note 1, |
| DRX cycle > [320]ms | [8] x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition NOTE 2,3 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max[ 600ms, Mpss/sss_sync_inter x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ [320]ms | max[ 600ms, Mpss/sss_sync_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > [320]ms | Mpss/sss_sync_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE 2,3 | TSSB_time_index_inter |
| --- | --- |
| No DRX | max[ 120ms, [3] x max(MGRP, SMTC period)x CSSFinter ] Note 1, |
| DRX cycle ≤ [320]ms | max[ 120ms, ceil(3 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > [320]ms | [3] x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.4-4: Time period for time index detection (Frequency range FR2)

| Condition | TSSB_time_index_inter |
| --- | --- |
| No DRX NOTE 2 | max[ 200ms, MSSB_index_inter x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ [320]ms | max[ 200ms, MSSB_index_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > [320]ms | MSSB_index_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being identified NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell. |  |

#### 9.3.4.1	Void

#### 9.3.4.2	Void

### 9.3.5	Inter frequency measurements

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in sub-clauses [TBD], [TBD], and [TBD], respectively,  as shown in table 9.3.5-1 and 9.3.5-2:

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE 2,3 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 200ms, [8] x max(MGRP, SMTC period)x CSSFinter] Note 1, |
| DRX cycle ≤ [320]ms | max[ 200ms, ceil(8 x 1.5) x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > [320]ms | [8] x DRX cycle x CSSFinter |
| … | … |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being measured NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (Frequency FR2)

| Condition NOTE2,3 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max[ 400ms, Mmeas_period_inter x max(MGRP, SMTC period) x CSSFinter] Note 1, |
| DRX cycle ≤ [320]ms | max[ 400ms, Mmeas_period_inter x max(MGRP, SMTC period, DRX cycle) x CSSFinter] Note 1, |
| DRX cycle > [320]ms | Mmeas_period_inter x DRX cycle x CSSFinter |
| … | … |
| NOTE 1: If different SMTC periodicities are configured for different inter-frequency carriers, the SMTC period in the requirement is the SMTC period of the inter-frequency carrier being measured NOTE 2: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 NOTE 3: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

#### 9.3.5.1	Void

#### 9.3.5.2	Void

#### 9.3.5.3	Void