3GPP TSG-RAN WG4 Meeting #89 	R4-1816152

Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5306 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for demodulation performance requirements for network-based CRS interference mitigation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NW_CRS_IM-Perf |  |  |  |  |  | Date: |  |  | 2018-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDSCH demodulation performance requirements with network CRS-IM are agreed to be introduced in Rel-15 for LTE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TM3 test is added for PDSCH UE demod tests for both FDD and TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements with network CRS-IM can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.3.6, 8.2.2.3.6, Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

##### 8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.1.3.6-2, with the addition of parameters in Table 8.2.1.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.1.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.1.3.6-1 Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  |  | Unit | Cell 1 | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz |  | 10 MHz |  |  |
| Downlink power allocation |  |  | dB |  |  | -3 |  |  | 0 |  |  |
|  |  |  |  |  |  | -3 (NOTE 1) |  |  | 0 |  |  |
|  |  |  |  |  |  | 0 |  |  | 0 |  |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  |  | Antenna port 0,1 |  |
| Cyclic Prefix |  |  |  |  |  | Normal |  |  | Normal |  |
| Cell ID |  |  |  |  |  | 0 |  |  | 1 |  |
| Transmission mode |  |  |  |  |  | 3 |  |  | NA (Note 4) |  |
| at antenna port |  |  | dBm/15kHz |  |  | -98 |  |  | -98 |  |
| (NOTE 2) |  |  | dB |  |  | Reference Value in Table 8.2.1.3.6-2 |  |  | 10 |  |
| Correlation and antenna configuration |  |  |  |  |  | Low (2x2) |  |  | Low (2x2) |  |
| Max number of HARQ transmissions |  |  |  |  |  | 4 |  |  | N/A |  |
| Redundancy version coding sequence |  |  |  |  |  | {0,1,2,3} |  |  | N/A |  |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  |  |  |  | Disabled |  |  | Enabled |  |  |
| CRS transmission |  |  |  |  |  | On |  |  | CRS is configured with CRS muting pattern as [00000011111111111000] where every 20 subframes consist of [11] subframes with full system BW CRS and [9] subframes with CRS only on the center 6 PRBs (Note 5) |  |  |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: In the muting pattern [00000011111111111000] 1 indicates DL subframes with full system BW CRS configured and 0 indicates DL subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |  |  |  |  |  |

Table 8.2.1.3.6-2 Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (NOTE 1) |  | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (NOTE 2) |  |
| 1 | R.11-x FDD | OP.1 FDD | NA | EVA5 | EVA5 | 70 | [14.9] | ≥2 |
| NOTE 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. NOTE 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |

<next change>

##### 8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)

The requirements are specified in Table 8.2.2.3.6-2, with the addition of parameters in Table 8.2.2.3.6-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of open-loop spatial multiplexing performence with 2 transmit antennas when the PDSCH transmission in the serving cell is interfered by CRS of one dominant interfering cell with network-based CRS interference mitigation. In Table 8.2.2.3.6-1, Cell 1 is the serving cell, and Cell 2 is interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2 respectively.

Table 8.2.2.3.6-1 Test parameters for Larger Delay CDD (FRC) with network-based CRS interference mitigation

| Parameter |  | Unit |  |  |  |  | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz |  |  | 10 MHz |  |  |  |  |
| Uplink downlink configuration |  |  |  | 4 |  |
| Special subframe configuration |  |  |  | 4 |  |
| Downlink power allocation |  |  | dB |  |  | -3 |  |  | 0 |  |  |  |
|  |  |  |  |  |  | -3 (Note 1) |  |  | 0 |  |  |  |
|  |  |  |  |  |  | 0 |  |  | 0 |  |  |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  |  | Antenna ports 0,1 |  |  |  |
| Cyclic Prefix |  |  |  |  |  | Normal |  |  | Normal |  |  |  |
| Cell ID |  |  |  |  |  | 0 |  |  | 1 |  |  |  |
| Transmission mode |  |  |  |  |  | 3 |  |  | NA (Note 4) |  |  |  |
| at antenna port |  |  | dBm/15kHz |  |  | -98 |  |  | N/A |  |  |  |
| (Note 3) |  |  | dB |  |  | Reference Value in Table 8.2.2.3.6-2 |  |  | 10 |  |  |  |
| Correlation and antenna configuration |  |  |  |  |  | Low (2x2) |  |  | Low (2x2) |  |  |  |
| Max number of HARQ transmissions |  |  |  |  |  | 4 |  |  | N/A |  |  |  |
| Redundancy version coding sequence |  |  |  |  |  | {0,1,2,3} |  |  | N/A |  |  |  |
| nw-BasedCRS-InterferenceMitigation-r15 |  |  |  | Disabled | Enabled |
| CRS transmission |  |  |  | On | CRS is configured with CRS muting pattern as [10UU11111110UU111000] where every 20 subframes consist of [11] subframes with full system BW CRS and [9] subframes with CRS only on the center 6 PRBs (Note 5) |
| Note 1: Note 2: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 3: All cells are time-synchronous. Note 4: Cell 2 has no PDSCH/PDCCH configured Note 5: The muting pattern [10UU11111110UU111000] follows the Uplink downlink configuration 4 where U indicates uplink subframes, 1 indicates DL subframes with full system BW CRS configured and 0 indicateds DL subframes including special subframes with CRS only on the center 6 PRBs configured. |  |  |  |  |  |  |  |  |  |  |

Table 8.2.2.3.1C-2 Minimum performance Large Delay CDD (FRC) with network-based CRS interference mitigation

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions (Note 1) |  | Reference Value |  | UE Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Fraction of Maximum Throughput (%) | SNR (dB) (Note 2) |  |  |
| 1 |  | R.11-x TDD | OP.1 TDD | NA | EVA5 | EVA5 | 70 | TBD | ≥2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

<next change>

Table A.3.3.2.1-9: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.11-x FDD |  |
| Channel bandwidth | MHz | 10 |  |
| Allocated resource blocks (Note 4) |  | 50 |  |
| Allocated subframes per Radio Frame |  | 9 |  |
| Modulation |  | 16QAM |  |
| Target Coding Rate |  | 1/2 |  |
| Information Bit Payload (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 10680 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 10680 |  |
| Number of Code Blocks (Notes 3 and 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 2 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 2 |  |
| Binary Channel Bits (Note 4) |  |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | 21648 |  |
| For Sub-Frame 5 | Bits | N/A |  |
| For Sub-Frame 0 | Bits | 17424 |  |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 19.224 |  |
| UE Category |  | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: Given per component carrier per codeword. Note 5: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1,2,3,4,6,7,8,9. |  |  |  |  |

<next change>

Table A.3.4.2.1-11: Fixed Reference Channel two antenna ports

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.11-x TDD |  |  |
| Channel bandwidth | MHz | 10 |  |
| Allocated resource blocks (Note 5) |  | 50 |  |
| Uplink-Downlink Configuration (Note 3) |  | 4 |  |
| Special subframe configuration |  | 4 |  |  |
| Allocated subframes per Radio Frame (D+S) |  | 7+1 |  |
| Modulation |  | 16QAM |  |
| Target Coding Rate |  | 1/2 |  |
| Information Bit Payload (Note 5) |  |  |  |
| For Sub-Frames 0,4,6,7,8,9 | Bits | 10680 |  |
| For Sub-Frames 1 |  | 7736 |  |
| For Sub-Frames 5 |  | NA |  |
| Number of Code Blocks (Notes 4 and 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 |  | 2 |  |
| For Sub-Frames 1 |  | 2 |  |
| For Sub-Frames 5 |  | NA |  |
| Binary Channel Bits (Note 5) |  |  |  |
| For Sub-Frames 0,4,5,6,7,8,9 | Bits | 21648 |  |
| For Sub-Frames 1 |  | 17424 |  |
| For Sub-Frames 5 |  | NA |  |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 7.1816 |  |
| UE Category |  | ≥ 2 |  |
| Note 1: 2 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: As per Table 4.2-2 in TS 36.211 [4]. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Given per component carrier per codeword. Note 6: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0,1,4,5,6,7,8,9. |  |  |  |  |

<end of the change>