3GPP TSG-RAN WG4 Meeting #89	R4-1816202

Spokane, USA, 12th – 16th Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to interband EN-DC OOBE emission requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - 6.5B.2.3 Inter-band EN-DC within FR1 section has references to LTE SEM and additional SEM but reference should be to whole OOBE clause 6.6.2 which includes also ACLR. Similar case for NR1 part. - 6.5B.2.4 Inter-band EN-DC including FR2 is empty - 6.5B.2.5 Inter-band EN-DC including both FR1 and FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to 6.5B.2.3 references and addition of requirements into 6.5B.2.4 and 6.5B.2.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | errors remain and requirements are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.2.3, 6.5B.2.4, 6.5B.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************** Start of changes *********************************************

## 6.5B	Output RF spectrum emissions for DC

### 6.5B.1	Occupied bandwidth for EN-DC

For intra-band contiguous EN-DC the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum.  The OBW shall be less than the aggregated channel bandwidth for EN-DC, denoted as ENBW in sub-clause 5.3B.

### 6.5B.2	Out-of-band emissions for EN-DC

#### 6.5B.2.1	Intra-band contiguous EN-DC

Unless otherwise stated, the OOBE limits specified for the DC combination in this sub-clause supercede any OOBE requirements specified for each sub-block in the respective TS [4] and [2].

The requirements apply to the sum of transmissions across all antenna connectors.

##### 6.5B.2.1.1	Spectrum emissions mask

The general spectrum emission for intra-band contiguous EN-DC is specified in Table 6.5B.2.1.1-1.

Table 6.5B.2.1.1-1.  General spectrum emission mask for intra-band contiguous EN-DC

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0 - 1 | Max(Round(10*log(0.15/ENBW)),-24) | 30 kHz |
| ± 1 - 5 | -10 | 1 MHz |
| ± 5 - ENBW | -13 | 1 MHz |
| ± ENBW – (ENBW+5) | -25 | 1 MHz |
| NOTE: ENBW refers to the aggregated channel bandwidth in MHz as defined in sub-clause 5.3B. |  |  |

##### 6.5B.2.1.2	Additional spectrum emissions mask

###### 6.5B.2.1.2.1	Requirements for network signalled value "NS_35"

When NS_35 is indicated in the MCG and NS_35 is indicated in the SCG, the requirements in Table 6.5B.2.1.2.1-1 apply in the frequency ranges immediately adjacent and outside the aggregated sub-blocks of the EN-DC configuration for DC_(n)71AA.

Table 6.5B.2.1.2.1-1: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

###### 6.5B.2.1.2.2	Requirements for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The Band 41/n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  Since the 26 dB emission bandwidth is implementation dependent, the transmission bandwidths occupied by RBs is used for the SEM. The emission bandwidth for LTE carriers is document in 36.101 [4], and the emission bandwidth for NR carriers is documented in 38.101-1 [2]. The total emission bandwidth for contiguous intra-band EN-DC is the sum of the emission bandwidth for each CC plus the guard band between contiguous CCs.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.2.2-1.

Table 6.5B.2.1.2.2-1: n41 SEM with NS_04

|  | Spectrum emission limit (dBm)/ measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 | -18 | -20 | -21 | -24 | -25 |  | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |
| NOTE: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |  |

##### 6.5B.2.1.3	Adjacent channel leakage ratio

For EN-DC operation with an E-UTRA sub-block immediately adjacent to an NR sub-block, the ACLR is defined as the ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW at nominal channel spacing. The UE shall meet the ACLR minimum requirement EN-DCACLR specified in Table 6.5B.2.1.3-1 with ENBW the sum of the sub-block bandwidths.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in 6.5B.2.1.3-1.

Table 6.5B.2.1.3-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth of EN-DC channel |  | 1.00*ENBW |
| Measurement bandwidth of adjacent channel |  | 0.95*ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz as defined in sub-clause 5.3B. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

#### 6.5B.2.2	Intra-band non-contiguous EN-DC

##### 6.5B.2.2.1	Spectrum emissions mask

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used.

##### 6.5B.2.2.2	Additional spectrum emissions mask

When additional spectrum emission mask or masks apply, the additional SEM(s) shall be used to calculate the composite SEM described in 6.5B.2.2.1.

##### 6.5B.2.2.3	Adjacent channel leakage ratio

For intra-band non-contiguous EN-DC when all UL sub-blocks consist of one component carrier the EN-DC Adjacent Channel Leakage power Ratio (EN-DCACLR) is the ratio of the sum of the filtered mean powers centred on the assigned sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than of the sub-block bandwidth than for that sub-block no EN-DCACLR requirement is set for the gap. In case the sub-block gap bandwidth Wgap is smaller than either of the sub-block bandwidths then no EN-DCACLR requirement is set for the gap. The assigned EN-DC sub-block power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in in [4] for the E-UTRA sub-block, and [2],[3] for the NR sub-block. If the measured adjacent channel power is greater than –50dBm then the EN-DCACLR shall be higher than the value specified in for E-UTRAACLR and NRACLR .

#### 6.5B.2.3	Inter-band EN-DC within FR1

Unless otherewise stated, the OOBE requirements specified in sub-clause 6.6.2 of [4] and sub-clause 6.5.2 of [2] apply for each component carrier.

The requirements apply to each antenna connector.

#### 6.5B.2.4	Inter-band EN-DC including FR2

Unless otherewise stated, the OOBE requirements specified in sub-clause 6.6.2 of [4] and sub-clause 6.5.2 of [3] apply for each component carrier.

#### 6.5B.2.5	Inter-band EN-DC including both FR1 and FR2

Unless otherewise stated, the OOBE requirements specified in sub-clause 6.6.2 of [4], sub-clause 6.5.2 of [2] and 6.5.2 of [3] apply for each component carrier.

****************************** End of changes *********************************************