3GPP TSG-RAN WG4 Meeting #89	R4-1816352

Spokane, US, 12 - 16 November 2018

Source:	China Telecom

Title:	TP to TS 38.141-1: Update of AWGN power level and FRC index for DFT-s-OFDM based PUSCH demodulation requirements

Agenda Item:	7.13.2.2.2

Document for:	Approval

- Introduction

In RAN4 #88bis meeting, the TP on conducted performance requirements for DFT-s-OFDM based PUSCH was approved in R4-1813748. This TP is to:

- Remove the unnecessary channel bandwidths in Table 8.2.2.4.2-1 for AWGN power level at the BS input.

- Update the FRC indices for the conducted DFT-s-OFDM based PUSCH test.

- Update the reference section number for the measurement system set-up according to the latest version of TS 38.141-1.

- Text Proposal

8.2.2	Performance requirements for PUSCH with transmission precoding enabled

8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

A test for a specific channel bandwidth is only applicable if the BS supports it.

The applicability of tests in 38.104 with different SCS and BW combinations is according to the following principle:

Editor’s note: Applicability rule is FFS.

8.2.2.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] subclause 8.2.2.

8.2.2.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.4	Method of test

8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M; see subclause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex D.5 and D.6 for BS type 1-C and type 1-H respectively .

8.2.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the SCS and channel bandwidth, defined in Table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DMRS | DMRS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |
|  | Number of additional DMRS symbols | 0, 1 |
|  | Number of DMRS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |
|  | DMRS port | 0 |
|  | DMRS sequence generation | NID=0, group hopping and sequence hopping are disabled |
| Time domain resource | PUSCH mapping type | A |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in [TBD].

4)	Adjust the equipment so that required SNR specified in Table 8.2.2.5-1 to 8.2.2.5-2 is achieved at the BS input.

5)	For each of the reference channels in Table 8.2.2.5-1 to 8.2.2.5-2 applicable for the base station, measure the throughput, according to [TBD].

8.2.2.5	Test Requirement

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.2.5-1 to 8.2.2.5-2.

Table 8.2.2.5-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix |  | Propagation conditions and correlation matrix (Annex TBD) |  | Fraction of maximum throughput |  | FRC (Annex A) | DMRS configuration |  | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 | 70 % | G-FR1-A3-29 | 1+0 | [2.8] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 | 70 % | G-FR1-A3-29 | 1+0 | [-1.8] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 | 70 % | G-FR1-A3-29 | 1+0 | [-5.2] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |

Table 8.2.2.5-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex TBD) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 | 70 % | G-FR1-A3-30 | 1+0 | [-1.1] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-2.0] |
|  | 4 | Normal | TDLB100-400 | 70 % | G-FR1-A3-30 | 1+0 | [-4.3] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-5.1] |
|  | 8 | Normal | TDLB100-400 | 70 % | G-FR1-A3-30 | 1+0 | [-7.6] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-8.0] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex C.