3GPP TSG-RAN WG4 Meeting #89		         R4-1816386

Spokane, US, 12th – 16th Nov, 2018

Agenda item:      7.13.1.3.1

Source:                Intel Corporation

Title:                     NR PDCCH simulation results

Document for:     Discussion

# Introduction

In RAN4#88bis NR PDCCH demodulation requirements were discussed and way forward [1] was agreed.

Power allocation - Assume EPRE ratio of PDCCH_DMRS to SSS is 0dB, and EPRE ratio of the PDCCH to PDCCH_DMRS is 0dB. 1 Symbol PDCCH Test - Test case 12: AL=4 for 2Rx, 4Rx - Test case 13: AL=8 for 2Rx; AL=4 for 4Rx Test case with AL=16 - Introduce test case with 2Rx - FFS for test case with 4Rx Test cases with 4Rx and interleaved CCE-to-REG Mapping - Test case 8, Bundle size =6 with interleave size as 3 - Option 1: Test case 6, 7 with REG bundle size =2, CORESET BW = 96 for both 2Rx and 4Rx - Option 2: For 4Rx, test cases with interleaved (cases 6,7), using REG bundle size =6 - Further evaluted above two options for case 6,7 and decided in RAN4#89 meeting.

In this contribution we present simulation results for PDCCH test cases to define requirements.

# Simulation Results

## Alignment results with 2Rx

For the test cases agreed in [1], we provide the results for alignment for SNR at 1% Pm-dsg for 2Rx. The results are with simplified TDL channel models agreed in [2].

Table 1: PDCCH Alignment Results with 2Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | 6.01 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x2 Low | -0.87 |
| 3 | 15 | 1_1 | 24 | 51 | 2 | 2 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 1x2 Low | 5.80 |
| 4 | 15 | 1_1 | 48 | 51 | 2 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | 3.58 |
| 5 | 15 | 1_1 | 48 | 51 | 2 | 8 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x2 Low | -4.02 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDL-A, 30ns, 10Hz | 1x2 Low | 5.33 |
| 6a | 30 | 1_0 | 90 | 41 | 1 | 2 | Interleaved | 6 | 3 | TDL-A, 30ns, 10Hz | 1x2 Low | 4.45 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDL-C, 300ns, 100Hz | 1x2 Low | 1.64 |
| 7a | 30 | 1_1 | 90 | 53 | 1 | 4 | Interleaved | 6 | 3 | TDL-C, 300ns, 100Hz | 1x4 Low | -0.16 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDL-C, 300ns, 100Hz | 2x2 Low | -3.67 |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 1x2 Low | 4.27 |
| 10 | 120 | 1_1 | 60 | 52 | 1 | 4 | Interleaved | 6 | 2 | TDL-A, 30ns, 300Hz | 1x2 Low | -0.13 |
| 11 | 120 | 1_1 | 60 | 52 | 1 | 8 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -2.18 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | 2.38 |
| 13 | 15 | 1_1 | 48 | 51 | 1 | 8 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 2x2 Low | -2.48 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x2 Low | -3.95 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 1x2 Low | -6.63 |
| 16 | 120 | 1_0 | 60 | 40 | 2 | 16 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -5.05 |

## Alignment results with 4Rx

For the test cases agreed in [1], we provide the results for alignment for SNR at 1% Pm-dsg with 4Rx. The results are with simplified TDL channel models agreed in [2].

Table 2: PDCCH Alignment Results with 4Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x4 Low | 0.47 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x4 Low | -4.61 |
| 3 | 15 | 1_1 | 24 | 51 | 2 | 2 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 1x4 Low | 0.59 |
| 4 | 15 | 1_1 | 48 | 51 | 2 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x4 Low | -1.61 |
| 5 | 15 | 1_1 | 48 | 51 | 2 | 8 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 2x4 Low | -7.04 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDL-A, 30ns, 10Hz | 1x4 Low | 4.09 |
| 6a | 30 | 1_0 | 90 | 41 | 1 | 2 | Interleaved | 6 | 3 | TDL-A, 30ns, 10Hz | 1x4 Low | -0.66 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDL-C, 300ns, 100Hz | 1x4 Low | 2.02 |
| 7a | 30 | 1_1 | 90 | 53 | 1 | 4 | Interleaved | 6 | 3 | TDL-C, 300ns, 100Hz | 1x4 Low | -3.82 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDL-C, 300ns, 100Hz | 2x4 Low | -6.79 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x4 Low | -2.49 |
| 13 | 15 | 1_1 | 48 | 51 | 1 | 4 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 2x4 Low | -3.29 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x4 Low | -7.76 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDL-C, 300ns, 100Hz | 1x4 Low | -9.39 |
| 15a | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDL-A, 30ns, 10Hz | 1x4 Med-A | -5.30 |

## Impairment Results

For the agreed PDCCH test cases we provide the results with impairment margin added in the table below.

Table 3: Impairment results for PDCCH test cases

| Test No. | Ant config | SNR @ 1% Pm-dsg | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- |
| 1 | 1x2 Low | 7.51 | 1x4 Low | 1.97 |
| 2 | 2x2 Low | 0.63 | 2x4 Low | -3.11 |
| 3 | 1x2 Low | 7.30 | 1x4 Low | 2.09 |
| 4 | 1x2 Low | 5.08 | 1x4 Low | -0.11 |
| 5 | 2x2 Low | -2.52 | 2x4 Low | -5.54 |
| 6 | 1x2 Low | 6.83 | 1x4 Low | 5.59 |
| 6a | 1x2 Low | 5.95 | 1x4 Low | 0.84 |
| 7 | 1x2 Low | 3.14 | 1x4 Low | 3.52 |
| 7a | 1x2 Low | 1.34 | 1x4 Low | -2.32 |
| 8 | 2x2 Low | -2.17 | 2x4 Low | -5.29 |
| 9 | 1x2 Low | 5.77 | - | - |
| 10 | 1x2 Low | 1.37 | - | - |
| 11 | 2x2 Low | -0.68 | - | - |
| 12 | 1x2 Low | 3.88 | 1x4 Low | -0.99 |
| 13 | 2x2 Low | -0.98 | 2x4 Low | -1.79 |
| 14 | 1x2 Low | -2.45 | 1x4 Low | -6.26 |
| 15 | 1x2 Low | -5.13 | 1x4 Low | -7.89 |
| 15a | - | - | 1x4 Med-A | -3.80 |
| 16 | 2x2 Low | -3.55 | - | - |

# Conclusion

In this paper we present alignment and impairment results for PDCCH to define NR PDCCH demodulation requirements for Rel-15.

# Reference

- R4-1814213, “Way forward on NR PDCCH demodulation requirements”, Ericsson

- R1-1813735, “Way Forward on simplified TDL channel model”, Huawei, HiSilicon, Spirent, Rohde & Schwarz, Keysight, Ericsson, Intel