3GPP TSG-RAN WG4 Meeting #89                 	R4-1816508

Spokane, USA, Nov 12th – 16th 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on NR PSCell addition and release delay for FR1 (Section A.4.8) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Adding test for NRPScell addition and relase delay for FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding tests for NRPScell addition and release delay for FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 NR PScell addition and release delay will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR…CR... TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### A.4.8 PSCell addition and release delay

#### A.4.8.1 Addition and Release Delay of known NR PSCell

##### A.4.8.1.1 Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delays under EN-DC are within the requirements stated in section 7.31.2 [15] for the case when the PSCell is known by the UE at the time of addition.

Supported test configurations are shown in A.4.8.1.1-1. The test parameters for the E-UTRA cell are given in Table A.3.7.2.1-1. The E-UTRA cell once set up is not changed across time.

The test parameters for NR cell are given in Tables A.4.8.1.1-2 and cell-specific parameters in A.4.8.1.1-3 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore, during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of period T5.

Table A.4.8.1.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 2 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 3 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| 4 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, FDD |
| 5 | LTE FDD, NR SCS 15 kHz, BW 10 MHz, TDD |
| 6 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.4.8.1.1-2: General Test Parameters for PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for E-UTRA cell and second for NR Cell |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour Cell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time to Trigger | S | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration on cell2 |  |  | FR1 PRACH configuration 2 | Captured in [A.3.8.2.1] |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | TBD | CQI reporting for PSCell every uplink subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | S | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | S | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | S | 1 | During this time the UE adds the PSCell. |
| T4 |  | S | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | S | 1 | During this time the UE releases the PSCell. |

Table A.4.8.1.1-3: Cell Specific Parameters for PSCell Addition and Release

| Parameter |  | Unit | Config | Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1,2,3,4,5,6 | 1 |  |  |  |  |
| NR RF Channel Number |  |  | 1,2,3,4,5,6 | 2 |  |  |  |  |
| TDD configuration |  |  | 1,4 | Not Applicable |  |  |  |  |
|  |  |  | 2,5 | TDDConf.1.1 |  |  |  |  |
|  |  |  | 3,6 | TDDConf.1.2 |  |  |  |  |
| BWchannel |  | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |
| BWP | DL BWP |  | 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  |  |
|  | UL BWP |  | 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | 1,4 | SR.1.1 FDD |  |  |  |  |
|  |  |  | 2,5 | SR.1.1 TDD |  |  |  |  |
|  |  |  | 3,6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | 1,4 | CR.1.1 FDD |  |  |  |  |
|  |  |  | 2,5 | CR.1.1 TDD |  |  |  |  |
|  |  |  | 3,6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | 1,4 | CCR.1.1 FDD |  |  |  |  |
|  |  |  | 2,5 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |
| SSB configuration |  |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  |
|  |  |  | 3,6 | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  |  | 1,2,4,5 | SMTC.1 |  |  |  |  |
|  |  |  | 3,6 | SMTC.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1,2,3,4,5,6 | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | 1,2,3,4,5,6 | N/A | -85 |  |  |  |
| Note2 |  | dBm/SCS | 1,2,4,5 | N/A | -85 |  |  |  |
|  |  |  | 3,6 | N/A | -82 |  |  |  |
|  |  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
|  |  |  | 1,2,3,4,5,6 | -infinity | 0 |  |  |  |
| SS-RSRPNote3 |  | dBm/SCS | 1,2,4,5 | -infinity | -85 |  |  |  |
|  |  |  | 3,6 | -infinity | -82 |  |  |  |
| IoNote3 |  | dBm/9.36MHz | 1,2,4,5 | N/A | -57 |  |  |  |
|  |  | dBm/38.1MHz | 3,6 | N/A | -51 |  |  |  |
| Propagation condition |  |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |

##### A.4.8.1.2 Test Requirements

The UE shall transmit the PRACH to PSCell at latest 232 msNote1 into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4

The UE shall stop sending CSI reports for PSCell in at latest [20]ms into T5.

All the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

Note1:	The PSCell addition delay can be expressed as follows as specified in Clause 7.31.2 [15]:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2ms

Where:

TRRC_delay = 20ms

Tprocessing = 20ms

Tsearch 	= 0

T∆ = 20ms

TPSCell_ DU = 16*10+10 = 170ms