3GPP TSG-RAN WG4 Meeting #89	R4-1816627

# Spokane, WA, USA, 12 – 16 November, 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | <CR#> | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Verification of beam correspondence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UEs that support beam correspondence have the ability to select a corresponding beam for UL transmission based on DL measurements without relying on network-assisted UL beam management. Beam correspondence should be verified by a minimum requirement (may be conditioned on capability indication). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements on beam correspondence is verified for power class 3 in terms of the minimum peak EIRP accordingand spherical coverage requirements. The requirement only applies in connected mode after beam refinement. Details on the RS configuration need not be cited in this specification (but are subject to provisions in TS 38.214). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements on beam correspondence are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.5 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2.4	Configured transmitted power

The UE can configure its maximum output power. The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement in each receiver branch as specified in 38.215.

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds

PPowerclass – MAX(MPRf,c, P-MPRf,c) – MAX{T(MPRf,c), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE power class as specified in sub-clause 6.2.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c as specified in sub-clause 6.2.2, P-MPRf,c the power management term for the UE and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. The tolerance T(∆P) for applicable values of ∆P (values in dB) is specified in Table 6.2.4-1.

Table 6.2.4-1: PUMAX,f,c tolerance

| Operating Band | ∆P (dB) | Tolerance T(∆P) (dB) |
| --- | --- | --- |
| n257, n258, n260, n261 | P = 0 | 0 |
|  | 0 < P ≤ 2 | [1.5] |
|  | 2 < P ≤ 3 | [2] |
|  | 3 < P ≤ 4 | [3] |
|  | 4 < P ≤ 5 | [4] |
|  | 5 < P ≤ 10 | [5] |
|  | 10 < P ≤ 15 | [7] |
|  | 15 < P ≤ X | [8] |
| NOTE: X is the value such that Pumax,f,c lower bound, PPowerclass - P – T(P) = minimum output power specified in subclause 6.3.1 |  |  |

### 6.2.5	Beam correspondence

## UEs which support beam correspondence shall have the ability to select a corresponding beam for UL transmission based on DL measurements without relying on network-assisted UL beam management.

## For power class 3 UEs which support beam correspondence, the requirement is fulfilled if the UE’s corresponding UL beams satisfy the minimum peak EIRP according to Table 6.2.1.3-1 and spherical coverage requirements according to Table 6.2.1.3-3 in CONNECTED MODE such that:

<Editors note: only one of the following sentences will remain after clarity for CSI-RS mandatory / other status >

-	DL RS are configured for either SSB only or CSI-RS only

## -	DL RS are configured for either SSB only or CSI-RS only, if UE supports

## 6.2A	Transmitter power for CA

< end of changes >