3GPP TSG-RAN WG4 Meeting RAN4#89		R4-1816785

Spokane, USA, 12-16 November, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0024 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-02 - polarization wording improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-11-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Wording improvements around polarization for the OTA reference sensitivity requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | R4-1815616. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is not clear with respect to polarization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |

----------- START OF CHANGE ----------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

active antenna system base station: BS system which combines an antenna array with an transceiver unit array. An AAS BS may include a radio distribution network

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

antenna connector: connector at the conducted interface of the BS type 1-C

antenna array: group of radiating elements characterized by the geometry and the properties of the array elements

antenna gain: ratio of the radiation intensity, in a given direction, to the radiation intensity that would be obtained if the power accepted by the antenna were radiated isotropically

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

array factor: radiation pattern of an array antenna when each array element is considered to radiate isotropically

NOTE:	When the radiation pattern of individual array elements are identical, and the array elements are congruent under translation, then the product of the array factor and the array element radiation pattern gives the radiation pattern of the entire array.

angle of arrival: is the direction of propagation of electromagnetic wave incident on an AAS BS antenna array

basic limit: emissions limit from a single transceiver unit of the non-AAS BS type, used for the formulation of the unwanted emissions and output power requirements for 1-C type NR BS, as well as for the formulation of the unwanted emissions minimum requirements for 1-H and 1-O types NR BS by using emissions scaling

beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power contour of the beam

beam direction pair: data set consisting of the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is found

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink.

NOTE:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE:	It is possible for the BS to transmit to and/or receive from one or more UE Bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

cell specific beam: Cell specific beam is a beam which is intended to facilitate communication for multiple UEs within a cell

cell splitting: division of the cell's coverage in a sector into multiple subsectors

NOTE:	The subsectors may be divided into the vertical and/or horizontal plane.

demodulation branch: single input to the demodulation algorithms

NOTE:	For UTRA a demodulation branch is referred to as a receive diversity branch or a UL MIMO branch. For E-UTRA a demodulation branch is referred to as an RX antenna in the performance requirement tables.

NOTE:	The term "RX antenna" in chapter 8 of the E-UTRA specification 3GPP TS 36.104 [8] does not refer to physical receiver antennas, but to the demodulation branches.

directions diagram: two-dimensional Cartesian diagram showing φ on the horizontal axis and minus θ on the vertical axis

directivity: ratio of the radiation intensity in a given direction from the antenna to the radiation intensity averaged over all directions

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

EMC antenna port:	RF connector referred to as antenna port in EMC requirements

equivalent isotropic radiated power: in a given direction, the relative antenna gain of a transmitting antenna with respect to the antenna gain of an isotropic radiating element multiplied by the net power accepted by the antenna from the connected transmitter

NOTE:	For an AAS BS the EIRP can be seen as the equivalent power radiated from an isotropic radiating element, producing the same field intensity as the field intensity radiated in the declared beam pointing direction of the active antenna system being considered.

equivalent isotropic sensitivity: power level relative to an isotropic antenna that is required to be incident on the AAS BS array from a specified azimuth/elevation direction in order to meet a specified receiver sensitivity requirement

NOTE:	EIS is directly related to field-strength via free-space impedance and effective aperture antenna area. EIS is expressed as the receiver power that would be collected by an isotropic antenna if it were subject to a uniform field around the whole sphere as the AAS BS array experiences in the specified azimuth/elevation direction.

front-to-back ratio: ratio of maximum directivity of an antenna to its directivity in a specified rearward direction

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

hybrid requirements set: Complete set of requirements applied to a hybrid AAS BS with both conducted and radiated requirements

minSENS: the lowest declared EIS value for the OSDD’s declared for OTA sensitivity requirement

multi-band connector: antenna connector of the BS type 1-C or TAB connector of the BS type 1-H, associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

multi-band RIB: operating band specific RIB which is paired with one or more additional operating band specific RIBs where the multiple bands are supported through common active electronic component(s)

OTA AAS BS: AAS BS which has a radiated RF interface only and conforms to the OTA requirements set

OTA coverage range: a common range of directions within which TX OTA requirements that are neither specified in the OTA peak directions sets nor as TRP are intended to be met

OTA peak directions set(s): set(s) of beam peak directions within which certain TX OTA requirements are intended to be met, all OTA peak directions set(s) are subsets of the OTA coverage range

NOTE:	The beam peak directions are related to a corresponding contiguous range or discrete list of beam centre directions by the beam direction pairs included in the set.

OTA REFSENS RoAoA: is the RoAoA determined by the contour defined by the points at which the achieved EIS is 3dB higher than the achieved EIS in the reference direction.

NOTE: This contour will be related to the average element/sub-array radiation pattern 3dB beam width

OTA requirements set:	complete set of OTA requirements applied to an OTA AAS BS.

radiating element: basic building block of an array element characterized by its radiation properties

radiation pattern: angular distribution of the radiated electromagnetic field or power level in the far field region

radio distribution network: passive network which distributes radio signals generated by the active transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the active transceiver unit array.

NOTE:	The number of transmission outputs from the RDN should be greater than or equal to the number of transmission inputs for a single frequency.

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

OTA sensitivity directions declaration: set of manufacturer declarations comprising an EIS value and the directions where it applies

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction.

receiver target: angles of arrival in which reception is performed

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD

receiver target reference direction: direction, inside the receiver target redirection range declared by the manufacturer for conformance testing

NOTE:	For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA.

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy compliance directions set

sensitivity RoAoA: RoAoA within which the declared EIS of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single band RIB: operating band specific RIB without any common active electronic component(s) shared with other operating bands

single direction requirement: AAS BS requirement which is applied in a specific direction within the OTA coverage range for the Tx and FFS for the receiver.

single-band TAB connector: TAB connector supporting either operation only in a single operating band, or operation in multiple operating bands without any common active electronic component(s)

TAB connector: transceiver array boundary connector

TAB connectors beam forming group: group of TAB connectors associated with an EIRP beam declaration, comprising of the complete set of TAB connectors from which a declared beam is transmitted

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transceiver unit: active unit consisting of transmitter and/or receiver which transmits and/or receives radio signals, and which may include passive RF filters

transceiver unit array: array of transceiver units which generate radio signals in the transmit direction and accept radio signals in the receive direction

TRP requirement: AAS BS requirements, which requires dual-polarized measurements of the figure of merit over the whole sphere around the DUT

UE channel bandwidth: The RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

## ----------- NEXT CHANGE -----------

## 10.3	OTA Reference sensitivity level

### 10.3.1	General

OTA reference sensitivity level is a directional requirement specified as an EIS level.

As OTA reference sensitivity is used as a reference for many of the interference requirements where it is important that the interferer is polarization matched to the receiver antenna it is also important that the wanted signal is polarization matched. Hence OTA reference sensitivity is specifically declared per polarization and the EIS values are under the assumption of polarization match.