3GPP TSG-RAN WG4 #89	R4-1815344

12 - 16 Nov 2018

Spokane, WA

Source: 	Qualcomm Incorporated

Title: 	NBIOT B26 guard band operation

Agenda item:	5.3.1

Document for:	Approval

# Introduction

As per FCC specification # 90.691, emission limits at lower band edge of UE operating in B26, has to meet -20dBm in 37.5 KHz BW (i.e. integrated from 813.9625MHz till 814MHz). For NB1 devices in guard band operation, this imposes a stringent emission spec than the 3GPP ACLR/SEM limits.

Fig. 1. FCC Band edge emission limits for UE operating in B26

As seen in Figure1, as per FCC# 90.691, the spurious emission @ band edge of B26 should be attenuated from the desired carrier by (which-ever is less of the below)

  - 116*log10(F/6.1), where F is in KHz and is taken as 18.75 (i.e. 37.5KHz / 2), since integration BW is 37.5KHz. i.e. 56.5 dBc

  - 50 + 10*log10(P) where P is transmitted power in W, i.e. 43 dBc for Power Class 3 & 40dBc for Power Class 5.

  - 80dB.

Hence as per FCC # 90.691, we need to have the emission @ lower Band edge of B26, to be below -20dBm (i.e. -43dBc for class 3 or -40dBc for class 5 operation) integrated over 37.5KHz BW (i.e. from 813.9625MHz to 814MHz). For NB1 device operating in Guard band with LO at 814.1MHz (i.e. ARFCN # 26691), we see that this band-edge emission imposes a much stringent requirement than 3GPP SEM/ACLR limits.

# Discussion

For NB1 device operating in B26, ARFCN # 26691, multi-tone allocation, we observe that we would not be able to meet the band edge emission specification of -43dBc, even with the modem base band waveform (without even accounting for DA/PA nonlinearities) as shown in Fig. 2.

Fig. 2. NB1 –12 tone, BB Waveform spectrum, Band edge emission for ARFCN#26691, i.e. LO # 814.1 MHz

If we limit the NB1 guard band allocation to LO > = 814.2 MHz , then we would be able to meet the band edge emission limit of -43 dBc as shown in Figure 3.

Similarly, in case of single tone operation, for 3.75KHz sub-carrier spacing, we see that if the NB1 tone is operating in guard band operating with LO @ 814.1MHz, then we would be failing the FCC band edge limit, if the tone is allocated in the first 10 position (i.e. Pos # 0 till Pos # 10) for class 3 operation.

Hence for NB1 device allocated in ARFCN#26691, and operating in single tone, 3.75KHz sub-carrier spacing, we would like to limit the allocation to tone positions greater than pos # 10 (i.e. restrict Pos#0 till Pos#10 and allow only from pos#11 to Pos #47 for ARFCN# 26691 in B26).

Fig. 3. NB1 –12 tone Power spectrum, Band edge emission for LO # 814.2 MHz

Fig. 4. NB1 – Single tone 3.75KHz SC spacing, tone Pos # 0, BB Waveform spectrum, Band edge for LO # 814.1 MHz

Fig. 5. NB1 – Single tone 3.75KHz SC spacing, Pos #11, BB Waveform spectrum, Band edge for LO # 814.1 MHz

# Conclusion

Proposal:

To meet the FCC band edge requirement of -43dBc in B26:

- For NB1 multi-tone allocation operating guard band of B26, limit the allocation of LO>= 814.2MHz (i.e. ARFCN # 26692)

- In case of single tone, 3.75KHz SC spacing, operating in ARFCN#26691 (i.e. LO = 814.1MHz), limit the allocation to tone positions greater than pos # 10 (i.e. restrict Pos#0 till Pos#10 and allow only from pos#11 to Pos #47).

# References

[1]  https://apps.fcc.gov/kdb/GetAttachment.html?id=XcMYGDYWjK37iKmgc7iw%2Fg%3D%3D