3GPP TSG RAN WG4 Meeting #90	R4-1900014

Athens, Greece, 25th February – 1st March, 2019

3GPP TSG-RAN WG1 Ad-Hoc Meeting 1901			R1-1901460

Taipei, Taiwan, 21 - 25 January 2019

Title:	Reply to reply LS on wideband carrier operation for NR-U

Response to:	R1-1900008/R4-1816658

Release:	Rel-16

Work Item:	NR_unlic-Core

Source:	RAN WG1

To:	RAN WG4, RAN WG2

Cc:

Contact Person:

#### Name:	Seonwook Kim

E-mail Address:	seonwook.kim@lge.com

Attachments:	none

1. Description:

RAN1 would like to thank RAN4 for the Reply LS on wideband carrier operation for NR-U. RAN1 has discussed both DL and UL transmission on a wideband carrier (> 20 MHz) in unlicensed spectrum and made the following agreements:

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

Agreement:

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

Please note that amongst a variety of options listed in [1], RAN1 agreed to adopt options 2 and 3 for DL wide-band operation and not to support multiple active BWP operation for both DL and UL. RAN1 respectfully asks RAN4 for a response to FFS parts in the above agreement, in addition to any pending input to questions as requested in [1].

2. Actions:

To RAN WG4

ACTION: 	RAN1 respectfully asks RAN4 to answer the above issues, as well as questions in [1].

To RAN WG2

ACTION: RAN1 respectfully asks RAN2 to take the above RAN1 agreements into account in their further work.

3. Date of Next TSG-RAN WG1 Meetings:

RAN1#96	25th February – 1st March 2019		Athens, Greece

RAN1#96-bis	8th – 12th April 2019		Xi’an, China

4. References:

- R1-1812026	“LS on wideband carrier operation for NR-U”, RAN1