3GPP TSG-WG RAN4 Meeting #90	R4-1900070

# Athens, Greece, 25st Feb – 1st Mar, 2019

Source: 	Nokia, Verizon

Title: 					TP to TR 36.716-03-01: CA_2A-5A-46E_BCS0

Agenda Item: 		7.3.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA configuration (3 bands DL with 1 band UL) CA DL_2A-5A-46E_BCS0 into the TR.

**************************** Start of changes ********************************************

## 5.X	CA_2-5-46

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 3-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-5A-46E | 2 |  |  | Yes | Yes | Yes | Yes | 110 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

∆TIB and ∆RIB values for CA_2-5-46 are already specified in TS36.101

Table 5.1.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-5-46 | 2 | 0.3 |
|  | 5 | 0.3 |

Table 5.1.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-5-46 | 2 | 0 |
|  | 5 | 0 |

### 5.X.3	REFSENS requirements

REFSENS requirements of 2A-5A-46A is defined in TS36.101 and can be applied for 2A-5A-46E.

Table 5.X.3-1: Table 7.3.1A-0eA Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-5A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |

**************************** End of changes ********************************************