3GPP TSG-RAN WG4 Meeting #90												  	         R4-1900088

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia, TMO US

Title: 					TP to TR 36.716-03-02: CA_3DL_2A-12A-66A_2UL

Agenda Item: 		7.6.2

Document for:	Approval

# 1	Introduction

This TP introduces the following LTE inter-band CA configuration (3 bands DL with 2 band UL) CA_2A-12A-66A into the TR.

| Combi nation | # CC for DL | CA configurations | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes and intra contiguous/non-contiguous CA (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-12-66 | 3 | CA_3DL_2A-12A-66A_2UL_2A-12A_BCS0 | Rel-16 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | To be added later | new | Completed: CA_3DL_2A-12A-66A_1UL_BCS0 Completed: CA_2DL_2A-12A_2UL_2A-12A_BCS0 |
| 2-12-66 | 3 | CA_3DL_2A-12A-66A_2UL_2A-66A_BCS0 | Rel-16 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | To be added later | new | Completed: CA_3DL_2A-12A-66A_1UL_BCS0 Completed: CA_2DL_2A-66A_2UL_2A-66A_BCS0 |
| 2-12-66 | 3 | CA_3DL_2A-12A-66A_2UL_12A-66A_BCS0 | Rel-16 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | To be added later | new | Completed: CA_3DL_2A-12A-66A_1UL_BCS0 Completed: CA_2DL_12A-66A_2UL_12A-66A_BCS0 |

*********************** Start of the TP ***************************************************

## 6.X	LTE Advanced Carrier Aggregation: Band 2A and Band 12A and Band 66A with 2 bands UL

### 6.X.1	List of specific combination issues

#### 6.X.1.1 Channel bandwidths per operating band for CA

Table 6.X.1.1-1: Bandwidth combinations for 3DL/2UL_CA band combinations

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-12A-66A | CA_2A-12A CA_2A-66A CA_12A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 6.X.1.2	Co-existence studies for LTE-A UL CA_2A-12A and DL CA_2A-12A-66A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.x.1.2-1

Table 6.x.1.2-1: Harmonic and IMD analysis for DL_CA_2A-12A-66A with UL_CA_2A-12A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 699 | 716 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1398 | 1432 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2097 | 2148 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1211 | 1134 | 2549 | 2626 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3121 | 512 | 418 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4399 | 4536 | 3248 | 3342 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5031 | 187 | 298 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 | 2422 | 5098 | 2388 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6249 | 6446 | 3947 | 4058 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6941 | 6684 | 1014 | 886 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4332 | 4118 | 1552 | 1723 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 | 8356 | 4646 | 4774 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 | 7162 | 5797 | 5968 |

Based on Table 6.x.1.2-1, there is no IMDs products fall into own Rx of Band 66.

#### 6.x.1.3	Co-existence studies for LTE-A UL CA_2A-66A and DL CA_2A-12A-66A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.x.1.3-1

Table 6.x.1.3-1: Harmonic and IMD analysis for DL_CA_2A-12A-66A with UL_CA_2A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5270 | 4930 | 5930 | 5620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 1310 | 2310 | 1990 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9160 | 8970 | 9290 |

Based on Table 6.x.1.3-1, the 3rd order IMD products by the band 2 and the band 66 falls into the Rx frequency of band 2, the 5th order IMD products by the band 2 and the band 66 falls into the Rx frequency of band 66, However the IMD problem is already covered in 2DL/2UL CA_2A-66A in Table 7.3.1A-0f in TS 36.101. For band 12, there is no harmonics and IMDs products that fall into own Rx of Band 12.

#### 6.x.1.4	Co-existence studies for LTE-A UL CA_12A-66A and DL CA_2A-12A-66A

For 2UL / 3DL own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.x.1.4-1

Table 6.x.1.4-1: Harmonic and IMD analysis for DL_CA_2A-12A-66A with UL_CA_12A-66A

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 5130 | 5340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 994 | 1081 | 2409 | 2496 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 382 | 278 | 2704 | 2861 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3108 | 3212 | 4119 | 4276 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 317 | 438 | 4414 | 4641 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2162 | 1988 | 4818 | 4992 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3807 | 3928 | 5829 | 6056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6421 | 6124 | 1154 | 1016 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3942 | 3698 | 1272 | 1463 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7539 | 7836 | 4506 | 4644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6528 | 6772 | 5517 | 5708 |

Based on Table 6.x.1.4-1, the 4th order IMD products by the band 12 and the band 66 falls into the Rx frequency of band 2. It should be noted that the 4th order IMD products falls within 1988 – 2162 MHz, which has 2MHz overlap with the Rx frequency of band 2 (1930 - 1990 MHz). Therefore, the 4th order IMD products from Tx in band 12 and 66 to Rx in band 2 could be observed.  However, this 4th order IMD products could happen only in the case where Tx in the uppermost edge of band 12 with the lowest edge band 66 paired with Rx in the uppermost edge of band 2 is utilized.

#### 6.x.1.5	MSD

When uplink CA configurations CA_2A-66A is paired with downlink CA configuration CA_2A-12A-66A, the 3rd order IMD products by the band 2 and the band 66 falls into the own Rx frequency of band 2, the 5th order IMD products by the band 2 and the band 66 falls into the own Rx frequency of band 66. For this case, the MSD value from 2DL/2UL CA_2A-66A in Table 7.3.1A-0f in TS 36.101 can be reused. MSD summary is shown in Table 6.X.1.5-1.

Table 6.x.1.5-1: MSD summary for UL CA_2A-66A and DL CA_2A-12A-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-12A-66A | CA_2A-66A CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 5 | 20 | FDD | IMD3 |
|  |  | 66 | 1775 | 5 | 25 | 2175 | 5 | N/A |  | N/A |
|  |  | 12 | 705 | 5 | 25 | 735 | 5 | N/A |  | N/A |
|  |  | 2 | 1883.3 | 5 | 25 | 1963.3 | 5 | N/A | FDD | N/A |
|  |  | 66 | 1750 | 5 | 25 | 2150 | 5 | 4 |  | IMD5 |
|  |  | 12 | 705 | 5 | 25 | 735 | 5 | N/A |  | N/A |

When uplink CA configurations CA_12A-66A is paired with downlink CA configuration CA_2A-12A-66A, the 4th order IMD products from uplink in band 12 and 66 to downlink in band 2 could be observed.  However, this 4th order IMD products could happen only in the case where uplink in the uppermost edge of band 12 with the lowest edge band 66 paired with downlink in the uppermost edge of band 2 is utilized. MSD summary is shown in Table 6.X.1.5-2.

Table 6.x.1.5-2: MSD summary for UL CA_12A-66A and DL CA_2A-12A-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-12A-66A | CA_12A-66A | 2 | 1907.5 | 5 | 25 | 1987.5 | 5 | [2.5] | FDD | IMD4 |
|  |  | 12 | 713.5 | 5 | 25 | 743.5 | 5 | N/A |  | N/A |
|  |  | 66 | 1712.5 | 5 | 25 | 2112.5 | 5 | N/A |  | N/A |

No MSD value is defined for CA_2A-12A-66A in TS 36.101, as a result the proposed number in table 6.x.1.5-2 is determined using CA_2A-12A-30A as a baseline. For CA_2A-12A-30A a MSD of 12 is given for a IMD4 covering the full band, 10 MHz, while there for CA_2A-12A-66A is a maximum of only 2 MHz overlap. Scaling the MSD of 12 proportional to the different level of overlap results in the proposed MSD of 2.5.

#### 6.X.1.6	∆TIB and ∆RIB values

∆TIB and ∆RIB values are already specified in TS36.101 for combination 3DL/1UL CA_2-12-66 which can be applied here.

Table 6.X.2-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-12A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |

Table 6.X.2-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-12A-66A | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |

*********************** End of the TP ***************************************************