3GPP TSG-RAN WG4#90 Meeting	R4-1900102

Athens, Greece, 25 Feb. – 1 Mar. 2019

Agenda item:	6.14.1.4.2

Title: 	Simulation results and discussion on NR CQI reporting

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] was approved for NR UE demodulation and CSI feedback performance requirements. And, the agreements related to the NR UE CSI performance requirements were reached in the WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

In RAN4-AH-1807 Meeting (July 2018), a WF [4] on NR UE CSI requirements was approved regarding the initial simulation assumptions for FR1 and FR2 CQI/PMI/RI tests. Companies were encouraged to provide initial simulation results and views on the assumptions and scenarios for NR UE CSI feedback tests. In RAN4#88 Meeting (August 2018), in [5], we provided our initial results for FR1 FDD and FR2 TDD CQI reporting tests. Also, in RAN4#88, RAN4#88bis and RAN4#89 Meetings, WFs [6, 7, 8] were agreed and companies were encouraged to bring CQI simulation results (wideband and sub-band CQI) under fading channel conditions. In this contribution, we first present our WB and SB CQI results under fading channel conditions. And then, we share our views on the NR CQI reporting test.

# Wideband CQI reporting results in fading channels

In this section, we provide our simulation results for FR1 FDD and TDD wideband CQI reporting in TDL-A fading channel conditions. The simulation assumptions can be found in [8]. Moreover, it was also agreed in [6] to reuse LTE test metric for WB CQI tests under fading channel conditions:

- a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time;

- the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 1: FR1 FDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 51.4% | 1.306 | 0.0997 |
| 7 | 50.95% | 1.419 | 0.104 |
| 12 | 45.6% | 1.295 | 0.1074 |
| 13 | 45.8% | 1.206 | 0.1159 |

Table 2: FR1 TDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 46.05% | 1.408 | 0.0843 |
| 7 | 44.95% | 1.322 | 0.0865 |
| 12 | 41.75% | 1.33 | 0.1028 |
| 13 | 40.20% | 1.213 | 0.1057 |

Table 3: FR1 FDD 4Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 3 | 48.35% | 1.288 | 0.0971 |
| 4 | 47% | 1.277 | 0.0959 |
| 9 | 41.85% | 1.276 | 0.1367 |
| 10 | 42.3% | 1.205 | 0.1389 |

Table 4: FR1 TDD 4Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 3 | 42.05% | 1.2995 | 0.1057 |
| 4 | 45.2% | 1.328 | 0.1153 |
| 9 | 40.45% | 1.3216 | 0.1381 |
| 10 | 40.05% | 1.214 | 0.1562 |

Table 5: FR2 TDD 2Rx WB CQI results

| SNR | α % not in +/- Med CQI | Throughput ratio (follow/medium) | BLER for follow CQI |
| --- | --- | --- | --- |
| 6 | 14.96% | 1.1912 | 0.0757 |
| 7 | 12.53% | 1.1177 | 0.0849 |
| 12 | 10.24% | 1.294 | 0.0968 |
| 13 | 7.02% | 1.1081 | 0.1085 |

Based on the simulation results, we have the following proposals:

Proposal 1: for FR1 FDD and TDD WB CQI test cases, reuse LTE requirements that:

- a%>20%

- gamma>1.05

- BLER>0.02

Proposal 2: for FR2 TDD WB CQI test cases, define requirements that:

- a%>2%

- gamma>1.02

- BLER>0.01

# Sub-band CQI reporting results in fading channels

In this section, we provide our initial simulation results for FR1 FDD sub-band CQI reporting in fading channel conditions. The simulation assumptions can be found in [8]. Moreover, it was also agreed in [6] to reuse LTE test metric for SB CQI tests under fading channel conditions:

- sub-band differential CQI offset level of 0 shall be reported at least α % of the time but less than β % for each sub-band;

- the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ γ;

- when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [TBD].

Table 6: FR1 FDD (Option 1: 2x2) SB CQI results

| SNR | Probability of 0 CQI offset | Throughput ratio (maxSB/medWB) | BLER for follow max SB CQI |
| --- | --- | --- | --- |
| 6 | 44.34% | 1.083 | 0.0564 |
| 12 | 37.18% | 1.091 | 0.1111 |

Table 7: FR1 FDD (Option 2: 1x2) SB CQI results

| SNR | Probability of 0 CQI offset | Throughput ratio (maxSB/medWB) | BLER for follow max SB CQI |
| --- | --- | --- | --- |
| 11 | 18.32% | 1.6367 | 0.3608 |
| 12 | 18.32% | 1.4909 | 0.3588 |

Based on simulation results in Table 6 and Table 7, it is observed that “Option 2: 1x2” antenna configuration achieves higher throughput ratio and more suitable for testing. Therefore,

Proposal 3: Apply “Option 2: 1x2” antenna configuration for FR1 SB CQI test cases.

# Conclusion

In this contribution, we present our WB and SB CQI results under fading channel conditions for alignment purpose, and have the following proposals:

Proposal 1: for FR1 FDD and TDD WB CQI test cases, reuse LTE requirements that:

- a%>20%

- gamma>1.05

- BLER>0.02

Proposal 2: for FR2 TDD WB CQI test cases, define requirements that:

- a%>2%

- gamma>1.02

- BLER>0.01

Proposal 3: Apply “Option 2: 1x2” antenna configuration for FR1 SB CQI test cases.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung, July 2018

[5] R4- 1809798, “Initial simulation results and discussion on NR CQI tests”, Intel Corporation, August 2018

[6] R4- 1811693, “Way forward on NR UE CSI requirements”, Samsung, August 2018

[7] R4- 1814235, “Way forward on NR UE CSI requirements”, Samsung, October 2018

[8] R4- 1816491, “Way forward on NR UE CSI requirements”, Samsung, November 2018