3GPP TSG-RAN WG4 Meeting #90	 	R4-1900110

Athens, GR, 25th Feb – 1st March, 2019

Agenda item:       5.5.4

Source:                Intel Corporation

Title:	       CQI reporting requirements with 1024QAM

Document for:     Discussion

# Introduction

In RAN4#89 the demodulation and CSI requirements for 1024QAM were finalized. Way forward on demodulation and CSI requirements for 1024QAM was approved [1]. For CQI reporting tests the following was agreed:

- CQI test point SNR = 28/29dB - No additional test point is introduced in WI phase. If companies are still willing to bring the results in Feb meeting, additional test point can be added in TEI of this work

In this contribution we present our motivation to introduce additional test point for CQI reporting test with 1024QAM.

# Discussion

With the introduction of 1024QAM a new CQI table is introduced in TS 36.213. In order to verify CQI reporting with 1024QAM it was agreed to introduce 1 test with TM1 under static channel condition. The agreed test point SNR for CQI reporting test is 28/29 dB. The high SNR verifies CQI reporting for 1024QAM, but with the introduction of new CQI table the entries for other modulation have been modified. In the CQI table below for 256QAM the entries deleted in order to define the 1024QAM CQI table are highlighted.

Table 1: 256QAM and 1024QAM CQI Tables

| Table 7.2.3-2: 4-bit CQI Table 2 (For 256QAM). CQI index; modulation; code rate x 1024; efficiency. 0; out of range. 1; QPSK; 78; 0.1523. 2; QPSK; 193; 0.377. 3; QPSK; 449; 0.877. 4; 16QAM; 378; 1.4766. 5; 16QAM; 490; 1.9141. 6; 16QAM; 616; 2.4063. 7; 64QAM; 466; 2.7305. 8; 64QAM; 567; 3.3223. 9; 64QAM; 666; 3.9023. 10; 64QAM; 772; 4.5234. 11; 64QAM; 873; 5.1152. 12; 256QAM; 711; 5.5547. 13; 256QAM; 797; 6.2266. 14; 256QAM; 885; 6.9141. 15; 256QAM; 948; 7.4063. | Table 7.2.3-2: 4-bit CQI Table FOR 1024QAM. CQI index; modulation; code rate x 1024; efficiency. 0; out of range. 1; QPSK; 78; 0.1523. 2; QPSK; 193; 0.377. 3; QPSK; 449; 0.877. 4; 16QAM; 378; 1.4766. 5; 16QAM; 616; 2.4063. 6; 64QAM; 567; 3.3223. 7; 64QAM; 666; 3.9023. 8; 64QAM; 772; 4.5234. 9; 64QAM; 873; 5.1152. 10; 256QAM; 711; 5.5547. 11; 256QAM; 797; 6.2266. 12; 256QAM; 885; 6.9141. 13; 256QAM; 948; 7.4063. 14; 1024QAM; 853; 8.3321. 15; 1024QAM; 948; 9.2578. |
| --- | --- |

The discontinuities in CQI table for 1024QAM compared to 256QAM are at CQI index [4, 5] and [5, 6]. In order to target the additional SNR test point around the region where entries have been deleted in the CQI table and having the same modulation order, CQI index [4, 5] are suitable.

Observation #1: The target SNR region for additional test point for CQI reporting with 1024QAM corresponds to CQI index of [4, 5]

Figure 1: CQI Reporting with 1024QAM

Based on simulation results the SNR range where median CQI of 4 is reported is 3 – 5 dB and median CQI of 5 is reported in SNR range 6 – 9 dB.

Observation #2: The SNR range where median CQI of 4 is reported is 3 – 5dB and CQI 5 is reported in SNR range 6 – 9 dB

SNR points of 4/5 dB or 6/7 dB are suitable additional test point SNRs for 1024QAM CQI reporting. We recommend introducing additional CQI reporting test at 4/5dB or 6/7 dB for 1024QAM CQI reporting.

Proposal #1: Introduce additional SNR test point of 4/5 dB or 6/7 dB for 1024QAM CQI reporting

For the proposed SNR levels, we observe the following results:

Table 2: CQI Reporting Test results with 1024QAM

|  | SNR | Med CQI | MCS for Med CQI | BLER @ Med | BLER @ Med-1 | BLER @ Med+1 |
| --- | --- | --- | --- | --- | --- | --- |
| 16QAM | 4 | 4 | 5 | 0 | 0 | 1 |
|  | 5 | 4 | 5 | 0 | 0 | 0.67 |
|  | 6 | 5 | 7 | 0 | 0 | 1 |
|  | 7 | 5 | 7 | 0 | 0 | 1 |
| 1024QAM | 28 | 14 | 23 | 0 | 0 | 0.55 |
|  | 29 | 14 | 23 | 0 | 0 | 1 |

# Conclusion

In this paper we present our motivation to introduce additional test point for CQI reporting test with 1024QAM. Our observations and proposals are as follows:

Observation #1: The target SNR region for additional test point for CQI reporting with 1024QAM corresponds to CQI index of [4, 5]

Observation #2: The SNR range where median CQI of 4 is reported is 3 – 5dB and CQI 5 is reported in SNR range 6 – 9 dB

Proposal #1: Introduce additional SNR test point of 4/5 dB or 6/7 dB for 1024QAM CQI reporting

# Reference

- R4-1816384, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon