3GPP TSG-RAN WG4 Meeting #90	 	R4-1900116

Athens, GR, 25th Feb – 1st March, 2019

Agenda item:       6.12.8.1.2

Source:                Intel Corporation

Title:	                     On Remaining Issues for Candidate Beam Detection

Document for:     Discussion

# Introduction

In RAN4#89 candidate beam detection (CBD) for link recovery procedure was discussed and the following agreements were made:

- Define requirements for CSI-RS based CBD with D=3; no requirements with D=1 - Evaluation period for CSI-RS based CBD is 3 samples (accuracy requirement is FFS) - Resource sharing factor (P) for CBD shall be same as RLM measurements - In FR2, for SSB and CSI-RS based CBD, N=8 always applies

In this contribution we address the remaining open issues related to candidate beam detection.

# Discussion

In RAN4#89, the evaluation period for CSI-RS based CBD was agreed as 3 samples, the same as SSB based CBD. But the accuracy requirements for SSB and CSI-RS based CBD are FFS.

In order to define the accuracy requirements for SSB based CBD, we measured the estimation error for SSB based L1-RSRP measurement for side condition of -3dB for FR1 and FR2. The evaluation period for SSB based CBD is 3 samples.

Table 1: SSB based L1-RSRP Measurement accuracy in FR1

|  | TDLA30; 10Hz | TDLB100; 400Hz | TDLC300; 100Hz |
| --- | --- | --- | --- |
| 15KHz SCS | 1.46 | 1.57 | 1.54 |
| 30KHz SCS | 1.57 | 1.54 | 1.57 |

Table 2: SSB based L1-RSRP Measurement accuracy in FR2

|  | TDLA30; 75Hz | TDLA30; 300Hz | TDLC60; 300Hz |
| --- | --- | --- | --- |
| 120KHz SCS | 1.59 | 1.66 | 1.65 |
| 240KHz SCS | 1.62 | 1.64 | 1.63 |

For SSB based L1-RSRP measurement we observe that the estimation error is ~1.7 dB with 3 samples without considering baseband implementation and RF margin. Taking into account baseband implementation and RF margin, we recommend defining the accuracy requirement as 5 dB.

Proposal #1: Define accuracy requirement for SSB based Candidate Beam Detection as 5dB

In order to define the accuracy requirements for CSI-RS based CBD, we measured the estimation accuracy for CSI-RS based L1-RSRP measurement for side condition of -3dB for 3 samples in FR1 and FR2. The results are captured in tables below.

Table 3: CSI-RS based L1-RSRP Measurement accuracy in FR1

|  | CSI-RS BW | TDLA30; 10Hz | TDLB100; 400Hz | TDLC300; 100Hz |
| --- | --- | --- | --- | --- |
| 15KHz SCS | 24 PRB | 1.57 | 1.68 | 1.62 |
| 30KHz SCS |  | 1.72 | 1.62 | 1.60 |
| 15KHz SCS | 48 PRB | 1.36 | 1.35 | 1.46 |
| 30KHz SCS |  | 1.35 | 1.34 | 1.44 |

Table 4: CSI-RS based L1-RSRP Measurement accuracy in FR2

|  | CSI-RS BW | TDLA30; 75Hz | TDLA30; 300Hz | TDLC60; 300Hz |
| --- | --- | --- | --- | --- |
| 60KHz SCS | 24 PRB | 1.76 | 1.68 | 1.76 |
| 120KHz SCS |  | 1.62 | 1.73 | 1.7 |
| 60KHz SCS | 48 PRB | 1.45 | 1.43 | 1.39 |
| 120KHz SCS |  | 1.41 | 1.41 | 1.41 |

With 3 samples evaluation period for CSI-RS based L1-RSRP measurement, we observe that the estimation error is ~1.8 dB across all scenarios for CSI-RS BW of 24PRB without baseband implementation and RF margin. Taking into account baseband implementation and RF margin, we recommend defining the accuracy requirement as 5 dB.

Proposal #2: Define accuracy requirement for CSI-RS based Candidate Beam Detection as 5dB

# Conclusion

In this paper we propose the accuracy requirements for candidate beam detection. Our proposals are as follows:

Proposal #1: Define accuracy requirement for SSB based Candidate Beam Detection as 5dB

Proposal #2: Define accuracy requirement for CSI-RS based Candidate Beam Detection as 5dB