3GPP TSG-RAN WG4 Meeting #90 																  	R4-1900138

Athens, GR, 25th Feb – 1st March, 2019

Agenda Item:	6.12.7.4

Source:	Intel Corporation

Title:	Discussion on NE-DC PSCell addition requirements

Document for:		Discussion

# Introduction

In the latest RAN4 meeting the overall scope of NE-DC requirements was agreed in [1]. In this contribution we provided some considerations on the requirements for E-UTRA PSCell addition delay.

# Discussion

In [1], NR PSCell addition and release delay requirements need to be introduced for NE-DC. Generally, according to RAN2 agreements [2] the overall procedure of PSCell addition can be illustrated in the figure below.

Figure 1. PSCell addition in MR-DC[2]

The delay of PSCell addition can be start from “RRCConnectionReconfiguation” and end by UE successful random access procedure. That is

Observation 1:  The delay of E-UTRA PSCell addition in NE-DC can be mainly composed of:

- RRC singling delay and RRC processing time

- E-UTRA cell search time: e.g. PSS/SSS detection and SFN detection

- E-UTRA SCell activation time(e.g. AGC settling)

- Other uncertainty in acquiring the first available PRACH

For RRC delay and processing time, it can rely on NR PCell transmission and reception. Thus they can following RRC delay in TS38.331[3] and Tprocessing in TS36.133[4]. Thus RRC processing time can be up 20ms.

As PSCell to be added in NE-DC is E-UTRA cell, the duration for Step 2&3 which shall be counted into the total addition delay shall follow the rules of LTE operations as what we specified in TS36.133. For example,

- SFN detection time for LTE is about 50ms.

- SCell activation time is 20ms and 30ms for the known and unknown cases respectively

More specially, in comparison EN-DC, the time of uncertainty in acquirement the first available PRACH needs more consideration. For example, in synchronous EN-DC, the maximum UL timing difference between NR PCell and E-UTRA PScell shall be required. In our view, the similar requirements on synchronous NE-DC shall be applied. Additionally, in [5], it is allowed for UE to support only single UL timing, even when the network configures two separate TAGs. That means the MTTD for intra-band contiguous MR-DC can be zero.

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC[4]

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 | 35.21 |
| NOTE 1: Void |  |  |

Regarding to these two aspects of NE-DC above, UE needs maintain the exact UL timing for both PCell and PScell. As a result, it is possible for UE with NE-DC capability to receive NR PRACH and E-UTRA PRACH simultaneously. In order to obtain successful first PRACH for both NR PCell and E-UTRA PSCell when PScell configuration,  the longer duration between TPCell_ DU and TPSCell_ DU shall be included in PSCell addition delay.

Observation 2:  Uncertainty in acquiring the first available PRACH can max{ TPCell_ DU , TPSCell_ DU }.

TPCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PCell. TPCell_ DU is up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of TS38.211 [3].TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the E-UTRA PCSell which is up 30ms.

The other important aspect of NE-DC to be considered is the asynchronous operation. As we stated in [6], in order to enable the deployment flexibility both synchronous and asynchronous deployment can be used as the baseline for NE DC RRM requirements.

Observation 3: the time for SFN timing acquiring shall be included in the total delay for PSCell additional if asynchronous NE-DC supported.

With these observations we can summarize:

Proposal 1: E-UTRA PSCell addition delay requirements in NE-DC can be: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + max{ TPCell_ DU , TPSCell_ DU }  + 2 ms

| Tconfig_PSCell | NE-DC: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + max{ TPCell_ DU , TPSCell_ DU } + 2 ms |  | Notes |
| --- | --- | --- | --- |
|  | Unknown | Known |  |
| TRRC_delay: RRC procedure delay | Defined in [TS38.331]. |  |  |
| Tprocessing: SW processing time needed by UE, including RF warm up period | 20 ms |  |  |
| Tsearch: the time of AGC and,PSS/SSS detection for LTE PSCell | 20ms | 30ms |  |
| T∆: For LTE PSCell, it is SFN timing acquiring | 50ms |  |  |
| TPSCell_ DU: the delay uncertainty in acquiring the first LTE available PRACH occasion in the LTE PSCell | 30ms. |  |  |
| TPCell_ DU: is the delay uncertainty in acquiring the first available PRACH occasion in the NR PCell. | up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of TS38.211. |  |  |

For the specification structure itself, in order to improve readability of TS38.133[5], the new separated section shall be used for this new NE DC PSCell addition/release requirements.

Proposal 2: New section in TS38.133 shall be added to specify E-UTRA PSCell addition/release requirements.

# Conclusion

In this contribution, further considerations on the remaining issues of NE-DC PSCell addition requirements. In conclusion, the following observations and proposals can be drawn:

Observation 1:  The delay of PSCell addition in NE-DC can be mainly composed of:

- RRC singling delay and RRC processing time

- E-UTRA cell search time: e.g. PSS/SSS detection and SFN detection

- E-UTRA SCell activation time(e.g. AGC settling)

- Other uncertainty in acquiring the first available PRACH

Observation 2:  Uncertainty in acquiring the first available PRACH can max{ TPCell_ DU , TPSCell_ DU }.

TPCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the NR PCell. TPCell_ DU is up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of TS38.211 [3].TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the E-UTRA PCSell which is up 30ms.

Observation 3: the time for SFN timing acquiring shall be included in the total delay for PSCell additional if asynchronous NE-DC supported.

Proposal 1: E-UTRA PSCell addition delay requirements in NE-DC can be: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + max{ TPCell_ DU , TPSCell_ DU }  + 2 ms

| Tconfig_PSCell | NE-DC: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + max{ TPCell_ DU , TPSCell_ DU } + 2 ms |  | Notes |
| --- | --- | --- | --- |
|  | Unknown | Known |  |
| TRRC_delay: RRC procedure delay | Defined in [TS38.331]. |  |  |
| Tprocessing: SW processing time needed by UE, including RF warm up period | 20 ms |  |  |
| Tsearch: the time of AGC and,PSS/SSS detection for LTE PSCell | 20ms | 20+10ms |  |
| T∆: For LTE PSCell, it is SFN timing acquiring | 50ms |  |  |
| TPSCell_ DU: the delay uncertainty in acquiring the first LTE available PRACH occasion in the LTE PSCell | 30ms. |  |  |
| TPCell_ DU: is the delay uncertainty in acquiring the first available PRACH occasion in the NR PCell. | up to x*10 +10 ms. x is defined in the table 6.3.3.2-2 of TS38.211. |  |  |

Proposal 2: New section in TS38.133 shall be added to specify E-UTRA PSCell addition/release requirements.

# References

- R4-1811344, “Way forward on RRM requirements for NE-DC”

- 3GPP TS 37.340

- 3GPP TS 38.331 v15.4.0

- 3GPP TS 38.133 v15.4.0

- 3GPP TS 38.306 v15.4.0

- R4-1900135, “Discussion on NE-DC interruption requirements”