3GPP TSG-RAN WG4 RAN4#90 Meeting	R4-1900199

# Athens, Greece, 25th Feb. – 1st March, 2019

Source:	Verizon, LG Electronics

Title:	TP for TR 36.716-03-02: LTE inter-band CA 3 bands DL with 2 bands UL

Agenda item:	7.6.2

Document for:	Approval

Introduction

The purpose of this document is to introduce co-existence studies and MSD analysis results [1] of LTE inter-band carrier aggregation 3 bands DL with 2 bands UL combinations for TR 36.716-03-02.

Text Proposal

<Start of TP>

----- Unchanged sections omitted -----

6.x LTE-A inter-band CA: Band  2 and Band 4 and Band 5 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  |  | Uplink CA configurations |  |  | E-UTRA Bands |  |  | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  |  |  | Maximum aggregated bandwidth [MHz] |  |  | Bandwidth combination set |
| CA_2A-2A-4A-5A | CA_4A-5A | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  |  |  |  | 4 |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_ 2A-2A-4A-5A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_with UL CA_

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 2472 | 2547 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 931 | 861 | 2534 | 2604 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 2571 | 2686 | 107 | 12 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 4244 | 4359 | 3358 | 3453 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 4281 | 4441 | 717 | 837 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 5954 | 6114 | 4182 | 4302 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD freqeuncy limit (MHz) | 1722 | 1862 | 5068 | 5208 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD freqeuncy limit (MHz) | 1686 | 1541 | 6196 | 5991 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD freqeuncy limit (MHz) | 5006 | 5151 | 7664 | 7869 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD freqeuncy limit (MHz) | 873 | 1038 | 3617 | 3432 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD freqeuncy limit (MHz) | 5892 | 6057 | 6778 | 6963 |

- 6.x.1.3	MSD

When uplink CA_4A_5A is paired with downlink CA_2A-2A-4A-5A, the 2nd order IMD product by band 4 and band 5 falls into the own Rx frequency band 5.

When uplink CA_4A_5A is paired with downlink CA_2A-2A-4A-5A, the 5th order IMD product by Band 4 and band 5 falls into its own Rx frequency band 5.

2nd and 5th IMD problems were already covered in Table 7.3.1A-0f: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations of TS36.101.

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 5 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations |  | E-UTRA Bands |  | 1.4 MHz |  | 3 MHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  |  | 20 MHz |  |  | Maximum aggregated bandwidth [MHz] |  | Bandwidth combination set |  |
| CA_2A-5B-66A-66A | CA_5A-66A | 2 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 5 |  | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 80 |  | 0 |  |  |
|  |  |  | 66 |  | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-5B-66A-66A and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_with UL CA_

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 132 | 12 | 2571 | 2736 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 4182 | 4327 | 5954 | 6189 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD freqeuncy limit (MHz) | 1912 | 1722 | 5068 | 5258 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD freqeuncy limit (MHz) | 6296 | 5991 | 1686 | 1516 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD freqeuncy limit (MHz) | 7664 | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD freqeuncy limit (MHz) | 3692 | 3432 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD freqeuncy limit (MHz) | 6778 | 7038 | 5892 | 6107 |

- 6.x.1.3	MSD

When uplink CA_5A_66A is paired with downlink CA_2A-5B-66A-66A, the 2nd order IMD product by band 5 and band 66 falls into the own Rx frequency band 5.

When uplink CA_5A_66A is paired with downlink CA_2A-5B-66A-66A, the 5th order IMD product by Band 5 and band 66 falls into its own Rx frequency band 5.

2nd and 5th IMD problems were already covered in Table 7.3.1A-0f: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations of TS36.101.

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 66 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 |  |  |  | Yes |  | Yes | Yes | Yes | 70 | 0 |
|  |  | 13 |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-13A-66A-66B and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_with UL CA_

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD freqeuncy limit (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD freqeuncy limit (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD freqeuncy limit (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD freqeuncy limit (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD freqeuncy limit (MHz) | 6684 | 6914 | 5751 | 5921 |

- 6.x.1.3	MSD

When uplink CA_13A_66A is paired with downlink CA_2A-13A-66A-66B, the 4th order IMD product by band 13 and band 66 falls into the own Rx frequency band 2.

In Table 6.X.1.3-1, evaluated MSD values of 3 bands DL_CA_2A-13A-66A-66B with 2 bands UL is shown.

Table 6.x.1.3-1: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-66A-66B

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | TBD | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  |  |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  |  |

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

6.x LTE-A inter-band CA: Band 2 and Band 13 and Band 48 DL with 2 bands UL

6.x.1	List of specific combination issues

- 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-13A-48A-48C | CA_2A-13A | 2 |  |  |  | Yes |  | Yes | Yes | Yes | 90 | 0 |
|  |  | 13 |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes | Yes |  | Yes | Yes |  |  |
|  |  | 48 |  | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- 6.x.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-13A-48A-48C and 
2 bands UL

For 3 bands DL with 2 bands UL, own receiver desensitization study 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were analyzed in Table 6.x.1.2-1.

Table 6.x.1.2-1: Co-existence study for DL CA_with UL CA_

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 777 | 787 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1554 | 1574 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 2331 | 2361 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 1133 | 1063 | 2627 | 2697 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 2913 | 3043 | 356 | 276 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 4477 | 4607 | 3404 | 3484 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD freqeuncy limit (MHz) | 4763 | 4953 | 421 | 511 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD freqeuncy limit (MHz) | 6327 | 6517 | 4181 | 4271 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD freqeuncy limit (MHz) | 2126 | 2266 | 5254 | 5394 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD freqeuncy limit (MHz) | 1298 | 1198 | 6863 | 6613 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD freqeuncy limit (MHz) | 4958 | 5058 | 8177 | 8427 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD freqeuncy limit (MHz) | 1339 | 1489 | 4176 | 3976 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD freqeuncy limit (MHz) | 6031 | 6181 | 7104 | 7304 |

- 6.x.1.3	MSD

When uplink CA_2A_13A is paired with downlink CA_2A-13A-48A-48C, there are no harmonic and IMD products falling into own Rx bands. Therefore, no MSD needs to be addressed,

- 6.x.1.4	∆TIB and ∆RIB values

The requirements of low-order combination from TS36.101 [2] can be applied.

----- Unchanged sections omitted -----

<End of TP>

- References

[1] R4-1812214, “TP on summary of self-interference analysis for new x bands DL (x=3,4,5) with 2 bands UL CA in rel-16”, LG Electronics Finland

[2] TS 36.101 v15.4.0