3GPP TSG-RAN WG4 Meeting #90                                         R4-1900303

Athens, GR, 25 Feb - 1 Mar, 2019

Agenda Item:   6.10.2.3

Source:        Samsung

Title:           View on BS TM2 synchronization

Document for:  Discussion

# Introduction

One remaining issue for BS conformance testing is the TM2 synchronization aspect as summarized in WF [1]. Regarding this issue, this contribution provides our preference on the candidate solutions.

# Discussion

There are totally 4 specific options captured in the WF. Before go through the options one by one, the fundamental thinking should be taken into account is,

- Configuration shall be aligned with physical layer design, i.e., special pattern for TM only shall be forbidden as much as possible.

- Both synchronization performance and test system design simplicity shall be considered.

Based on above thinking, the comparison among solutions is summarized as table below.

Table 1: comparison among option in WF

| Solution | Consistency with physical layer | Synchronization performance | Test system design complexity |
| --- | --- | --- | --- |
| Option 1: keep existing IM2 configuration | Yes | May be issue due to significant lower DMRS density in frequency domain | No issue identified |
| Option 2: Create a synchronization slot(s) with full bandwidth DM-RS which will not be used for measurement | Yes | The same performance as other TM can be envisioned | No issue identified |
| Option 3: Create similar full bandwidth allocation of DMRS | No | The same performance as other TM can be envisioned | Additional effort to construct this pattern |
| Option 4: Create more single PRB used for testing | Yes | Performance depends on frequency density of DMRS | Additional effort to construct this pattern |

According to the comparison, if it can be concluded that with the option 1 the similar sync performance can be maintained or could be confirmed by TE vendor, option 1 would be with the first priority. Otherwise, option 2 would be the preference.

If option 2 is selected, the proposed change for NR-FR1-TM2 and NR-FR2-TM2 would be the same as below:

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2 /

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB for slots under measurement | Slot; RB; n. 3n; 0; n=01,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated for slots under measurement | NRB-1 |
| Location of slot for synchronization only | Slot; RB#. 0; NRB. |

In addition, to insure the sync performance of existing TM2, beside DMRS, the other supplement solution could be to introduce TRS for frequency and timing compensation. The example configuration of TRS for FR1 and FR2 is exhibited in below table. However, it may also have impact on test system complexity. Hence it is just shown here for discussion.

Table 2: example configuration for TRS in TM2

| CSI-RS for tracking | Parameter | Unit | Value for FR1 | Value for FR2 |
| --- | --- | --- | --- | --- |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | k0=2 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 |  |
|  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  | CDM Type |  | ‘No CDM |  |
|  | Density (ρ) |  | 3 |  |
|  | CSI-RS periodicity | ms | 10 |  |
|  | CSI-RS offset | Slots | 15 kHz SCS: 5 for CSI-RS resource 1 and 2 6 for CSI-RS resource 3 and 4 30 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |

# Conclusion

In this contribution, our understanding on BS TM2 sync problem is shared. The corresponding change on spec is also proposed.

# Reference

[1] R4-1816728, WF on Synchronization design for TM2 for FR1 and FR2

[2] 38.211, NR Physical channels and modulation

[3] 38. 101-4, User Equipment (UE) radio transmission and reception Part 4: Performance requirements