# 3GPP TSG-RAN WG4 Meeting #90                                                                  R4-1900317

# Athens, Greece, 25th Feb - 1st March, 2019

Agenda Item:	6.14.2.1

Source:	ZTE Corporation

Title:	Excel script for deriving NR performance requirements

Document for:	Approval

# Introduction

In RAN4 #89, a rule was approved on how to derive NR performance requirements for all the agreed cases [1] from all of the simulation results, both ideal and impairment, provided by different companies:

- Procedure to derive the performance requirements:

  - Only inputs that consist of a pair of ideal and impaired results can be taken into account.

  - If the ideal span <= [2dB]:

    - The AVERAGE impairment results can be used for the performance requirement with [] in the TPs;

  - Else if the ideal span is larger than [2dB]:

    - The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available:

      - The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the TPs.

    - Otherwise put TBD for the related performance requirements.

  - [If the span of the impairment results after removal the outliers (if any) are larger than [3dB], then the procedure cannot be applied, related performance requirement remain TBD.](*)

  - The agreed SNR values during this meeting remains in [] and can be updated if more or updated results are collected from companies in future meeting or technical issues are identified.

As discussed during the meeting, it would be quickly and automatically computed if an excel script implements the above rule. Hereby we attach a function named DerivingPerfReq in excel VBA language for this purpose.

# Discussion

Public Function DerivingPerfReq(SVin As Range) As Double

Dim i, N, M As Long

Dim Min_ideal, Max_ideal, Min_impairment, Max_impairment As Double

Dim Min_ideal_pos, Max_ideal_pos, Min_impairment_pos, Max_impairment_pos As Integer

Dim Avg_ideal, Avg_impairment As Double

Dim SimVal(100) As Double

Dim Avg As Double

Dim Avg_pos As Double

Dim IdealSpan_Threshold, ImpairmentSpan_Threshold As Double

Dim num_Effective As Integer

IdealSpan_Threshold = 2

ImpairmentSpan_Threshold = 3

N = SVin.Rows.Count

If N > 1 Then

'MsgBox ("Must be in the same row")

DerivingPerfReq = -200

Exit Function   ' exit function

End If

Avg = SVin.Cells(1, 1).Value

N = SVin.Count

num_Effective = 0

'Copy in simulation results

For i = 0 To N - 1

If IsEmpty(SVin.Cells(1, i + 1).Value) = False Then

SimVal(num_Effective) = SVin.Cells(1, i + 1).Value

num_Effective = num_Effective + 1

Else

If i Mod 2 = 0 Then ' excluding the company's input if either ideal or impairment is missing

i = i + 1

End If

End If

Next i

M = (N / 2)

If N <> 2 * M Then

'MsgBox ("Simulation results must be provided in pairs of ideal and impairment results")

DerivingPerfReq = -101

End If

M = num_Effective

If M < 3 Then

'MsgBox ("Not enough Simulations: at least 3 companies")

DerivingPerfReq = -100

Else:

Min_ideal = SimVal(0)

Max_ideal = SimVal(0)

Min_impairment = SimVal(1)

Max_impairment = SimVal(1)

Min_ideal_pos = 0

Max_ideal_pos = 0

Min_impairment_pos = 1

Min_impairment_pos = 1

Avg_ideal = Min_ideal

Avg_impairment = Min_impairment

' Find the spans and average for each

For i = 1 To M - 1

If Min_ideal > SimVal(2 * i) Then

Min_ideal = SimVal(2 * i)

Min_ideal_pos = i

End If

If Max_ideal < SimVal(2 * i) Then

Max_ideal = SimVal(2 * i)

Max_ideal_pos = i

End If

If Min_impairment > SimVal(2 * i + 1) Then

Min_impairment = SimVal(2 * i + 1)

Min_impairment_pos = i

End If

If Max_impairment < SimVal(2 * i + 1) Then

Max_impairment = SimVal(2 * i + 1)

Max_impairment_pos = i

End If

Avg_ideal = Avg_ideal + SimVal(2 * i)

Avg_impairment = Avg_impairment + SimVal(2 * i + 1)

Next i

Avg_ideal = Avg_ideal / M

Avg_impairment = Avg_impairment / M

' Check if ideal span is within IdealSpan_Threshold dB

If (Max_ideal - Min_ideal) <= IdealSpan_Threshold Then

If (Max_impairment - Max_impairment) <= ImpairmentSpan_Threshold Then 'allowed span for impairment results

DerivingPerfReq = Avg_impairment

Else:

DerivingPerfReq = -102 ' Too large span for impairment results

End If

Else

DerivingPerfReq = -103 ' Not enough input to remove outlier(s)

' removing outlier(s)

Do While M >= 3

' find outlier

Avg = Abs(SimVal(0) - Avg_ideal)

Avg_pos = 0

For i = 1 To M - 1

If Abs(SimVal(2 * i) - Avg_ideal) > Avg Then

Avg = Abs(SimVal(2 * i) - Avg_ideal)

Avg_pos = i

End If

Next i

' excluding the outlier

For i = Avg_pos To M - 2

SimVal(2 * i) = SimVal(2 * i + 2)

SimVal(2 * i + 1) = SimVal(2 * i + 3)

Next i

' M dec

M = M - 1

'updating new span and average

Min_ideal = SimVal(0)

Max_ideal = SimVal(0)

Min_impairment = SimVal(1)

Max_impairment = SimVal(1)

Min_ideal_pos = 0

Max_ideal_pos = 0

Min_impairment_pos = 1

Min_impairment_pos = 1

Avg_ideal = Min_ideal

Avg_impairment = Min_impairment

' Find the spans and average for each

For i = 1 To M - 1

If Min_ideal > SimVal(2 * i) Then

Min_ideal = SimVal(2 * i)

Min_ideal_pos = i

End If

If Max_ideal < SimVal(2 * i) Then

Max_ideal = SimVal(2 * i)

Max_ideal_pos = i

End If

If Min_impairment > SimVal(2 * i + 1) Then

Min_impairment = SimVal(2 * i + 1)

Min_impairment_pos = i

End If

If Max_impairment < SimVal(2 * i + 1) Then

Max_impairment = SimVal(2 * i + 1)

Max_impairment_pos = i

End If

Avg_ideal = Avg_ideal + SimVal(2 * i)

Avg_impairment = Avg_impairment + SimVal(2 * i + 1)

Next i

Avg_ideal = Avg_idea / M

Avg_impairment = Avg_impairment / M

' Check if ideal span is within IdealSpan_Threshold dB

If (Max_ideal - Min_ideal) <= IdealSpan_Threshold Then

If (Max_impairment - Max_impairment) <= ImpairmentSpan_Threshold Then 'allowed span for impairment results

DerivingPerfReq = Avg_impairment

Exit Do   ' stop do loop

End If

End If

Loop ' Do while loop for removing outlier

End If

End If

DerivingPerfReq = Round(DerivingPerfReq, 1)

End Function

# Reference

[1] R4-1816594 WF for test case SNR derivation procedure, Nokia