# 3GPP TSG-RAN WG4 Meeting #90                                                                  R4-1900319

# Athens, Greece, 25th Feb - 1st March, 2019

Agenda Item:	6.14.2.2.1

Source:	ZTE Corporation

Title:	Simulation results for NR PUSCH FR2

Document for:	Approval

# Introduction

In RAN4 #89, a WF was approved capturing further agreements on PUSCH demodulation [1] together with another WF on how to derive the demodulation performance requirements according to the input simulation results from different companies [2]. And in our companion paper [3] we share an Excel script for implementing this derivation rule.

In this contribution, we provide the simulation results on PUSCH FR2 with both ideal and realistic assumptions.

# Numerical results, FR2

1T2R

| PUSCH 1T2R |  |  |  |  | Ideal | Realistic |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -4.2 | -0.6 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 10.7 | 14.1 |
|  |  | MCS20 | TDLA30-75 | DMRS 1 | 12.8 | 16.1 |
|  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -4.4 | -0.9 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 9.5 | 12.8 |
|  |  | MCS20 | TDLA30-75 | DMRS 1 | 12.3 | 15.7 |
|  | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -4.1 | -0.6 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 10.1 | 13.6 |
|  |  | MCS20 | TDLA30-75 | DMRS 1 | 12.4 | 15.6 |
|  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -4 | -0.6 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 9.3 | 12.6 |
|  |  | MCS20 | TDLA30-75 | DMRS 1 | 12.4 | 15.6 |
|  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -4.2 | -0.6 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 9.7 | 13.2 |
|  |  | MCS20 | TDLA30-75 | DMRS 1 | 12.8 | 16.1 |

2T2R

| PUSCH 2T2R |  |  |  |  | Ideal | Realistic |
| --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -0.8 | 2.4 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | Na | NA |
|  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -0.9 | 2.3 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 18.5 | 21.7 |
|  | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0.6 | 2.6 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | Na | NA |
|  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0.9 | 2.4 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 17.8 | 22.3 |
|  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -0.9 | 2.4 |
|  |  | MCS16 | TDLA30-300 | DMRS 1 | 18.9 | 22.4 |

# Reference

[1] R4-1816347 WF for NR PUSCH demodulation, Nokia

[2] R4-1816594 WF for test case SNR derivation procedure, Nokia

[3] R4-1900317 Excel script for deriving NR performance requirements, ZTE