3GPP TSG-RAN WG4 Meeting #90                                                                         R4- 1900353

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	7.11.3

Source:	Intel Corporation

Title:	Discussion about Rel-16 additional MTC enhancements for non-BL UE in LTE

Document for:	Discussion

# Introduction

In [1], there is agreed WID about Additional MTC enhancements for LTE. In this contribution, we will provide our views about RRM part.

# Discussion

The objective is to specify the following set of improvements for machine-type communications, we choose the topic that is related to RAN4:

Improved DL transmission efficiency and/or UE power consumption:

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

- Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

Extreme coverage for non-BL UEs:

- Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4]

  - Enhancements to idle mode mobility

  - UE demodulation performance requirements for 2 RX antennas and full duplex FDD

  - Dual layer DL reception

  - Feedback based on CSI-RS

  - ETWS/CMAS in connected mode

Stand-alone deployment:

- Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs [RAN1, RAN2, RAN4]

  - This deployment mode should support legacy operation for legacy BL/CE UEs.

Mobility Enhancement:

- Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2]

- Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs [RAN4, RAN2]

Coexistence with NR:

- Study NR and LTE specifications to identify possible issues related to coexistence of LTE-MTC with NR [RAN4, RAN1, RAN2]

# RRM related discussion

For BL UEs, the RRM related topics may be:

  - These UEs are not limited by the measurement bandwidth and thus can perform wideband RSRP measurement for mobility purposes

  - Consider improving the DL RSRP and RSRQ measurement accuracy, through use of RSS

  - Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs

  - Study NR and LTE specifications to identify possible issues related to coexistence of LTE-MTC with NR

For non-BL UEs, the primary items discussed within RAN1 for CE mode A and B improvements are Dual layer DL reception and CSI-RS based feedback, which are demodulation part related. Topics that may be related to RRM are:

  - Enhancements to idle mode mobility

For the enhancements to idle mode mobility, RSS can provide measurement improvement, it’s designed for BL UE currently.

In RAN1 #94 meeting, there had the following agreement on use of RSS for measurement improvement [2].

Agreement From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility - RAN1 to identify the related parameters in the next RAN1 meeting

In RAN1 #94bis meeting, there had the following agreement on use of RSS for measurement improvement[3].

Agreement In using RSS for measuring RSRP of neighbour cells, the following parameters are signalled to the UE for each cell: - ce-rss-periodicity-config: RSS periodicity {160, 320, 640, 1280} ms - ce-rss-duration-config : RSS duration {8, 16, 32, 40} subframes - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames - ce-rss-powerBoost-config : RSS power offset relative to LTE CRS {0, 3, 4.8, 6} dB The parameters related in the use of RSS for measuring RSRP of cells are signalled in the SI. Send LS R1-1811934 to RAN4: - Request to evaluate the benefits and feasibility of using RSS for RSRP measurements compared to using LTE CRS - For measurement purposes, assess the benefits and feasibility of linking RSS to CRS

However, it’s not clear whether it can be used for non-BL UE as well. For non-BL UE, these UEs are not limited by the measurement bandwidth and thus can perform wideband RSRP measurement for mobility purposes. However, for a small system bandwidth, e.g., 1.4MHz, RSS may also be implemented.

It may need to identify whether RSS can be included in enhancements to idle mode mobility for non-BL UE.

Proposal 1: Identify whether RSS can be included in enhancements to idle mode mobility for non-BL UE.

For other aspects related to enhancements in idle mode mobility for non-BL UE, it needs to be clarified.

Proposal 2: clarification is needed on the WI objective “Enhancements to idle mode mobility” for non-BL UE.

# Conclusion

In this contribution the remaining issue about RLM are discussed. The following conclusion can be drawn:

Proposal 1: Identify whether RSS can be included in enhancements to idle mode mobility for non-BL UE.

Proposal 2: clarification is needed on the WI objective “Enhancements to idle mode mobility” for non-BL UE.

# References

[1] 3GPP, RP-181878, Additional MTC enhancements for LTE

[2] 3GPP, R1-1810051, Report of RAN1#94 meeting, ETSI

[3] 3GPP, Draft Report of 3GPP TSG RAN WG1 #94bis, v0.1.0