3GPP TSG-RAN WG4 Meeting #90 	R4-1900376

Athens, Greece, Feburary 25 – March 1, 2018

Agenda item:	6.12.7.5.1

Source:	Intel Corporation

Title:	On RRC based BWP switching Delay

Document for:	Discussion

# Introduction

In RAN4 #89 meeting, the issues on RRC/MAC based BWP switching delay has been identified [1]. The open problems regarding on this is listed as follows:

-	RRC based BWP switching delay for both cases:

- BWP switch to another already configured BWP;

- BWP switch to a newly configured BWP in the same RRC re-configuration massage.

-	Whether to define core delay and interruption requirements MAC-based BWP switch

In this contribution we provide our view on the delay requirements for RRC based and MAC based BWP switching.

# RRC based BWP switching delay

In current spec TS38.133, the RRC based BWP switching is defined as follows:

Following on previous discussion [2], the RRC based BWP switching delay TBWPswitchDelayRRC will include the following components:

- Time for preparing for RF/baseband reconfiguration (software overhead)

- Time for RF tuning (applying new RF/baseband parameters)

In Figure 1, the timeline for RRC based BWP switching delay (in the unit of slot) is depicted

Figure 1: Illustration of the definitions of BWP switching delay in the unit of slot

In TS38.133, the DCI-based BWP switching delay for Type 1 UE is 600µs +3 OFDM symbols (round to slot) and for Type 2 UE is 2ms +3 OFDM symbols (round to slot). The detailed requirement is as follows:

The defined DCI-based BWP switching delay includes DCI decoding time (350 µs), time for preparing for RF/baseband reconfiguration and time for RF tuning/re-tuning. Thus, we can obtain that

TBWPswitchDelayRRC = TBWPswitchDelay   – (350 µs + 3 OFDM symbol)

The calculated value for TBWPswitchDelayRRC is listed in Table 1

Table 1: TBWPswitchDelayRRC in µs

| SCS | 350 µs (DCI decoding delay)+ 3 OFDM symbol | TBWPswitchDelay for Type 1 delay in µs | TBWPswitchDelay for Type 2 delay in µs | TBWPswitchDelayRRC for Type 1 delay in µs | TBWPswitchDelayRRC for Type 2 delay in µs |
| --- | --- | --- | --- | --- | --- |
| 15kHz | 564.29 | 1000 | 3000 | 435.71 | 2435.71 |
| 30kHz | 457.14 | 1000 | 2500 | 542.86 | 2042.86 |
| 60kHz | 403.57 | 750 | 2250 | 346.43 | 1846.43 |
| 120kHz | 376.79 | 750 | 2125 | 373.21 | 1748.21 |

Finally, TBWPswitchDelayRRC values in above table are needed to be round to number of slot shown in Table 2. Thus, it is proposed:

Proposal 1: Depending on UE capability, UE shall finish RRC based BWP switch within the time duration TBWPswitchDelayRRC defined in Table 2.

Table 2: RRC based BWP switching delay

| SCS | Type 1 delay in Slot | Type 2 delay in Slot |
| --- | --- | --- |
| 15kHz | 1 | 3 |
| 30kHz | 2 | 5 |
| 60kHz | 2 | 8 |
| 120kHz | 3 | 14 |

# MAC based BWP switching delay

MAC based BWP switching is similar to timer-base based BWP switching, where if the BWP inactivity timer expires (no DL scheduling for the UE for a while on the current BWP) or if the UE has to perform an UL and due to various reasons, the UL is on another BWP (initial BWP), the UE autonomously switches to the other BWP where the UE has the information of the centre-frequency, BW as well as the SCS.

Since timer-based BWP switching has been defined in TS38.133, MAC based BWP switching delay can share the same requirement as timer-based BWP switching. Thus, it is proposed,

Proposal 2: MAC based BWP switching delay shall have the same requirement as timer-based BWP switching.

# Other remaining issues on BWP switching delay and Interruption due to BWP switching

Furthermore, there are some other reaming issues on BWP switching delay: 1) the uncertainty of UL BWP switching delay due to the effect of timing advance; 2) parameter list which cause interruption other than SCS when BWP switches.

For the UL BWP switching delay uncertainty due to TA impact, there is related description in TS 38.214:

Following the same mechanism as TS 38.214, it is proposed that for UL BWP switching, the value of TA +TA offset is added to the delay requirement for DL BWP switching.  However, there is mismatch between 38.214 and 38.133 on d2,2 since in RAN4 spec BWP switching delay is defined in slot while in RAN1 spec Tproc,2  is calculated in time. For clarification, we propose defining UL BWP which include the impact of TA in TS 38.133.

One option is just to count the entire TA value as the difference between UL and DL BWP switching delay. Another option is to use this margin to compensate a part of TA value and the remaining part of TA (if it is positive) is the difference between UL and DL BWP switching delay. Since the actual time for BWP switching is 600µs + 3 OFDM symbols for Type 1 delay and 2000µs + 3 OFDM symbols for Type 2 delay, there are some time margin in the existing delay requirement in TS38.133. The margin is defined as X shown in Table 3. Thus, it is proposed:

Proposal 3: When BWP switch occurs in UL and UE operates on FDD mode, after UE receives BWP switching request at DL slot n (for DCI-based BWP switch), or the BWP-inactivity timer expires at DL slot n (for timer-based BWP switch), UE shall be able to transmit PUSCH (for UL active BWP switch) on the new BWP no later than at UL slot n +TBWPswitchDelay +Y, where TBWPswitchDelay defined in Table 8.6.2-1 in TS38.133.

There are two options for values of Y:

Option 1:  Y equals to the smallest number of slots (depending on SCS) that longer than (TA + TA offset);

Option 2:  Y is calculated by taking into account the existing margin in BWP switching delay.

- Y equals to zero if  (TA + TA offset) ≤ X;

- Y equals to the smallest number of slots (depending on SCS) that longer than (TA + TA offset –X) if (TA + TA offset) > X.

X is defined in Table 3 shown below.

Table 3.  X values for UL BWP switching delay

| SCS | For Type 1 Delay UE (µs) | For Type 2 Delay UE (µs) |
| --- | --- | --- |
| 15kHz | 185.95 | 784.85 |
| 30kHz | 292.96 | 392.48 |
| 60kHz | 96.48 | 196.24 |
| 120kHz | 123.24 | 98.12 |

Another issue is the parameter list which causes interruption other than SCS when BWP switches. Currently, the parameters listed in Table 8.2.1.2.7-2 in TS 38.133 include “maxNrofCodeWordsScheduledByDCI”. For Rel-15 the requirements are defined for 2RX and 4RX cases only. So technically speaking, in Rel-15, the maximum number of layers is 4.

For NR two MIMO codewords are used only in case a number of PDSCH MIMO layers is > 4 based on TS 38.211 (see below). So, if UE reports that it has max 4 MIMO layers, then maxNrofCodeWordsScheduledByDCI can take value = 1 only and it will not change for all BWPs on this carrier. Thus, we should omit this parameter from the list causing interruption other than SCS when BWP switches.

Proposal 4: Remove the parameter maxNrofCodeWordsScheduledByDCI from Table 8.2.1.2.7-2 in TS38.133, in which the parameters causing interruption other than SCS are listed.

# Conclusion

In this contribution, some issues on BWP switching delay and interruption are discussed and the proposals are listed as follows,

Proposal 1: Depending on UE capability, UE shall finish RRC based BWP switch within the time duration TBWPswitchDelayRRC defined in Table 2.

Table 2: RRC based BWP switching delay

| SCS | Type 1 delay in Slot | Type 2 delay in Slot |
| --- | --- | --- |
| 15kHz | 1 | 3 |
| 30kHz | 2 | 5 |
| 60kHz | 2 | 8 |
| 120kHz | 3 | 14 |

Proposal 2: MAC based BWP switching delay shall have the same requirement as timer-based BWP switching.

Proposal 3: When BWP switch occurs in UL and UE operates on FDD mode, after UE receives BWP switching request at DL slot n (for DCI-based BWP switch), or the BWP-inactivity timer expires at DL slot n (for timer-based BWP switch), UE shall be able to transmit PUSCH (for UL active BWP switch) on the new BWP no later than at UL slot n +TBWPswitchDelay +Y, where TBWPswitchDelay defined in Table 8.6.2-1 in TS38.133.

There are two options for values of Y:

Option 1:  Y equals to the smallest number of slots (depending on SCS) that longer than (TA + TA offset);

Option 2:  Y is calculated by taking into account the existing margin in BWP switching delay.

- Y equals to zero if  (TA + TA offset) < X;

- Y equals to the smallest number of slots (depending on SCS) that longer than (TA + TA offset –X) if (TA + TA offset) ≥ X.

X is defined in Table 3 shown below.

Table 3.  X values for UL BWP switching delay

| SCS | For Type 1 Delay UE (µs) | For Type 2 Delay UE (µs) |
| --- | --- | --- |
| 15kHz | 185.95 | 784.85 |
| 30kHz | 292.96 | 392.48 |
| 60kHz | 96.48 | 196.24 |
| 120kHz | 123.24 | 98.12 |

Proposal 4: Remove the parameter maxNrofCodeWordsScheduledByDCI from Table 8.2.1.2.7-2 in TS38.133, in which the parameters causing interruption other than SCS are listed.

# References

[1] R4-1816689, “WF for RRC-based BWP switch”, Nokia, Nokia Shanghai Bell, Mediatek, RAN4#89

[2] R4-1809875, “Further discussion on BWP switching Delay”, Intel, RAN4#88

[3] R4-1803283, “LS on BWP switching delay”, Intel, RAN4#86