3GPP TSG-RAN WG4 Meeting #90	<TDoc#>

Athens, Greece, 25th Feb – 1st Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 38.817-01 | CR |  | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CA MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 14-Feb-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CA MPR 38.101-2 is not well understood and needs clarifications. Proper place is TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CA MPR derivation for FR2 is slightly different than in FR2 and it seems usefull to add explantions for TR what parameters in tables in TS 38.101-2 mean |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.2A Transmit power for CA

### 7.2A.1 Void

### 7.2A.2 Maximum power reduction for CA

FR2 enables very wideband aggregated channel bandwidths CA. RAN4 agreed to limit contiguous UL CA up to 800 MHz and 8 CC.  Non-contiguous intra-band UL CA was deemed out of Rel-15 scope but non-contiguous DL CA is defined.

Maximum power reduction is a function of modulation but MPR for CA is also a function of cumulative aggregated channel bandwidth instead of channel BW. Cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs. This means CA MPR is applicable when UE is configured for CA either in UL or in DL. Reason for this was the limittation in UE TX because of gain droop over the large bandwidth. The Figure 7.2A.2-1 shows the principle.

Figure 7.2A.2-1 Gain limitation in TX lineup for large cumulative aggregated channel bandwidth.

If the device and TX lineup is sized for meeting full power with 400 MHz channel BW, it will suffer power droop of the size of ∆G when wider bandwidth is configured. The assumption here is that UE can not change signal path BW between TX and RX since the very short TX to RX switching time allovance.

It should be noted that definition of MPR for CA also defines applicable DL cumulative aggregated bandwidth. For Rel-15 specifications, support is defined up to 1400 MHz bandwidth in Table 6.2A.2.2-1 of TS 38.101-2. The configurations in section 5.5A do not define what is the maximum cumulative aggregated bandwidth applicable for a specific configuration but UE declares its maximum cumulative aggregated bandwidth with IE intraBandFreqSeparation for NC CA configurations.

intraBandFreqSeparation capability exists seprately for both UL and DL but for UL in Rel-15 is not used since Rel-15 does not support NC UL CA.