3GPP TSG RAN WG4 Meeting #90	R4-1900452

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda item:	9.16.1

Source:	SoftBank Corp.

Title:	On protection requirement in Japan for n41 30MHz CBW

Document for:	Approval

1　Introduction

The WI of adding 30MHz CBW to n41 was approved in RAN#82 [1]. This paper addresses a domestic regulation around n41 for Japan and some consideration on NR specific issues. This paper also proposes a simulation campaign to determine appropriate operational methods/conditions.

2　Background

For 2.5GHz TDD in Japan, there has been a requirement for B41 UE to protect a satellite service as -25dBm/MHz @ 2530-2535MHz and -30dBm/MHz @ 2505-2530MHz[2]. 

In LTE, thanks to CC CBW up to 20MHz, this requirement can be satisfied with UE general SEM alone under a specific CC arrangement, i.e. 2545-2555MHz(10MHz) and 2555-2575MHz(20MHz) for 30MHz occupied by Opeartor A and it was agreed that there was no specific description in TS36.101. For 30MHz DL operation, intra-band contiguous CA (1UL/2DL) is an acceptable solution. The situation is depicted in Figure 1 below. (Note that in Operator B spectrum holding (2595-2645MHz), no issue would be seen on protection of the satellite service since -30dBm/MHz can always be guaranteed by SEM for the range below 2535MHz, even if 50MHz is used as a single CC.)

Figure 1: Current 10MHz/20MHz operation in LTE

For PC2, since a UE can support the same SEM requirements of PC3 UE, it won’t bear any problem as long as the protection is kept only with the SEM.

But in NR, we need to seek for a scheme to handle 30MHz CBW conforming to the protection requirement. As depicted in Figure 2, general SEM of 30MHz CBW apparently violates the requirements and huge (17dB based on SEM) backoff is expected. Practically speaking, even if NS/A-MPR is introduced for 30MHz CBW, some lower RBs would not be easy to use.

Figure 2: Protection requirement and 30MHz CBW UE SEM

3　Consideration

3.1   Relation between UE CBW and configured BWP

Unlike LTE, we need to take into account flexibility on CBWs and relation with BWPs.

Firstly, it should be noted that RAN4 RF requirements are thoroughly based upon (UE) CBW while the current RRM signalling simply indicates BS CBW equivalent and a set of BWP configurations in the numbers/positions of RBs. UE CBW is somewhat left ambiguous in this context and the lack of well-defined UE CBW might be a risk against regulatory requirements.

This aspect is being discussed in RAN4 under the agenda of “SCS restriction with BWP operation” such as [2] and in RAN#82 a RAN2 CR was agreed to add signalling to indicate UE-specific CBW in RRC_Connected state [3]. But it seems that we may need to discuss further on how to handle the issue in detail and the following proposals might be adjusted by decisions made for the argument.

[Observation-1] Relation between UE RF requirements (based on UE CBW) and configured BWP still seems outstanding in RAN4 and some of the proposals in this paper might be revisited depending on agreements on the relation.

Given that any BWPs within a UE CBW shall satisfy SEM requirements relevant to the UE CBW, specifying UE CBW placement for the operation will imply that ant BWP configuration within the UE CBW should be permitted. In this sense, in the following discussion, UE CBW is solely considered and any BWPs within the UE CBW shall be permitted to be configured.

[Proposal-1] Given that any BWPs within a UE CBW shall satisfy SEM requirements relevant to the UE CBW, UE CBW placement is considered and any BWPs within the UE CBW shall be permitted to be configured.

3.2 	For UEs not supporting 30MHz CBW

It has been anticipated in RAN4 that a “legacy” UE be operated in a newly defined CBW, utilizing a UE CBW smaller than the new CBW. In this case, a gNB is requested to configure a “rational” BWP confined within a UE CBW supported. For n41, 10, 15 and 20MHz CBW shall be supported from REL-15 then, depending on the outcome of “SCS restriction” discussion, one of these would be used.

For an n41 UE without 30MHz support, a scheme similar to LTE (Figure 1) can be applied: operation (BWP) shall be confined either in the lower 10MHz or the upper 20MHz. The UE can be operated based only on general SEM so no additional restriction is required. Concerning 10MHz, the UE CBW can be placed higher in frequency than the lower 10MHz: this means 10MHz UE CBW can be placed anywhere within 30MHz without violating protection requirement.

[Proposal-2] For a legacy UE not supporting 30MHz CBW, operation is limited to any 10MHz CBW or the upper 20MHz CBW placements without additional constraint (i.e. based on general SEM).

Since the scenario above relies on general SEM and a PC2 UE can commit the same SEM requirement as per a PC3 UE, it is likely that the PC2 UE can be operated in the same manner as a PC3 UE.

[Proposal-3] For a legacy PC2 UE not supporting 30MHz CBW, the same limitation can be applied as per a PC3 UE.

Note that clarification is needed on the precise range of “lower 10MHz” and “upper 20MHz” in the context of 30MHz single CC. We will prepare a contribution to propose the specific ranges. Current idea is ranges to fit within a single 10/20MHz CC placed in 2545-2555/2555-2575MHz respectively.

3.3 	For UEs supporting 30MHz CBW

A UE supporting 30MHz CBW could also support NS/A-MPR relevant to the CBW. Then there seems no problem to enforce necessary backoff with a new NS. However, there are a couple of issues to be addressed.

3.3.1   Backward compatibility issue for NS addition

For introducing a new NS, an issue is in a backward compatibility on “unknown NS”: RAN2 in [4] has already defined that UE shall consider a cell as barred if an unknown NS is broadcasted in the cell in yellow marker below. (Green marker indicates a case where an NS is supported.)

TS38.331 (v15.4.0) ##### 5.2.2.4.2 Actions upon reception of the SIB1 Upon receiving the SIB1 the UE shall: 1> store the acquired SIB1; 1> if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN: 2> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN; 1> if in RRC_CONNECTED while T311 is not running: 2> disregard the frequencyBandList, if received, while in RRC_CONNECTED; 2> forward the cellIdentity to upper layers; 2> forward the trackingAreaCode to upper layers; 2> apply the configuration included in the servingCellConfigCommonSIB; 1> else: 2> if the UE supports one or more of the frequency bands indicated in the frequencyBandList for downlink and one or more of the frequency bands indicated in the frequencyBandList for uplink or one or more of the frequency bands indicated in the frequencyBandList for supplementary uplink, if configured, and they are not downlink only bands, and 2> if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList of FrequencyInfoUL-SIB for FDD or of FrequencyInfoDL-SIB for TDD for the frequency band selected by the UE (for the downlink and uplink or supplementary uplink, if configured), and 2> if the UE supports the bandwidth of the initial uplink BWP and of the initial downlink BWPs indicated in the locationAndBandwidth fields: 3> forward the cellIdentity to upper layers; 3> forward the trackingAreaCode to upper layers; 3> if in RRC_INACTIVE and the forwarded trackingAreaCode does not trigger message transmission by upper layers: 4> if the serving cell does not belong to the configured ran-NotificationAreaInfo: 5> initiate an RNA update as specified in 5.3.13.8; 3> forward the ims-EmergencySupport to upper layers, if present; 3> forward the eCallOverIMS-Support to upper layers, if present; 3> apply the configuration included in the servingCellConfigCommon; 3> apply the specified PCCH configuration defined in 9.1.1.3; 3> if the UE has a stored valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with sub-clause 5.2.2.1: 4> use the stored version of the required SIB; 3> if the UE has not stored a valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s), in accordance with sub-clause 5.2.2.1: 4> for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting: 5> acquire the SI message(s) as defined in sub-clause 5.2.2.3.2; 4> for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting: 5> trigger a request to acquire the SI message(s) as defined in sub-clause 5.2.2.3.3; 3> apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList; 3> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList: 4> apply the additionalPmax; 3> else: 4> apply the p-Max; 2> else: 3> consider the cell as barred in accordance with TS 38.304 [20]; and 3> perform barring as if intraFreqReselection is set to notAllowed; Editor′s Note: To be further updated when content of the SIB1 has been completed.

From the description above, the use of a new NS/A-MPR will block UEs that do not recognize the added NS.

[Observation-2] The addition of a new NS/A-MPR will bar legacy UEs that do not recognize the added NS.

Following the logic in LTE cases, it seems that NS addition is almost prohibitive and RB restriction is most likely in principle. On the other hand, it would be too restrictive to use RB restriction scheme as we have already supported various modulation schemes that might have different backoff characters. Anyway, we need to discuss internally with simulation results and will propose how to define 30MHz CBW operation.

3.3.2	BWP limitation for TDD

NR can offer flexible BW setting through BWP configuration. For TDD bands, however, the current RAN1/2 specs have put a limitation to use the same center frequency (Fc) of UL BWP and DL BWP [5].

TS38.213 (v15.4.0), sec 12 “Bandwidth part operation” [Unnecessary parts skipped] For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by bwp-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by bwp-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the bwp-Id of the DL BWP is same as the bwp-Id of the UL BWP. [Unnecessary parts skipped]

This poses an unfavourable limitation for trying to control UL unwanted emission: for example, (1) to confine UL RBs with BWP, some limitation should be accepted on the configuration of DL BWP to be paired. Moreover, (2) if a gNB configures a DL BWP for full 30MHz operation, the Fc of UL BWP shall also be assumed in the center of the 30MHz (2560MHz) regardless of the bandwidth of UL BWP or actual RB allocation. (3) To utilize RBs far from a victim to mitigate pain, we need to configure an UL BWP of full RBs (or closer). The examples of the limitation are depicted in Figure 3.

Figure 3: Limitations of BWP in TDD mode

In the long run, because of this limitation, we should assume Fc=2560MHz in UL BWP (the center of 30MHz of interest, more precisely 2560.005MHz due to 15k channel raster...) to make use of full 30MHz bandwidth in DL and then UL RB restriction using BWP, such as asymmetric DL/UL BWP configuration, may not always work effectively in TDD case.

The setting of Fc=2560MHz with UL 30MHz BWP will exhibit the worst-case scenario for IMD of edge 1RB allocation in 30MHz CBW so it is expected that the scenario cover all the possible cases. Thus it is proposed to run simulation this case.

[Proposal-4] Simulation should be conducted for 30MHz CBW with Fc=2560MHz to permit 30MHz DL configuration with assumptions in section 4.

3.4  　Practical CBW configurations

As discussed above, there are several limitations imposed so practical scenarios would be limited to:

1) For legacy UEs, any 10MHz (CBW-1) and/or upper 20MHz (CBW-2) schemes are needed where no RB restriction is required.

2) For new UEs, 30MHz (CBW-3) is likely under some RB restrictions to make use of the expanded UE CBW.

3) Any 10MHz/upper 20MHz scheme without RB restriction (CBW-1/2) can also be used for the new UEs.

Figure 4: Assumption on CBW arrangements

Then, to simplify the scheme and to reduce the burden, it is proposed to assume 3 CBWs and RB restriction is only applied/evaluated to 30MHz CBW (CBW-3). The proposals are summarized in Table 1.

Table 1: Summary on assumption of CBW configurations and usage

|  | CBW-1 (Any 10MHz) | CBW-2 (Upper 20MHz) | CBW-3 (Full 30MHz) |
| --- | --- | --- | --- |
| UE not supporting 30MHz | Yes | Yes | No |
| UE supporting 30MHz | Yes | Yes | Yes |
| RB Restriction/A-MPR | No | No | Yes |

We might revisit the assumption if simulation results or further inspection show a serious drawback.

4　Simulation conditions

As discussed above, we would like to ask for simulation results on:

Tx CBW=30MHz, 2545 – 2575MHz (Fc=2560MHz) Protection requirements: -25dBm/MHz in 2530 – 2535MHz, -30dBm/MHz in 2505 – 2530MHz Results: Charts of Necessary backoff required vs. RBstart and Lcrb Power class: both PC2 and PC3 considered Modulation orders: DFT-s-OFDM: π/2-BPSK, QPSK, 16QAM, 64QAM, 256QAM, CP-OFDM: QPSK, 16QAM, 64QAM, 256QAM SCS: 15kHz, 30kHz and 60kHz (we guess 15kHz is the worst case due to narrower guard band and the result can be converted to 30kHz/60kHz case)

For setting a PA operating point, conventional NR conditions (I/Q=LO=28dBc, Counter IM3=60dBc) are assumed.

5　Conclusion

This paper addresses 30MHz CBW operation of n41 in Japan. Some proposals are made including simulation conditions. Comments are welcome on the observations and proposals in this paper, UE simulation results not to mention.

Proposals are listed below:

[Proposal-1] Given that any BWPs within a UE CBW shall satisfy SEM requirements relevant to the UE CBW, UE CBW placement is considered and any BWPs within the UE CBW shall be permitted to be configured.

[Proposal-2] For a legacy UE not supporting 30MHz CBW, operation is limited to any 10MHz CBW or the upper 20MHz CBW placements without additional constraint (i.e. based on general SEM).

[Proposal-3] For a legacy PC2 UE not supporting 30MHz CBW, the same limitation can be applied as per a PC3 UE.

[Proposal-4] Simulation should be conducted for 30MHz CBW with Fc=2560MHz to permit 30MHz DL configuration with assumptions in section 4.

Reference

RP-182828		New WI proposal: Addition of 30MHz channel bandwidth for NR band n41, KDDI

R4-1814147		WF on UE behavior and network configuration without SCS restriction, Qualcomm, Huawei

RP-182896	   UE specific channel bandwidth signaling (Company CR to TS38.331), Qualcomm

TS38.331 (v15.4.0)

TS38.213 (v15.4.0)