3GPP TSG-RAN4#90 Meeting  		R4-1900473

Athens, Greece, 25th Feb – 1st Mar, 2019

Title:	Discussion on TCI state switching requirements

Source:	CATT

Agenda Item:	6.12.7.6

Document for:	Discussion

- Introduction

In RAN4 #89 meeting, RAN4 had some initial discussion on TCI state switching requirements, and a WF in [1] on TCI state switching delay was agreed, which is captured as follows:

Switching Delay - gNodeB can indicate the UE to switch its active TCI state - Both the old and new active TCI states will be from the set of RRC configured TCI states - Open items - Define delays associated with an active TCI state switch - Are interruptions needed on other active cells - If needed, duration of these interruptions - Interested companies to provide inputs to above open items in RAN4#90

In this contribution, we discuss the open issues for TCI state switching requirements.

- Discussion

The TCI state is defined to configure a quasi-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH. And the UE can be configured with number of TCI-state configurations within the higher layer parameter. UE is required to track only active TCI states, and gNB may change the transmission beam direction and inform UE the beam switching state by configuring the TCI states via RRC reconfiguration, MAC-CE or DCI. From RRM aspect, the TCI state switching delay and the corresponding interruption requirements shall be defined due to the RRC processing or MAC-CE/DCI decoding operation.

Based on RAN1’s agreements, TCI state switching can be configured by the following operation:

- RRC reconfiguration based, the active TCI state can be changed via RRC reconfiguration signalling;

- MAC-CE based, the UE is configured with a list of TCI state configurations, then UE will be activated/deactivated one of TCI state configuration in the list by decoding MAC-CE signalling;

- DCI based, the UE can be configured with a list of up to M TCI-state configurations by RRC configuration, where M depends on the UE capability. Then, the UE is received an activation command used to map up to 8 TCI states by MCE-CE activation, and DCI will indicate dynamically to UE one of the activated TCI states.

For RRC reconfiguration-based TCI state switching, the TCI state switching delay should be the RRC processing time, when UE is received RRC signalling including TCI state configuration at slot n, UE shall complete the TCI state switching no later than slot n+TRRC_processing, where the RRC processing time could be 15ms-20ms. During the RRC processing time, the interruption due to RRC reconfiguration operation is expected, the interruption time could refer to the interruption requirements due to RRC reconfiguration defined in section 8.2 TS.38.133.

Proposal 1: For RRC-based TCI state switching, the TCI state switching delay should be the RRC reconfiguration processing time, TRRC_processing. And an interruption due to RRC reconfiguration operation is expected.

For MAC-CE based TCI state switching, the UE is request to change the active TCI state by receiving MACE-CE command including TCI-state information. During this procedure, UE needs some time to decode MAC-CE, as illustrated in figure 1. As we discussed in SCell activation section, 3ms was specified in RAN1 due to the MAC-CE decoding and the preparation time. Since UE is not required to do RF retuning, thus, no interruption requirement should be defined.

Proposal 2: For MAC-CE-based TCI state switching, the TCI state switching delay should be 3ms, which is the MAC-CE decoding time including the related preparation time.

For DCI-based TCI state switching, UE can be configured with a list of up to M TCI-state configurations by RRC configuration, where M depends on the UE capability. Then, the UE is received an activation command used to map up to 8 TCI states by MCE-CE activation, and DCI will indicate dynamically to UE one of the activated TCI states. If the target TCI state is one of up to 8 TCI states activated by MAC-CE, then the switching delay should be the DCI decoding time. As we discussed for BWP switching delay in [2], for DCI decoding, it takes about 350us for UE complete PDCCH decoding and AGC adjustment. Thus, we propose to 0.5ms can be defined for TCI state switching.

Proposal 3: For DCI-based TCI state switching, if the target TCI state is one of TCI states activated by MAC-CE, the TCI state switching delay should be 0.5ms, which is the DCI decoding time.

If the target TCI state is one of up to M TCI state configured by RRC configuration, but not one of TCI states activated by MAC-CE, in this case, UE needs additional time for MAC-CE decoding used to activate a list of TCI states including the target TCI state configuration.

Proposal 4: For DCI-based TCI state switching, if the target TCI state is one of up to M TCI state configured by RRC configuration, but not one of TCI states activated by MAC-CE, the TCI state switching delay should be 3.5ms, which is the MAC-CE decoding time including the related preparation time and the DCI decoding time.

- Conclusion

In this contribution, we discuss the open issues on TCI state switching requirements, and provide the proposals as follows:

Proposal 1: For RRC-based TCI state switching, the TCI state switching delay should be the RRC reconfiguration processing time, TRRC_processing. And an interruption due to RRC reconfiguration operation is expected.

Proposal 2: For MAC-CE-based TCI state switching, the TCI state switching delay should be 3ms, which is the MAC-CE decoding time including the related preparation time.

Proposal 3: For DCI-based TCI state switching, if the target TCI state is one of TCI states activated by MAC-CE, the TCI state switching delay should be 0.5ms, which is the DCI decoding time.

Proposal 4: For DCI-based TCI state switching, if the target TCI state is one of up to M TCI state configured by RRC configuration, but not one of TCI states activated by MAC-CE, the TCI state switching delay should be 3.5ms, which is the MAC-CE decoding time including the related preparation time and the DCI decoding time.

- Reference

[1]	R4-1816678,	WF on delay on TCI state switch and Scell activation with multiple TCI states, Qualcomm

[2]	R4-1808724,	On BWP switching Delay, Intel