3GPP TSG-RAN WG4 Meeting #90                 	 R4-1900523

Athens, Greece, February 25th– March 1st, 2019

Agenda Item:	6.13.3.2

Source:	MediaTek Inc.

Title:	Array gain in RSRP reporting

Document for:		Discussion

- Introduction

In this paper, we discuss the array gain difference in RSRP reporting, which comes from different number of Rx antennas. For an example, L1-RSRP is measured through fine beam formed through 8 antennas and L3-RSRP is measured through rough beam form through 1 single antenna.

- Discussion

We use a very simple example to facilitate the discussion. The example is described as following.

- UE has 2 antennas: ANT#1 and ANT#2

  - On ANT#1, the received signal is h1 and thermal noise n1 with variance σ12.

  - On ANT#2, the received signal is h2 and thermal noise n2 with variance σ22.

    - For simplicity, we assume h12=h22=1 and the noise variance σ12=σ22=1

- When UE uses ANT#1 to form the a rough beam.

  - The signal power Prough is h12=1

  - The noise power is σ12=1

  - The SNRrough is h12σ12=1

- When UE uses both antennas to form a fine beam

  - Assuming ideal phase shifting coefficients h1* on ANT#1 and h2* on ANT#2, the signal power Pfine is h1h1*+h2h2*2=4

  - The noise power is σ12+σ22=2

  - The SNRfine is h1h1*+h2h2*2σ12+σ22=42

From above example, the power gain of fine beam over rough beam is 4. However, the SNR gain is only 2. This example can be easily generalized to the cases with different number of antennas for rough beam and fine beam. In general, if the number of antennas for find beam is N1 and the number of antennas for rough beam is N2, we can observe that the beamforming gain on signal power is N1N22, while the actual SNR gain is N1N2. This mismatch between power gain and SNR gain will increase when we have more antennas used for fine beam over rough beam.

If more practical scenario is considered, e.g., h12≠h22 or non-ideal phase shifting coefficients, the mismatch could get smaller, but would never disappear. The gain on the signal power level is contributed by two factors. One is the array gain which comes from the number of Rx antennas. The more Rx antennas are used, the total power received is of course higher. The other one is coherent combining gain, which is achieved by properly selecting the phase shifting coefficients. However, the array gain will not contribute to the SNR, because the more antennas are used for combining, the higher total noise power comes at the same time.

Observation 1: When using N1 Rx antennas to form the fine beam for measuring L1-RSRP and N2 Rx antennas to form the rough beam for measuring L3-RSRP, ideally the L1-RSRP will be (N1/N2)2 times higher than L3-RSRP, but in SNR only N1/N2 times higher.

This mismatch may create some problem in the real field.

- As we know, SNR is the metric which reflects the decoding performance. For an examples, when decoding PDSCH, using 2 antennas for decoding will roughly achieve 3dB gains than using 1 antenna. But in RSRP reporting, using 2 antenna for combining could achieve 6dB higher RSRP than using 1 antenna.

- When UE is measuring both SS-RSRP and SS-SINR, the difference between this 2 quantities will depend on the number of Rx antenna used by UE, especially in FR2 which is a noise-limited environment.

In our opinion, RAN4 should first confirm whether the reported RSRP value should include the array gain or not.

Proposal 1: RAN4 to confirm the reported RSRP value should include the array gain or not.

- Summary

In this paper, we discuss the issue of difference in reported RSRP value due to different number of Rx antennas. We have the following observation and proposal

Observation 1: When using N1 Rx antennas to form the fine beam for measuring L1-RSRP and N2 Rx antennas to form the rough beam for measuring L3-RSRP, ideally the L1-RSRP will be (N1/N2)2 times higher than L3-RSRP, but in SNR only N1/N2 times higher.

Proposal 1: RAN4 to confirm the reported RSRP value should include the array gain or not.