3GPP TSG-RAN WG4 Meeting #90						R4- 1900559

Athens, Greece, 25th February - 1st March, 2019

Source:		NTT DOCOMO, INC.,

Title:	Clarification on multiband relaxation requirement

Agenda Item:	6.7.8.2.

Document for:	Discussion

Introduction

RAN4#89 meeting defined multiband relaxation requirements for peak and spherical EIRP. RAN5#4-5G-NR Adhoc, however, sent an LS for clarifying RAN4 assumption [1] to develop their test specification. This contribution addresses clarification of RAN4’s common understanding of the multiband relaxation requirements. Based on the discussion result, an LS reply [2] should be sent to RAN5.

Discussion

Base line assumption of multi-band relaxation requirement

This section provides RAN5 assumptions (questions) and discuss RAN4 understanding on them. Below is RAN5 questions described in [1] (attached excel file). There are some assumptions of RAN5 in that. This contribution aims at clarifying RAN4 views on those aspects.

For assumption 1, in the RAN4#89, there was no discussion on whether UE declares relaxation value for each band or not. ΔMBP/S is calculated between measurement results and peak or spherical requirement. In our understanding, it depends on RAN5 test spec on whether UE declares ΔMBP/S for each band or not.

For assumption 2, and 4, whether bands are supported or not itself depends on UE capability. Hence, in order to consider a UE as “PASS”, at least all the relevant requirements for bands whose capabilities are signalled by the UE. In the cases of Example 1 and 2 (above RAN5 LS on assumption 4), UE is considered “FAIL” since if capability of 4-bands are set. Note that in the first place, if UE does not set capability of a certain band, that band cannot be tested. Then, it is passed that capablity of 3-bands (except for n260) are set even though the UE has designed as including n260 (of course, ΣMBP/S shall be re-calculated as supporting 3-band case).

Based on the understanding, below proposal is made with regard to assumption 1, 2 and, 4 of above RAN5 LS (same understanding as RAN5).

Proposal 1: ΔMBP/S is defined based on measured value, the pass/fail judgment is not done at each supporting band but for a UE (do not consider which band is pass or fail).

Proposal 2: ΣMBP/S is calculated based on the ΔMBP/S, the pass/fail judgment is done whether ΣMBP/S exceed requirement value or not.

For assumption 3, margin for EIRP ΔMBP/S has not been discussed in RAN4.The margin against requirement in a band should not be included in calculation of ΣMBP/S. If it allows, it means there is no limit for a band in effect and it is large impact on NW design. The requirement does not work as minimum requriment. There is not such kind of background in the past RAN4 discussion, so, proposal 3 is made as follows.

Proposal 3: Even if a band has margin against requirement, the margin is not taken into account on ΣMBP/S calculation.

For assumption 5, one single ΔMBP/S value is developed for each band. The worst value shall be adopted among all the defined test points for each of the bands since regardless of the CHBW, SCS and position of the CHBW etc., the UE has to fulfil all the corresponding requirements. Then, these are used for ΣMBP/S Thus, our understanding is that RAN5 assumption is correct.

Proposal 4: The worst value among the test points is selected as ΔMBP/S for the band.

Conclusion

In this contribution, some clarification for multi-band relaxation requirement is discussed based on the assumption of [1]. Bellows are proposed as RAN4 common understanding. Based on the agreement, LS reply [2] should be sent to RAN5.

Proposal 1: ΔMBP/S is defined based on measured value, the pass/fail judgment is not done at each supporting band but for a UE (do not consider which band is pass or fail).

Proposal 2: ΣMBP/S is calculated based on the ΔMBP/S, the pass/fail judgment is done whether ΣMBP/S exceed requirement value or not.

Proposal 3: Even if a band has margin against requirement, the margin is not taken into account on ΣMBP/S calculation.

Proposal 4: The worst value among the test points is selected as ΔMBP/S for the band.

References

[1]    R5-190729, LS on Multi-band relaxation, TSG RAN WG5, RAN5#4-5G-NR Adhoc,

[2]    R4-19xxxxx, LS reply on Multi-band relaxation, RAN4, RAN4#90,