3GPP TSG-WG RAN4 Meeting #90	R4-1900592

# Athens, Greece, 25st Feb – 1st Mar, 2019

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.716-03-01: CA_5-46-66

Agenda Item: 			7.3.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 3 bands DL with 1 band UL for the CA bands CA_5-46-66.

- DL_5A-46E-66A_BCS0

- DL_5A-46A-66A-66A_BCS0

- DL_5A-46C-66A-66A_BCS0

- DL_5A-46D-66A-66A_BCS0

- DL_5A-46E-66A-66A_BCS0

**************************** Start of changes ********************************************

## 5.X	CA_5-46-66

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 3-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_5A-46E-66A | 5 |  |  | Yes | Yes |  |  | 110 | 0 |
|  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-46A-66A-66A | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  | 46 |  |  |  |  |  | Yes |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-46C-66A-66A | 5 |  |  | Yes | Yes |  |  | 90 | 0 |
|  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-46D-66A-66A | 5 |  |  | Yes | Yes |  |  | 110 | 0 |
|  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-46E-66A-66A | 5 |  |  | Yes | Yes |  |  | 130 | 0 |
|  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.X.2	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of CA_5-46-66 specified in TS36.101 can be applied to CA_5-46-66-66.

Table 5.1.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_5-46-66, CA_5-46-66-66 | 5 | 0.3 |
|  | 66 | 0.3 |

Table 5.1.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_5-46-66, CA_5-46-66-66 | 5 | 0 |
|  | 66 | 0 |

### 5.X.3	REFSENS requirements

REFSENS requirements of CA_5A-46A-66A defined in TS36.101 can be applied for CA_5A-46E-66A, CA_5A-46A-66A-66A, CA_5A-46C-66A-66A, CA_5A-46D-66A-66A, and CA_5A-46E-66A-66A.

Table 5.X.3-1: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_5A-46E-66A, CA_5A-46A-66A-66A, CA_5A-46C-66A-66A, CA_5A-46D-66A-66A, CA_5A-46E-66A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |

**************************** End of changes ********************************************