3GPP TSG-RAN WG4 Meeting #90	R4-1900711

# Athens, Greece, 25th Feb. – 1st Mar., 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Darft CR for Measurement Gap Sharing in NE-DC (Section 9.1.2.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The mechanism of mesuarement gap sharing for NE-DC in Rel-15 late drop is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The mechanism of measurement gap sharing in NE-DC; and its corresponding performance requirements for gap-shared measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement gap sharing mechanism for NE-DC in Rel-15 late drop is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

#### 9.1.2.1	NSA: Measurement Gap Sharing

For E-UTRA-NR dual connectivity UE configured with per-UE measurement gap, measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure cells on intra-frequency carriers or when SMTC configured for intra-frequency measurement are fully overlapping with per-UE measurement gaps, and when UE is configured to identify and measure cells on inter-frequency carriers, E-UTRA gap-needed inter-frequency carriers and inter-RAT UTRAN carriers and/or inter-RAT GSM carriers.

For E-UTRA-NR dual connectivity UE configured with per-FR1 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-FR1 measurement gaps, and when UE is configured to identify and measure cells on FR1 inter-frequency carriers, E-UTRA gap-needed inter-frequency carriers, inter-RAT UTRAN carriers and/or inter-RAT GSM carriers.

For E-UTRA-NR dual connectivity UE configured with per-FR2 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers or when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-FR2 measurement gaps, and when UE is configured to identify and measure cells on FR2 inter-frequency carriers.

For E-UTRA-NR dual connectivity UE, when network signals “01”, “10” or “11”, where X is a signalled RRC parameter measGapSharingConfig [2][16]and is defined as in Table 9.1.2.1-1,

-	the performance of intra-frequency measurements with  no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, shall consider the factor Kintra = 1 / X * 100,

-	the performance of intra-frequency measurements with measurement gaps as specified in section 9.2.6  shall consider the factor Kintra = 1 / X * 100,

-	the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 shall consider the factor  Kinter = 1 / (100 – X) * 100,

For E-UTRA-NR dual connectivity UE, when network signals “00” indicating equal splitting gap sharing, X is not applied and the performance of intra-frequency measurements as specified in section 9.2.5 and section 9.2.6, the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 are FFS.

For NR-E-UTRA dual connectivity UE configured with per-UE measurement gap, measurement gap sharing shall be applies when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-UE measurement gaps, and when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers, when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-UE measurement gaps or when UE is configured to identify and measure cells on inter-frequency carriers, E-UTRA gap-needed inter-frequency carriers, inter-RAT UTRAN carriers and/or inter-RAT GSM carriers.

For NR-E-UTRA dual connectivity UE configured with per-FR1 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR1 intra-frequency carriers or when SMTC configured for FR1 intra-frequency measurement are fully overlapping with per-FR1 measurement gaps, and when UE is configured to identify and measure cells on FR1 inter-frequency carriers, E-UTRA gap-needed inter-frequency carriers, inter-RAT UTRAN carriers and/or inter-RAT GSM carriers.

For NR-E-UTRA dual connectivity UE configured with per-FR2 measurement gap, measurement gap sharing shall be applied when UE requires measurement gaps to identify and measure cells on FR2 intra-frequency carriers or when SMTC configured for FR2 intra-frequency measurement are fully overlapping with per-FR2 measurement gaps, and when UE is configured to identify and measure cells on FR2 inter-frequency carriers.

For NR-E-UTRA dual connectivity UE, when network signals “01”, “10” or “11”, where X is a signalled RRC parameter measGapSharingConfig [2][16]and is defined as in Table 9.1.2.1-1,

-	the performance of intra-frequency measurements on FR1 with  no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement on FR1 are fully overlapping with measurement gaps, shall consider the factor Kintra-FR1 = 1 / X * 100,

-	the performance of intra-frequency measurements on FR1 with measurement gaps as specified in section 9.2.6  shall consider the factor Kintra-FR1 = 1 / X * 100,

-	the performance of intra-frequency measurements on FR2 with no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement on FR2 are fully overlapping with measurement gaps, shall consider the factor Kintra-FR2 =  1 / (100 – X) * 100,

-	the performance of intra-frequency measurements on FR2 with measurement gaps as specified in section 9.2.6  shall consider the factor Kintra-FR2 = 1 / (100 – X) * 100,

-	the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 shall consider the factor  Kinter = 1 / (100 – X) * 100,

For NR-E-UTRA dual connectivity UE, when network signals “00” indicating equal splitting gap sharing, X is not applied and the performance of intra-frequency measurements as specified in section 9.2.5 and section 9.2.6, the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 are FFS.

Table 9.1.2.1-1: Value of parameter X for EN-DC and NE-DC measurement gap sharing

| measGapSharingConfig | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | 25 |
| ‘10’ | 50 |
| ‘11’ | 75 |

### >>>>>End of Change 1<<<<<