3GPP TSG-RAN WG4 Meeting #90	R4-1900789

# Athens, GR, 25th  Feb – 1st March, 2019

Agenda item:	          	6.12.8.1.2

Source:		MediaTek Inc.

Title:		Discussion on requirements for L1-RSRP measurement for reporting in FR2

Document for:	Discussion

- Introduction

In this the paper, our views on the N factor and scheduling restriction for L1-RSRP reporting in FR2 are provided.

- N factor and scheduling restrictions for L1-RSRP measurement for reporting in FR2

One open issue is that if N=1 can apply for CSI-RS which is configured with repetition parameter “OFF”. From our point of view, the RX beam can be fixed for the CSI-RS with repetition “OFF” if it is Type-D QCLed to SSB for L1-RSRP beam reporting or CSI-RS with repetition “ON”, i.e., if the QCL source is provided. Since the RX beam sweeping has been performed on the QCL source, the RX beam for CSI-RS with repetition “OFF” can follow the indicated QCL source. The corresponding N=1 condition can reuse the N=1 condition of CSI-RS based BFD [1].

Proposal 1: For CSI-RS configured with repetition “OFF”, if it is Type-D QCLed to SSB for L1-RSRP beam reporting or CSI-RS with repetition “ON”, N=1 can apply. Otherwise, N=1 does not apply and scheduling restriction should be applied.

Currently the scheduling restriction is depending on whether N=1 is applying. However, for the CSI-RS configured with repetition parameter “ON”, it has agreed the scheduling restriction should be always applied in FR2, regardless whether N=1 is applying or not.

Observation 1: Scheduling restriction for CSI-RS configured with repetition “ON” should be always applied in FR2, regardless whether N=1 is applying or not.

Proposal 2: If N=1 can apply for CSI-RS configured with repetition “OFF”, the scheduling restriction for CSI-RS configured with repetition “ON” should be separately captured.

- Conclusion

In the contribution, we discuss the requirement for L1-RSRP measurement for reporting. We have the following observations and proposals:

Proposal 1: For CSI-RS configured with repetition “OFF”, if it is Type-D QCLed to SSB for L1-RSRP beam reporting or CSI-RS with repetition “ON”, N=1 can apply. Otherwise, N=1 does not apply and scheduling restriction should be applied.

Observation 1: Scheduling restriction for CSI-RS configured with repetition “ON” should be always applied in FR2, regardless whether N=1 is applying or not.

Proposal 2: If N=1 can apply for CSI-RS configured with repetition “OFF”, the scheduling restriction for CSI-RS configured with repetition “ON” should be separately captured.

- Reference

[1] TS38.133, NR; Requirements for support of radio resource management, V15.4.0.