3GPP TSG-RAN WG4 Meeting #90	                                                          R4-1900795

Athens, GR, 25th Feb – 1st March, 2019

Agenda Item:	6.13.4.13

Source: 	MediaTek Inc.

Title: 	Remaining issues on RLM test cases

Document for:	Discussion

1	Introduction

In the RAN4-89, SSB-based RLM tests are discussed [1]. However, there are some remaining issues in FR2 including

- the number of RLM RS

- RX beam sweeping

  - Timer settings

In the contribution, we discuss these issues and provide views.

2	 FR2 INS and OOS tests with 2 SSB based RLM RSs

Configuring 2 SSBs as RLM-RSs is beneficial and it can be used to test the following OOS and INS behaviours. To verify mentioned OOS and INS behaviours, Figured 1 and Figure 2 demonstrate OOS and INS RLM tests with 2 SSBs, respectively. The corresponding SNR level settings are highlighted in Table 1 and Table 2 as well.

- OOS: When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers.

- INS: When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, Layer 1 of the UE shall send an in-sync indication for the cell to the higher layers

Figure 1: Proposed FR2 OOS test case

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| ssb-Index 0 AoA Configuration | Config 1, 2 |  | TBD |  |  | TBD |  |  |
| ssb-Index 1 AoA Configuration | Config 1, 2 |  | TBD |  |  | TBD |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1, 2 | dB | 1 | -7 | -15 | 1 | -7 | -15 |
| ssb-Index 1 SNR | Config 1, 2 |  | 1 | -15 | -15 | 1 | -15 | -15 |
|  | Config 1, 2 | dBm/15KHz | -98 |  |  | -98 |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  | TDL-A 30ns 75Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |  |  |  |

Table 1: Proposed OTA related parameters for FR2 OOS test case

Figure 2: Proposed FR2 INS test case

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| ssb-Index 0 AoA Configuration | Config 1, 2 |  | TBD |  |  |  |  | TBD |  |  |  |  |
| ssb-Index 1 AoA Configuration | Config 1, 2 |  | TBD |  |  |  |  | TBD |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1, 2 | dB | 1 | -7 | -15 | -4.5 | 1 | 1 | -7 | -15 | -4.5 | 1 |
| ssb-Index 1 SNR | Config 1, 2 |  | 1 | -15 | -15 | -15 | -15 | 1 | -15 | -15 | -15 | -15 |
|  | Config 1, 2 | dBm/15KHz | -98 |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Proposed OTA related parameters for FR2 INS test case

Proposal 1: For FR2 SSB-based RLM test cases, two RLM-RSs configured as RLM RSs. And the correspond SNR level settings are shown in Figure 1, Figure2, Table 1 and Table 2.

3	RX beam sweeping

In RAN4-89, the conditions of N = 1 were discussed but not finalized [2] [3]. For SSB based RLM, N=1 may not applicable. Therefore we prefer introduce N = 8 for all FR2 test cases.

Proposal 2: N = 8 for all FR2 RLM tests.

3.1	FR2 INS test

As shown in Table 3, longer evaluation period is expected due to N = 8.

| Test case | TEvaluate_out (ms) |
| --- | --- |
| Test 1 in Table A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2 | 4800 |
| Test 2 in Table A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2 | 9600 |
| Test 1 in Table A.5.5.1.4.1-2/ Table A.7.5.1.4.1-2 | 14400 |

Table 3: TEvaluate_out values in FR2 INS test cases

In INS tests, at least one OOS indicator shall occur during T3 and T3 value shall be no less than TEvaluate_out. However, for some legitimate UEs which send out OOS indication earlier, T310 may be expired during T3 and the undesired procedure, i.e. re-establishment procedure, may be triggered as well [4]. The one straight forward candidate solution is to choose the maximum T310 value. However, it doesn’t work because the maximum T310 value of 6000ms is still less than TEvaluate_out in Test 2 in A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2 and Test 1 in Table A.5.5.1.4.1-2/ Table A.7.5.1.4.1-2. Likewise, T310 may be expired during D1 in Test 1 of Table A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2. To solve this issue for these test cases, we prefer the following settings

- For Test 1 in Table A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2,

  - SMTC periodicity is 160ms. SMTC is partially overlapped with RLM RS. (P = 8/7)

  - T310 is configured as 6000 ms

  - TEvaluate_out = 1840ms. (T3, T4, T5, D1) = ([1860]ms, [20]ms, [2000]ms, [960]ms)

- For Test 2 in Table A.5.5.1.2.1-2/ Table A.7.5.1.2.1-2,

  - SMTC periodicity is 160ms. SMTC is fully overlapped with measurement gap. SMTC is partially overlapped with RLM RS (P = 2)

  - T310 is configured as 6000 ms

  - TEvaluate_out = 3200ms. (T3, T4, T5, D1) = ([3220]ms, [20]ms, [2000]ms, [1640]ms)

- For Test 1 in Table A.5.5.1.4.1-2/ Table A.7.5.1.4.1-2,

  - SMTC periodicity is 160ms and SMTC is partially overlapped with RLM RS (P = 8/7)

  - T310 is configured as 6000 ms

  - TEvaluate_out = 5520ms. (T3, T4, T5, D1) = ([5540]ms, [20]ms, [2000]ms, [960]ms)

Proposal 3: In FR2 INS tests,

- SMTC periodicity is 160ms and SMTC is partially overlapped with RLM-RS occasions.

- T310 is configured as 6000ms

Proposal 4: For FR2 INS Test 1 in non-DRX mode, (T3, T4, T5, D1) = ([1860]ms, [20]ms, [2000]ms, [960]ms).

Proposal 5: For FR2 INS Test 2 in non-DRX mode, (T3, T4, T5, D1) = ([3220]ms, [20]ms, [2000]ms, [1640]ms).

Proposal 6: For FR2 INS Test 1 in DRX mode, (T3, T4, T5, D1) = ([5540]ms, [20]ms, [2000]ms, [960]ms).

Figure 3: T310 is expired in INS test cases

3.2	FR2 OOS test

Currently DRX cycle is 640ms in FR2 OOS DRX test and its corresponding TEvaluate_out is up to 153.6 s. To shorten required testing time, we prefer 40 ms DRX cycle in FR2 OOS DRX test.

Proposal 7: For FR2 OOS DRX tests, DRX cycle is 40ms.

4	Summary

Based on the discussion in section 2, 3, 4 and 5, we have the following proposals:

Proposal 1: For FR2 SSB-based RLM test cases, two RLM-RSs configured as RLM RSs. And the correspond SNR level settings are shown in Figure 1, Figure2, Table 1 and Table 2.

Proposal 2: N = 8 for all FR2 RLM tests.

Proposal 3: In FR2 INS tests,

•	SMTC periodicity is 160ms and SMTC is partially overlapped with RLM-RS occasions.

•	T310 is configured as 6000ms

Proposal 4: For FR2 INS Test 1 in non-DRX mode, (T3, T4, T5, D1) = ([1860]ms, [20]ms, [2000]ms, [960]ms).

Proposal 5: For FR2 INS Test 2 in non-DRX mode, (T3, T4, T5, D1) = ([3220]ms, [20]ms, [2000]ms, [1640]ms).

Proposal 6: For FR2 INS Test 1 in DRX mode, (T3, T4, T5, D1) = ([5540]ms, [20]ms, [2000]ms, [960]ms).

Proposal 7: For FR2 OOS DRX tests, DRX cycle is 40ms.

5	Reference

[1], RAN4#89 Meeting report

[2], R4-1815170, `` Further discussion on RLM requirements,’’ Huawei, HiSilicon

[3], R4-1900794, ``Discussion on RLM’’

[4], TS 38.331 `` Radio Resource Control (RRC) protocol specification,’’