3GPP TSG-RAN WG4 Meeting #90                                                                 R4-1900839

Athens, Greece, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1: modification of requirements for network signalled value NS_04 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilcon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | “Transmission bandwidth” represents active RBs within the transmission bandwidth configuration, the usage is not correct in subclause 6.5.2.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Transmission bandwidth” is replaced by “maximum transmission bandwidth configuration” and “emission bandwidth” for NS_04. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of “Transmission bandwidth” is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5.2.3.2	Requirements for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  Since the 26dB emission bandwidth is implementation dependent, the maximum transmission bandwidth configuration occupied by RBs is used for the SEM.

Table 6.5.2.3.2-1: n41 emission bandwidths for CP-OFDM

| SCS (kHz) | Channel bandwidths (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | 9.36 | 14.22 | 19.08 | 38.88 | 48.6 | N.A | N.A | N.A | N.A |
| 30 | 8.64 | 13.68 | 18.36 | 38.16 | 47.88 | 58.32 | 78.12 | 88.02 | 98.28 |
| 60 | 7.92 | 12.96 | 17.28 | 36.72 | 46.8 | 56.88 | 77.04 | 87.12 | 97.20 |

Table 6.5.2.3.2-2: n41 emission bandwidths for DFT-S-OFDM

| SCS (kHz) | Channel bandwidths (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | 9.00 | 13.50 | 18.00 | 38.88 | 48.60 | N/A | N/A | N/A | N/A |
| 30 | 8.64 | 12.96 | 18.00 | 36.00 | 46.08 | 58.32 | 77.76 | 87.48 | 97.20 |
| 60 | 7.20 | 12.96 | 17.28 | 36.00 | 46.08 | 54.00 | 72.00 | 86.40 | 97.20 |

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.2-3.

Table 6.5.2.3.2-3: n41 SEM with “NS_04”

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0 - 1 | -10 | -10 | -10 | -10 |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  | -10 |  |  |  | 1 MHz |
| ± 1 - 5 |  | -10 |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  | -13 |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  | -25 |  |  |  |  |  |  |  |  |
| NOTE: X is defined in Table 6.5.2.3.2-1 for CP-OFDM and 6.5.2.3.2-2 for DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |

<End of Changes>