3GPP TSG-RAN WG4 Meeting #90	R4-1900920

Athens, Greece, 25th February – 1st March, 2019

Agenda item:		6.12.3.1.2

Source:	MediaTek Inc.

Title:	Clarification on UE behavior after measurement gap

Document for:		Discussion

# 1	Introduction

In current spec TS36.133[1], the total interruption time on E-UTRAN serving cells caused by measurement gap and the UE behavior after measurement gap are specified as follows:

During the measurement gaps the UE: - shall not transmit any data - is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell and any SCell. - is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell, PSCell, and SCell. If the UE supporting dual connectivity is configured with PSCell, during the total interruption time as shown in Figure 8.1.2.1-1, the UE shall not transmit and receive any data in SCG. In addition, for UE supporting E-UTRA-NR dual connectivity, if MG timing advance of 0.5ms is applied, the UE: - shall not transmit any data - is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell and any SCell. - is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell, PSCell, and SCell. in subframes fully or partially overlapping with the measurement gaps on E-UTRAN serving cells. The total interruption time on E-UTRAN serving cells is (MGL+1) subframes. In the uplink subframe occurring immediately after the measurement gap, - if the following conditions are met then it is up to UE implementation whether or not the UE can transmit data: - all the serving cells belong to E-UTRAN TDD; - the measurement objects do not include any NR carrier frequency; - if the subframe occurring immediately before the measurement gap is an uplink subframe. - Otherwise the UE shall not transmit any data.

Besides, in TS38.133[2], it is already agreed to only use the DL timing to decide the MG starting point and the UE behavior after measurement gap are also specified

In the slot occurring immediately after measurement gap and fully non-overlapped with measurement gap - It is up to UE implementation whether or not the UE is able to conduct transmission if all the symbols in the slot are uplink symbols. Editor notes: FFS if very large TA is applied.

In this paper, the remaining issues on UE behavior after measurement gap are discussed.

# 2	Clarification on UE behavior after measurement gap in TS36.133

Based on current spec, when MG timing advance is not applied, the UE behavior after measurement gap is clear. In TS36.133, shall UE transmit any data in the uplink subframe occurring immediately after the measurement gap depends on subframe type occurring immediately before the measurement gap. If it is a DL subframe, UE shall not transmit any data. If it is a UL subframe, it is up to UE implementation whether or not the UE can transmit data. The corresponding figures are shown as follows:

Figure 1. UE behavior after measurement gap when MG timing advance is not applied.

However, when MG timing advance is applied, the UE behavior after measurement gap is not addressed yet.

Observation 1: UE behavior after measurement gap In TS36.133 is unclear when MG timing advance is applied.

As shown in Figure 2, it is straightforward that UE can determine whether or not to transmit data when the reference subframe is an UL subframe; however, shall we still restrict UE not to transmit any data when the reference subframe is a DL subframe? From our point of view, it is possible for UE to transmit data if the UL TA is less than the MGTA.

Observation 2: In TS36.133, when MG timing advance is applied, the uplink transmission occurring immediately after the measurement gap is possible.

Figure 2. UE behavior after measurement gap when MG timing advance is applied.

# 3	Clarification on UE behavior after measurement gap in TS38.133

In TS38.133, it was agreed that UE shall always use the DL timing of the latest subframe occurring immediately before the configured measurement gap among serving cells to determine the measurement gap starting point. In the slot occurring immediately after measurement gap and fully non-overlapped with measurement gap, it is up to UE implementation whether or not the UE is able to conduct transmission. From our point of view, only specify the UE behavior on 1 slot after measurement gap is not enough. As we mentioned in last meeting [3]. For NR SCSs, the max NTA value specified in TS 38.213 for random access could be up to

NTA=3846*16*642u*Tc=3846*16*Ts≈2slots

In case of random access response, a timing advance command [11, TS 38.321], , for a TAG indicates values by index values of = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG for subcarrier spacing of kHz is given by . is defined in [4, TS 38.211] and is relative to the subcarrier spacing of the first uplink transmission from the UE after the reception of the random access response. In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of the current value, , to the new value, , by index values of = 0, 1, 2,..., 63, where for a subcarrier spacing of kHz, .

Observation 3: The maximum TA in NR could be up to 2 slots.

As shown in Figure 3, when NTA≈2slots, the impacted UL slots number occurring immediately after measurement gap and fully non-overlapped with measurement gap may also be up to 2 slots, e.g., UL slots #7 and #8 Therefore, we suggest that

(a)

(b)

Figure 3. UE behavior after measurement gap (a) when MG timing advance is not applied, and (b) when MG timing advance is applied.

Proposal 1: In Rel-15, it is up to UE implementation whether or not the UE is able to conduct transmission in 2 slots occurring immediately after measurement gap and fully non-overlapped with measurement gap.

Proposal 2: In later releases, RAN4 can study if there exists an appropriate way for network to know the exact UE behavior after measurement gap.

# 4	Summary

In this contribution, we observe that

Observation 1: UE behavior after measurement gap In TS36.133 is unclear when MG timing advance is applied.

Observation 2: In TS36.133, when MG timing advance is applied, the uplink transmission occurring immediately after the measurement gap is possible.

Observation 3: The maximum TA in NR could be up to 2 slots.

And we propose

Proposal 1: In Rel-15, it is up to UE implementation whether or not the UE is able to conduct transmission in 2 slots occurring immediately after measurement gap and fully non-overlapped with measurement gap.

Proposal 2: In later releases, RAN4 can study if there exists an appropriate way for network to know the exact UE behavior after measurement gap.

# 5	Reference

[1] TS36.133, Jan., 2019.

[2] TS38.133, Jan., 2019.

[3] R4-1814894, UE Behavior Before and After Measurement Gap, Mediatek, Nov., 2018.